Table 2-2. Review of Single Phase Measurements in Stirred Tanks

Table 2-2(a). Parametric sensitivity of fluid dynamic measurements in stirred tanks reported by Rutherford et. al. (1996)

| Parameters | Power Number (NP) | # Pumping Number (NQ) | Dimensionless mean velocity(V/Vtip) | Dimensionless root mean squared velocity (V`/Vtip) |
| --- | --- | --- | --- | --- |
| tb/D | 3 fold increase causes 35% reduction in power number | 3 fold increase causes 15% reduction. Difference goes down with increasing radial distance from impeller tip | 3 fold increase causes 20% reduction in mean velocities. Difference decreases with radial distance from impeller tip | 3 fold increase causes 25% reduction in RMS. Difference decreases with increase in radial distance from impeller tip |
| tD/D | 3 fold increase causes 35% reduction | 3 fold increase causes 15% reduction. Difference goes down with increasing radial distance from impeller tip | 3 fold increase causes 20% reduction in mean velocities. Difference decreases with radial distance from impeller tip | 3 fold increase causes 25% reduction in RMS. Difference decreases with increase in radial distance from impeller tip |
| D/T | No effect | No effect | No effect | No effect |

(refer to Figure 2-2 for the dimensions),.

Table 2-2(b). Parameters of the System Used for Validation in This Study

| Researcher | T(mm) | D/T | H/T | Hc/T | tb/D | tD/D | Re =ND2/ | Fluid |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Cutter (1967) | 290 | 1/3 | 1 | 1/3 | _______ | _______ | 34600- 120000 | Water |
| Cooper and Wolf (1968) | 380 | .27 & 0.4 | 1 | 1/3 | _______ | ______ | 50000 | Water |
| Drobholov et. al. (1978) | 1000 | 1/3 | 1 | 1/3 | _______ | ______ | 185200 & 288000 | Water |
| Chen et. al. (1988) | 89 | 1/3 | 1 | 1/3 | ______ | ______ | 1630,3260 & 6520 | Water |
| Wu and Patterson (1989) | 270 | 1/3 | 1 | 1/3 | ______ | ______ | 30000 | Water |
| Ranade and Joshi (1990) | 300 | 1/3 | 1 | ½ | .008 & .0016 | .027 * .022 | 56000 | Water |
| Yianneskis and Whitelaw (1993) | 294 | 1/3 | 1 | 1/3 | .03 | .03 | 54000 | Water |
| Zhou and Kresta (1996) | 240 | ½ | 1 | ½ | _______ | _______ | 52200 | Water |
| Schaefer et. al. (1996) | 150 | 1/3 | 1 | 1/3 | .04 | .04 | 800, 1200 & 2000 | White oil |
| Current Work | 200 | 1/3 | 1 | 1/3 | .008 | .015 | 8200,12300,16400 & 32800 | Water |

For Dimensions refer to Figure 2-2. Re=ND2ν