WASHINGTON UNIVERSITY

SEVER INSTITUTE OF TECHNOLOGY

DEPARTMENT OF CHEMICAL ENGINEERING

______________________________________________________________

PERFORMANCE STUDIES OF TRICKLE BED REACTORS

by

Mohan R. Khadilkar

Prepared under the direction of Prof. M. P. Dudukovic and Prof. M. H. Al-Dahhan

___________________________________________________________________

A dissertation presented to the Sever Institute of

Washington University in partial fulfillment

of the requirements for the degree of

DOCTOR OF SCIENCE

August, 1998

Saint Louis, Missouri, USA

WASHINGTON UNIVERSITY

SEVER INSTITUTE OF TECHNOLOGY

DEPARTMENT OF CHEMICAL ENGINEERING

____________________________________________________

ABSTRACT

_____________________________________________________________________

PERFORMANCE STUDIES OF TRICKLE BED REACTORS

by  Mohan R. Khadilkar

__________________________________________________________

ADVISORS: Prof. M. P. Dudukovic and Prof. M. H. Al-Dahhan

___________________________________________________

August, 1998

Saint Louis, Missouri, USA

_______________________________________

A thorough understanding of the interaction between kinetics, transport, and hydrodynamics in trickle bed reactors under different reaction and operating conditions is necessary to design, scale-up, and operate them in such a way as to achieve the best performance. In this study, systematic experimental and theoretical investigations have been carried out to study trickle bed performance in different modes of operation and reaction conditions in order to improve our understanding of the factors governing scale-up and performance.

The first part of this study is focused on comparison of performance of down-flow (trickle bed reactors-TBR) and up-flow reactors (packed bubble columns-PBC) without and with fines to assess their applicability as test reactors for scale-up and scale-down studies for different reaction systems (gas and liquid reactant limited). This has been accomplished by experimentation on hydrogenation of a-methylstyrene to cumene in hexane solvent over 2.5% Pd on alumina extrudate catalyst as a test reaction, and, by comparing the predictions of existing models for both modes of operation with each other and with the data. Comparison of results obtained at different pressures, liquid reactant feed concentrations, and gas flow rates has been presented, and differences in performance of the two reactor modes of operation explained on the basis of the observed shift from gas limitation to liquid limitation. Experiments in the bed diluted with fines have been conducted to demonstrate the fact that hydrodynamics and kinetics can be de-coupled by using fines. It has also been shown that the advantage of upflow or downflow mode of operation depends on whether liquid or gas reactant is rate limiting, and that an approximate criterion for identifying the limiting reactant can explain most of the data reported in the literature on these two modes of operation. Comparison of experimental observations and predictions of the reactor scale and pellet scale models available in the literature is also reported. A rigorous steady state model for the solution of the reactor and pellet scale flow-reaction-transport phenomena based on multicomponent transport is proposed to overcome assumptions in earlier models such as non-volatile reactants, dilute solutions, isothermal, isobaric operation, and constant phase velocities. Predictions of this model are compared against data available in literature for a system with volatile liquids.

The second part of this study is devoted to investigating the performance of trickle bed reactors under unsteady state liquid flow modulation (periodic operation) for gas and liquid limited reactions. Exploitation of the opportunity of alternately and systematically supplying the liquid and gaseous reactants to the catalyst during and after the liquid pulse, respectively, has been shown to result in performance different from that obtained under steady state conditions. The effect of key parameters such as extent of gas/liquid limitation, total cycle period, cycle split, liquid mass velocity, and liquid solid contacting have been investigated experimentally to demonstrate the cause-effect relationships in unsteady state operation. Performance enhancement has been demonstrated to be dependent on cycling parameters such as cycle period, split and induced flow modulation frequency for gas limited conditions. It has been observed to be strongly dependent on extent of catalyst wetting under liquid limited conditions. Rigorous modeling of the interphase transport of mass and energy based on the Maxwell-Stefan approach at the reactor and catalyst level has been used to simulate the processes occurring under unsteady state conditions for a general multi-component system. Reactor performance results for several flow modulation simulation tests for a hydrogenation reaction have been presented and discussed.

# Contents

Page

List Of Tables	ix

List Of Illustrations	x

Acknowledgements	xiv

Nomenclature	xvi

Chapter 1. Introduction	1

1.1 Motivation	4

1.1.1 Comparison of Down-flow (Trickle Bed Reactor-TBR) and  Up-flow (Packed Bubble Column-PBC)  Reactors	4

1.1.2 Unsteady State Operation of Trickle Bed Reactors	7

1.2 Objectives	12

1.2.1 Comparison of Down-flow (TBR) and Up-flow (PBC) Performance	12

1.2.2 Unsteady State Operation of Trickle Bed Reactors	13

Chapter 2. Background	15

2.1 Laboratory Reactors – Performance Comparison and Scaleup Issues	15

2.1.1 Literature on Performance Comparison	15

2.1.2 Criterion for Gas and Liquid Reactant Limitation	16

2.2 Literature on Unsteady State Operation of Trickle Bed Reactors	20

2.2.1 Strategies for Unsteady State Operation	21

2.3 Review of Models for TBR Performance	27

2.3.1 Steady State Models	27

2.3.2 Unsteady State Models for Trickle Bed Reactors	37

2.4 Modeling Multicomponent Effects	43

2.5 Balance Relations for Multiphase Systems	47

Chapter 3. Experimental Facility	50

3.1 High Pressure Trickle Bed Setup	50

3.1.1 Reactor and Distributors for Upflow and Downflow	50

3.1.2 Gas-Liquid Separator and Level Control	51

3.1.4 Liquid and Gas Delivery System	52

3.1.5 Data Acquisition and Analysis	57

3.2 Operating Procedures and Conditions	58

3.2.1 Steady State TBR-PBC Comparison Experiments	58

3.2.2 Bed Dilution and Experiments with Fines	60

3.2.3 Unsteady State Experiments	62

Chapter 4. Experimental Results	65

4.1 Steady State Experiments on Trickle Bed Reactor and Packed Bubble Column	65

4.1.1 Effect of Reactant Limitation on Comparative Performance of TBR and PBC	65

4.1.2 Effect of Reactor Pressure on Individual Mode of Operation	69

4.1.3 Effect of Feed Concentration of -methylstyrene on Individual Mode of Operation	70

4.1.5 Effect of Gas Velocity and Liquid-Solid Contacting Efficiency	75

4.2 Comparison of Down-flow (TBR) and Up-flow (PBC) Reactors with Fines	77

4.2.1 Effect of Pressure in Diluted Bed on Individual Mode of Operation	80

4.2.2 Effect of Feed Concentration in Diluted Bed on Individual Mode of Operation	80

4.3 Unsteady State Experiments in TBR	82

4.3.1 Performance Comparison for Liquid Flow Modulation under Gas and Liquid Limited Conditions	82

4.3.2 Effect of Modulation Parameters (Cycle Period and Cycle Split) on Unsteady State TBR Performance	85

4.3.3 Effect of Amplitude (Liquid Mass Velocity) on Unsteady State TBR Performance	88

4.3.4 Effect of Liquid Reactant Concentration and Pressure on Performance	90

4.3.5 Effect of Cycling Frequency on Unsteady State Performance	93

4.3.6 Effect of Base-Peak Flow Modulation on Performance	96

Chapter 5. Modeling Of Trickle Bed Reactors	98

5.1 Evaluation of Steady State Models for TBR and PBC	98

5.1.1 Reactor Scale Model (El-Hisnawi et al., 1982)	98

5.1.2 Pellet Scale Model (Beaudry et al., 1987).	100

5.2 Unsteady State Model for Performance of Trickle Bed Reactors in Periodic Operation	110

5.2.1 Reactor Scale Transport Model and Simulation	113

5.2.2 Flow Model Equations	118

5.2.3 Multicomponent Transport at the Interface	124

5.2.4 Catalyst Level Rigorous and Apparent Rate Solution	128

Chapter 6. Conclusions	146

6.1 Recommendations for Future Work	148

Appendix A. Slurry Experiments: Intrinsic Rate  At High Pressure	151

Appendix B. Correlations Used In Model Evaluation	154

Appendix C. Flow Charts For The Unsteady State Simulation Algorithm	155

Appendix D Maxwell-Stefan Equations For Multicomponent Transport	158

Appendix E Evaluation Of Parameters For Unsteady State Model	163

Appendix F. Experimental Data From Steady And Unsteady Experiments	166

Appendix G. Simulation of Flow using CFDLIB	185

Appendix H. Improved Prediction of Pressure Drop in High Pressure Trickle Bed Reactors	196

References	204

VITA	209

# List Of Tables

Table	Page

Table 2. 1 Identification of the Limiting Reactant for Literature and Present Data.	Error! Bookmark not defined.

Table 2. 2 Literature Studies on Unsteady State Operation in Trickle Beds	24

Table 2. 3 Review of Recent Steady State Reaction Models for Trickle Bed Reactors	29

Table 2. 4 Review of Unsteady State Models for Trickle Bed Reactors	39

Table 3. 1 Catalyst and Reactor Properties for Steady State Experiments	61

Table 3. 2 Range of Operating Conditions for Steady State Experiments	61

Table 3. 3 Catalyst and Reactor Properties for Unsteady State Conditions	64

Table 3. 4 Reaction and Operating Conditions for Unsteady State Experiments	64

Table 5. 1 Governing Equations for El-Hisnawi (1982) Model	104

Table 5. 2 Governing Equations for Beaudry (1987) Model	105

Table 5. 3 Typical Equation Vector for the Stefan-Maxwell Solution of Gas-Liquid Interface	127

Table 5. 4 Typical Stefan Maxwell Equation Vector for a Half Wetted Pellet	130

Table 5. 5 Energy Flux Equations for Gas-Liquid, Liquid-Solid, and Gas-Solid Interfaces	132

Table 5. 6 Equation Set for Single Pellet Model	134

Table 5. 7 Catalyst Level Equations for Rigorous Three Pellet Model	136

Table 5. 8 List of Model Variables and Equations	137

Table A. 1 Rate Constants Obtained from Slurry Data at Different Pressures	152

# List Of Figures

Figure	Page

Figure 1. 1 Trickle bed Reactor: Flow Regimes and Catalyst Wetting Conditions	3

Figure 2. 1 Time Averaged SO2 Oxidation Rates of Haure et al. (1990)	26

Figure 2. 2 Experimental and Predicted Temperature Profiles of Haure et al.  (1990)	26

Figure 2. 3 Enhancement in Periodic Operation Observed by Lange et al. (1993)	26

Figure 3. 1 Reactor and Gas-Liquid Separator	54

Figure 3. 2 Down-flow and Up-flow Distributor for TBR and PBC Operation	55

Figure 3. 3 Experimental Setup for Unsteady State Flow Modulation Experiments	56

Figure 3. 4 Data Acquisition System	57

Figure 3. 5 Basket Reactor  Catalyst Stability Test for Palladium on Alumina Catalyst	58

Figure 4. 1 Trickle Bed and Up-flow Performance at CBi=7.8%(v/v) and Ug =4.4 cm/s at 30 psig.	68

Figure 4. 2 Comparison of Down-flow and Up-flow Performance at CBi=3.1%(v/v) at 200 psig.	68

Figure 4. 3 Effect of Pressure at Low -methylstyrene Feed Concentration on Upflow Reactor Performance.	71

Figure 4. 4 Effect of Pressure at Low -methylstyrene Feed Concentration (3.1% v/v) on Downflow Performance.	71

Figure 4. 5 Effect of Pressure at Higher -methylstyrene Feed Concentration on Downflow Performance.	72

Figure 4. 6 Effect of -methylstyrene Feed Concentration at 100 psig on Upflow Performance.	73

Figure 4. 7 Effect of -methylstyrene Feed Concentration at 100 psig on  Downflow Performance.	73

Figure 4. 8 Effect of -methylstyrene Feed Concentration at 200 psig on Downflow Performance.	74

Figure 4. 9 Effect of -methylstyrene Feed Concentration at 200 psig on Upflow Performance.	74

Figure 4. 10 Effect of  Gas Velocity on Reactor Performance at 100 psig.	76

Figure 4. 11 Pressure Drop in Downflow and Upflow Reactors and Contacting Efficiency for Downflow Reactor at 30 and 200 psig.	76

Figure 4. 12 Effect of Fines on Low Pressure Down-flow Versus Up-flow Performance	79

Figure 4. 13 Effect of Fines on High Pressure Down-flow Versus Up-flow Performance	79

Figure 4. 14 Effect of -methylstyrene Feed Concentration at Different Pressures on Performance of Downflow with Fines.	81

Figure 4. 15 Effect of -methylstyrene Feed Concentration at Different Pressures on Performance of Upflow with Fines.	81

Figure 4. 16 Comparison of Steady and Unsteady State Performance under Liquid Limited Conditions ( < 4)	84

Figure 4. 17 Comparison of Steady and Unsteady State Performance under Gas Limited Conditions ( ~25)	84

Figure 4. 18 Effect of  Cycle Split () on Unsteady State Performance under Gas Limited Conditions	87

Figure 4. 19 Effect of Total Cycle Period () on Unsteady State Performance under Gas Limited Conditions	87

Figure 4. 20 Effect of Liquid Mass Velocity on Unsteady State Performance under Gas Limited Conditions	89

Figure 4. 21 Effect of Liquid Reactant Feed Concentration on Unsteady State Performance under Gas Limited Conditions	92

Figure 4. 22 Effect of Operating Pressure on Unsteady State Performance under Gas Limited Conditions	92

Figure 4. 23 Effect of Cycling Frequency on Unsteady State Performance under Gas Limited Conditions	95

Figure 4. 24 Effect of Cycling Frequency on Unsteady State Performance	95

Figure 4. 25 Unsteady State Performance with BASE-PEAK Flow Modulation under Liquid Limited Conditions	97

Figure 4. 26 Effect of Liquid Mass Velocity on Steady State Liquid-Solid Contacting Efficiency	97

Figure 5. 1 Upflow and Downflow Performance at Low Pressure (gas limited condition): Experimental data and model predictions	107

Figure 5. 2 Upflow and Downflow Performance at High Pressure (liquid limited condition): Experimental data and model predictions	107

Figure 5. 3 Effect of Feed Concentration on Downflow Performance	108

Figure 5. 4 Effect of Feed Concentration on Predicted Upflow Performance	108

Figure 5. 5 Estimates of volumetric mass transfer coefficients in the range of operation from published correlations (G-L (downflow) Fukushima and Kusaka (1977), L-S (downflow) Tan and Smith (1980), G-L (upflow) Reiss (1967), L-S (upflow) Spechhia (1978)).	109

Figure 5. 6 Phenomena Occurring in Trickle Bed under Periodic Operation	111

Figure 5. 7  Representation of the Catalyst Level Solution	135

Figure 5. 8: Transient Alpha-methylstyrene (-MS) Concentration Profiles at Different Axial Locations	140

Figure 5. 9 Transient Alpha-methylstyrene Concentration Profile Development with Time (shown in seconds in the legend table).	140

Figure 5. 10 Axial Profiles of Cumene Concentration at Different Simulation Times (shown in seconds in the legend table)	141

Figure 5. 11 Transient Hydrogen Concentration Profiles at Different Axial Locations	141

Figure 5. 12 Transient Liquid Holdup Profiles at Different Axial Locations in Periodic Flow	142

Figure 5. 13 Transient Liquid Velocity Profiles at Different Axial Locations in Periodic Flow	143

Figure 5. 14 Cumene Concentration Profiles during Periodic Flow Modulation	144

Figure 5. 15 Intra-catalyst Hydrogen Concentration Profiles during Flow Modulation for a Previously Externally Wetted Catalyst Pellet at Different Axial Locations.	145

Figure 5. 16 Intra-Catalyst Alpha-methylstyrene Concentration Profiles during Flow Modulation for a Previously Externally Dry Catalyst Pellet at Different Axial Locations.	145

Figure A. 1 Slurry conversion versus time at different pressures	153

Figure A. 2 Comparison of the Model Fitted Alpha-methylstyrene concentrations to experimental values	153

# Acknowledgements

I wish to express my heartfelt gratitude to my advisor Professor Dudukovic for the opportunity to undertake this work under his supervision. His advice, encouragement of independent thought, and patience was indeed invaluable in making this work possible. I would also like to thank my co-advisor Prof. Al-Dahhan, whose advice and interest in experimental aspects of the project made it possible for me to overcome many obstacles. I would like to acknowledge the members of my committee, namely, Prof. Babu Joseph, Prof. R.Gardner, Dr. Manuk Colakyan of Union Carbide Corporation, and Dr. Ramesh Gupta of Exxon Research and Engineering for taking interest in my work and examining  my thesis and providing useful comments and suggestions.

I wish to acknowledge the financial support of the Industrial Participants of the CREL consortium, which made the research possible. In particular, I would like to thank Dr. Colakyan of Union Carbide Corporation for allowing me to gain practical experience related to the first part of this thesis by participating and reviewing my work on many aspects of this project. I am also grateful to Dr. Ramesh Gupta for his ideas and insights into the implementation of the second part of this work. Acknowledgements are also due to the Agricultural Group at Monsanto Company, particularly Dr. R. Kahney, Dr. S. Chou, and G. Ahmed for their project on complex reaction networks, experience form which was helpful in several aspects of this work. I would also like to thank Dr. Patrick Mills of DuPont Central Research for his advice and help with model development on systems with volatiles.

Several people at CREL and in the Chemical Engineering Department have made significant contributions in make my work and life at Washington University. In particular, I would like to thank Mr. Yuanxin Wu and Mr. Yi Jiang for their help and support in the experimental work on several projects undertaken in the trickle bed reactor laboratory. My sincere acknowledgements also go to Steve Picker and John Krietler for their help in modifying and maintaining the high pressure trickle bed facility in good shape throughout the projects involved. My sincere gratitude to Dr. S. Kumar and Dr. Y. Yamashita for their help in resolving numerous computational aspects of the projects. My sincere acknowledgements also go to Dr. M. Kulkarni, Dr. S. Karur, and Dr. S. Degaleesan whose help and encouragement was invaluable throughout my stay at Washington University.

I sincerely acknowledge the help and assistance offered by all the past and present members of CREL including Zhen Xu, P. Gupta, S. Roy, Alain Chone, Dr. J. Chen, Dr. Y. Pan, Marco Roveda and many others. I also wish to thank the entire Chemical Engineering Department, particularly the secretaries for their help with numerous formalities. Finally, thanks to my roommates and friends whose company made my stay in St. Louis a pleasant and memorable one.

Last, but not the least, my heartfelt gratitude goes to my parents and my sister for their patience and support and their sustained belief in my abilities which help me get through the vicissitudes of student life.

Mohan R. Khadilkar

Washington University, St. Louis

August, 1998

# Nomenclature

aGS	gas-solid interfacial area (m2/m3).

aGL	gas-solid interfacial area (m2/m3).

a'GL	gas-Liquid total interfacial area (m2/m3).

aLS	liquid-solid interfacial area (m2/m3).

ct	molar density (mol/m3)

C(AMS)	alpha-methylstyrene concentration

C*(H2) 	saturation concentration of hydrogen

CA*	concentration of gaseous reactant in liquid phase (mol/m3)

CBi 	concentration of liquid reactant in liquid phase (mol/m3)

CiL 	liquid phase concentration of species i (mol/m3).

CiCP	concentration of species i in the catalyst (mol/m3).

CiG	gas phase concentration of species i (mol/m3).

CPmixG	gas phase specific heat (J/kg K).

CPmixL	liquid phase specific heat (J/kg K).

De	effective diffusivity (m2/s)

DeA	effective diffusivity of gaseous reactant in the catalyst (m2/s)

DeB 	effective diffusivity of liquid reactant in the catalyst (m2/s)

Dei	effective diffusivity of reactant i in the catalyst (m2/s)

EV	energy transfer flux from bulk gas to gas-liquid interface (J/ m2s).

EL	energy transfer flux from gas-liquid interface to liquid (J/ m2s).

ES	energy transfer flux from solid to liquid-solid interface (J/ m2s).

ELS	energy transfer flux from liquid-solid interface to liquid (J/ m2s).

g	gravitational acceleration (m/s2).

GaL	liquid phase Galileo number.

GaG	gas phase Galileo number.

HiV 	partial molar enthalpy of component i in vapor phase (J/mol).

HiL 	partial molar enthalpy of component i in liquid phase (J/mol).

HiS 	partial molar enthalpy of component i in liquid phase (J/mol).

hL	gas-liquid interface to liquid heat transfer coefficient (J/ m2 s K).

hLS	solid-liquid interface to liquid heat transfer coefficient (J/ m2 s K).

hS	solid-liquid interface to solid heat transfer coefficient (J/ m2 s K).

hV	gas-liquid interface to gas heat transfer coefficient (J/ m2 s K).

J	molar diffusion flux relative to average velocity (mol/m2s).

(ka)GL 	volumetric gas-liquid mass transfer coefficient ( 1/s)

kLS.aLS volumetric liquid-solid mass transfer coefficient (1/s)

[kLik]	liquid side mass transfer coefficient (m/s).

[kVik]	liquid side mass transfer coefficient (m/s).

ke 	effective thermal conductivity of catalyst (J/m s K).

K	interphase momentum transfer coefficient (kg/m3).

L	mass velocity of liquid (kg/m2s)

L	half the length of characteristic pellet (m).

Mi	molecular weight of species i (kg/kgmole).

NiV	flux of component i (vapor to gas-liquid interface) (mol/m2s).

NiL	flux of component i (gas-liquid interface to liquid) (mol/m2s).

NiLS	flux of component i (liquid to liquid-solid interface) (mol/m2s).

NiS	flux of component i (liquid-solid interface to solid) (mol/m2s).

P	operating pressure (psig)

P	pressure (N/m2).

DP	pressure drop (N/m2).

QL, QG	volumetric flow rate of liquid, gas

q	heat flux (J/m2s)

RW	reaction rate on fully wetted catalyst pellet (mol/m3s).

RDW	reaction rate on half wetted catalyst pellet (mol/m3s).

RD	reaction rate on fully externally dry catalyst pellet (mol/m3s).

ROV	overall reaction rate (mol/m3s).

ReL	liquid phase Reynolds number.

ReG	gas phase Reynolds number.

(rA)obs 	observed rate of reaction (mol/m3s)

Sx	catalyst external surface area (m2)

TCP	catalyst temperature (K)

TG	gas temperature (K)

TA	ambient temperature (K)

TL 	liquid temperature (K)

TI 	gas-liquid interface temperature (K).

TILS 	liquid-Solid interface temperature (K).

UIL	liquid interstitial (actual) velocity (m/s).

UIG	gas interstitial (actual) velocity (m/s).

Ug	gas velocity (m/s)

VL, VG	superficial velocity of liquid, gas

VR	reactor volume, (m3)

Vp  	catalyst Pellet Volume ( m3)

X(ss)	steady state conversion

X(us)	unsteady state conversion

x	mole fraction in the liquid phase.

x'	spatial coordinate in catalyst pellet (m).

y	mole fraction in the gas phase.

z	axial coordinate (m).

Greek Symbols

	reactant limitation criterion (=De (AMS) C(AMS) feed/ De (H2) C*(H2))

	total cycle period, s

	cycle split

- flow modulation frequency, Hz

b	bootstrap matrix in Maxwell-Stefan formulation

a',b'	order of reaction with respect to species i and j.

e,eB	bed voidage.

eG	gas holdup.

eP	porosity.

eL	liquid holdup.

esL 	static holdup of liquid.

ni	stoichiometric coefficient of component i

rL	liquid phase density (kg/m3).

rG	gas phase density (kg/m3).

hCE	contacting efficiency.

yL	dimensionless liquid phase drag.

yG	dimensionless gas phase drag.

li	difference in component molar enthalpies (J/mol).

axial coordinate (m).

temporal coordinate (s).

# Chapter 1. Introduction

Trickle bed reactors are packed beds of catalyst with cocurrent down-flow of gas and liquid, and are widely used in hydrocracking, hydrodesulfurization, etc., in the petroleum industry, as well as in some selected chemical industry applications for hydrogenation, oxidation, and chlorination and more recently in the waste treatment and biochemical processing industry. In fact, tonnage wise, trickle beds are the most used reactors in the entire chemical and related industries (1.6 billion metric tons annual processing capacity (Al-Dahhan et al. (1997)), with an enormous capital invested in design, set-up and operation of this type of reactors. Trickle bed reactors are generally operated at high pressure (up to 30 MPa) and temperature (up to 300 o C) in the low interaction or trickle flow regime  i.e., the flowing gas is the continuous phase and liquid flows as rivulets and films over the catalyst particles (Figure 1.1). The packing, which occupies 55 - 65 % of the bed volume, is usually a supported catalyst in the form of different shapes (spheres, extrudates, trilobes etc.) and sizes ranging from 1 to 3 mm. Due to large throughputs, reactor volumes can be as high as 200 m3 with bed heights up to 20 m in some cases. Trickle bed reactors have several advantages over other type of multiphase reactors such as: plug flow like flow pattern, high catalyst loading per unit volume of liquid, low power requirements, low energy dissipation, and greater flexibility with respect to production rates and operating conditions used. Some disadvantages of trickle bed reactors are their lower catalyst effectiveness factors due to large particle sizes, higher pressure drops, limited use in viscous or foaming liquids, sensitivity to thermal effects and inefficient heat removal. Despite the fact that the advantages outweigh the disadvantages, trickle bed reactors have not found applications to their fullest potential due to difficulties associated with their design and uncertainty in the scale-up strategies used for their commercial application. These difficulties are introduced by the complexities of two phase flow, flow distribution, and incomplete catalyst wetting, which are not duplicated in laboratory scale experiments to the same extent (Saroha and Nigam, 1996). In order to expand the horizon of applications of these reactors, it is necessary to understand all the relevant complex phenomena, on the macro, meso and micro scale as well as their interactions that can affect reactor performance (Satterfield, 1975; Ramachandran and Chaudhari, 1983; Dudukovic and Mills, 1986; Gianetto and Silveston, 1986). Some of these phenomena include intrinsic and apparent kinetics on the catalyst scale, gas and liquid flow and distribution on the catalyst and reactor scale, and their coupling with catalyst wetting and interphase transport of mass, momentum, and energy, and finally on the conversion and selectivity of the reactor.

The above mentioned phenomena are typically studied in laboratory reactors at much lower gas and liquid mass velocities (L < 0.1 kg/m2s and G < 10-2 kg/m2s in laboratory reactors as compared to L < 50 kg/m2s, and G < 25 kg/m2s in industrial units) to maintain the same scale up variable (liquid hourly space velocity, LHSV). Scale-up strategies for trickle bed reactors to date have been considered an art and have not been developed beyond the realm of hydrodesulphurization and to some extent, hydrotreating. The proper choice of laboratory reactors for testing of catalysts and feedstocks, in order to scale up or scale down has not been dealt with comprehensively. This results in commercial trickle bed reactors which are either grossly over-designed or perform well below design criteria. At the same time, a reliable method for scale-down in investigating new catalysts and feedstocks is needed for rapid selection of optimal processing conditions and for cost effective performance of existing commercial units. Thus, rationalization of scale-up procedures is needed. Since the investment (to the order of millions of dollars for each unit) has already been made, it may be worthwhile to investigate if a strategy exists to obtain an enhancement in performance by modifying the method of operation, such as unsteady state operation, and whether an optimal performance can be obtained in an existing reactor by using this strategy. Any enhancement in performance of the pre-existing reactors, even by a few percent, would translate to a significant financial gain without further capital investment. Also, any small improvement in the design of new reactors can also lead to substantial savings in the future.

Figure 1. 1 Trickle bed Reactor: Flow Regimes and Catalyst Wetting Conditions

The major goal of this study is to conduct systematic experimental and theoretical comparison of the performance of trickle bed reactors under different modes of operation. The first part focuses on comparison of performance of laboratory scale trickle bed reactor (TBR, down-flow) and packed bubble column (PBC, up-flow) reactors, without and with fines, to ascertain their use as test reactors for scale-up and scale-down studies based on different reaction systems (gas and liquid limited). The second part focuses on studying (theoretically and experimentally) the effect of periodic operation of trickle bed reactors on their performance and the magnitude of this effect as a function of the system used (gas and liquid limited) over a wide range of operating conditions that covers from poorly irrigated to completely wetted beds.

Thesis Organization

The thesis chapters are titled using generic chapter titles to be consistent with the thesis format. Each chapter is divided into sections that cover the first part of the study followed by sections pertinent to the second part of the work, and each section title clearly identifies to which part of the work it pertains.

## 1.1 Motivation

### 1.1.1 Comparison of Down-flow (Trickle Bed Reactor-TBR) and  Up-flow (Packed Bubble Column-PBC)  Reactors

Trickle bed reactors are packed beds of catalyst over which liquid and gas reactants flow cocurrently downwards, whereas in packed bubble columns the two phases are in up-flow. In laboratory scale trickle beds (typically few inches in diameter) packed with the commercially used catalyst shapes and sizes (the reactor to catalyst particle diameter ratio is undesirably low), low liquid velocity is frequently used in order to match the liquid hourly space velocity (LHSV) of the commercial unit. These conditions give rise to wall effects, axial dispersion, maldistribution and incomplete catalyst wetting which are not observed to the same extent in commercial reactors. Hence, in laboratory reactors, an accurate estimate of catalyst wetting efficiency is essential to determine their performance (Dudukovic and Mills, 1986, Beaudry et al., 1987). The reaction rate over externally incompletely wetted packing can be greater or smaller than the rate observed over completely externally wetted packing. This depends on whether the limiting reactant is present only in the liquid phase or in both gas and liquid phases. For instance, if the reaction is liquid limited and the limiting reactant is nonvolatile, such as occurs in some hydrogenation processes, then a decrease in the catalyst-liquid contacting efficiency reduces the surface available for mass transfer between the liquid and catalyst causing a decrease in the observed reaction rate. However, if the reaction is gas limited, the gaseous reactant can easily access the catalyst pores from the externally dry areas and consequently a higher reaction rate is observed with decreased level of external catalyst wetting (Dudukovic and Mills, 1986). Thus, the difficulties of using trickle bed reactors in laboratory scale investigation for scale-up and scale-down are mainly caused by the interactions between the gas, the liquid and the solid-catalyst phases; all of these interactions being strongly dependent on the reacting system used.

Hence, up-flow reactors are frequently used in laboratory scale studies for testing catalysts and alternative feedstocks for commercial trickle bed processes, since in them complete catalyst wetting is ensured and better heat transfer (due to continuous liquid phase), and higher overall liquid-solid mass transfer coefficients can be achieved. However, as will be shown in the present study, and as the diversity of literature results discussed herein indicate, the relative merit and the performance of up-flow and trickle beds is dependent on the reaction system used. Up-flow, as a test reactor, may not portray the trickle bed reactor performance for scale-up and scale-down for each and every reaction and operating condition. It is therefore important to investigate the comparative performance of both reactors in order to address the following important questions: a) When will up-flow outperform down-flow and vice versa?  b) When can up-flow be used to produce accurate scale-up data for trickle bed operation?

Another alternative for scale-up and scale-down studies that is practiced in industry is the use of trickle bed reactors diluted with fines (which are inert particles an order of magnitude smaller in size compared to the catalyst pellets). The lack of liquid spreading (due to the use of low liquid velocities) in laboratory reactors is compensated by fines which provide additional solids contact points over which liquid films flow. This improved liquid spreading helps achieve the same liquid-solid contacting in laboratory reactors at much lower liquid velocities as obtained in industrial units at higher liquid velocities. Fines can thus decouple the hydrodynamics and kinetics, and provide an estimate of the true catalyst performance of the industrial reactor by improving wetting and catalyst utilization in a laboratory scale unit at space velocities identical to those in industrial reactors. The diluted bed studies reported in the open literature investigated the performance of down-flow only, but did not compare it with up-flow performance (without or with fines), nor did they incorporate the impact of the reaction system (Van Klinken and Van Dongen, 1980; Carruthers and DeCamillo, 1988; Sei, 1991; Germain, 1988; and Al-Dahhan, 1993). It is noteworthy to mention that the use of fines in up-flow reactors would also eliminate the possibility of channeling, while still making use of the improved spreading due to dilution.

Most of the studies reported in the open literature deal with atmospheric pressure air-water systems and very little is available at high pressure at which the transport and kinetics may be quite different and result in completely different performance as will be shown. A recent review by Al-Dahhan et al. (1997) elaborates on high pressure hydrodynamic studies and touches upon the dearth of reaction studies at high pressure to complement them.

### 1.1.2 Unsteady State Operation of Trickle Bed Reactors

Industrial trickle bed reactors have been conventionally designed through a stepwise empirical approach and operated under steady state conditions, typically at high pressures and temperatures in order to obtain desirable reaction-transport behavior in these reactors. High pressures are used to remove or reduce the extent of gas reactant limitation and keep the by products and poisons in solution. Higher temperatures are used to improve reaction rates and fluidity (in case of petroleum feeds). In hydrogenation applications, high temperature gives the added advantage of higher hydrogen solubility and can thus reduce operating pressures. Any improvement in performance can help reduce operating pressure and temperature. The development and acceptance of alternative approaches for designing and operating existing and new reactors depends on a better understanding of their performance under different operating conditions. Thorough understanding and optimal design of trickle bed reactors is complicated by the presence of multiple catalyst wetting conditions induced by two-phase flow, which can affect the reactor performance depending upon whether the reaction is gas reactant limited or liquid reactant limited (Mills and Dudukovic, 1980). Typically, high liquid mass velocities and completely wetted catalyst are desirable for liquid limited reactions, whereas low liquid mass velocities and partially wetted catalyst are preferable in gas limited reactions. Due to the competition between the phases to supply reactants to the catalyst, the possibility of performance enhancement by operating under unsteady state conditions exists in these reactors. Unsteady state (periodic) operation can, in principle, yield better performance, reduce operating pressures, improve liquid distribution, and help control temperature (Silveston, 1990). It has not yet been used industrially as an established strategy due to lack of information on operating conditions when performance enhancement can be obtained. It is also unknown as yet as to the choice of the correct tuning parameters, which can give an optimal performance under periodic operation. Unsteady state operation has been shown to yield better performance in other multiphase reactors on an industrial scale but has not been tried on industrial trickle beds due to difficulty in prediction and control of unsteady state conditions. Industrial implementation of unsteady (periodic) operation in trickle bed reactors will follow rigorous modeling, simulation, and laboratory scale experimental investigation on test reaction systems. Unsteady state operation can also yield a better insight into the reaction-transport interactions occurring during steady state operation. Performance improvement in reactors, both new and existing ones, can have significant economic impact due to high capital costs and large capacity, particularly in the refining industry.

Unsteady state operation has not been seriously considered in design of trickle beds, due to the empirical approach without recourse to any serious modeling. But the development of advanced computational tools, and hence predictive capabilities, demands rethinking of existing strategies for design and operation. The modeling effort in unsteady state trickle bed operation has been preliminary and inconclusive (Silveston 1990) due to the large (and previously unavailable) computational effort required . This has resulted in a lack of a generalized model or theoretical analysis of the phenomena underlying unsteady state performance. Experimental data on periodic operation in laboratory scale trickle beds is limited to a very few studies. Another concern in operating trickle bed reactors under unsteady state is the problem of control which can be addressed with advanced controllers available today. It is now plausible to consider reaping the benefits which can be obtained by operating industrial trickle beds under unsteady state conditions.

The use of trickle bed reactors under unsteady state or periodic conditions is motivated by different factors depending upon the reacting system used. Before getting into details of periodic operation we must address the two scenarios under which such a strategy can be employed to achieve improvement in performance.

#### Gas Limited Reactions

These are reactions where the limiting reactant is in the gas phase, and the performance of the trickle bed is governed by the access of this gaseous reactant to the catalyst. The access of the limiting reactant, through the catalyst areas wetted by liquid, to the catalyst particle is subject to an additional resistance due to the presence of the external liquid film, and higher external wetting of the catalyst under these circumstances can be detrimental to the accessibility of the gaseous reactant to the reaction sites in the catalyst. The externally dry zones which exist at low liquid mass velocities, and the resulting incomplete external catalyst wetting, result in improved performance of the trickle bed reactor for a gas limited reaction (Beaudry et al., 1987). For this to happen, the catalyst must be completely internally wetted and replenished with liquid phase reactant from time to time to avoid liquid limited behavior to occur in these externally dry pellets. In trickle bed reactors operated at steady state conditions, this may not occur due to the fact that the externally dry pellets may not get fresh liquid reactant frequently enough, and may remain depleted of liquid reactant indefinitely, so that any advantage gained due to easier access of gas through dry areas to the particles will be negated. This can be accentuated by the localized temperature rise and local increased evaporation of the liquid reactant, and the advantage due to partial wetting may not be seen after an initial surge in reaction rate. This temporary advantage could be sustained if the catalyst were to be doused with liquid reactant periodically followed by the supply of gaseous reactant during the period of low external wetting, thus facilitating the access of the gaseous reactant to the catalyst which is internally full of liquid reactant. Hence, periodic rewetting of the catalyst may be the best way to exploit the partial wetting phenomena for maximum benefit.

It has also been observed in several cases that the liquid phase product(s) of the reaction may be the cause of decreased catalytic activity or may exhibit an inhibiting effect on the progress of reaction in catalyst pores (Haure et al., 1990). This necessitates the periodic removal of products by large amount of fresh solvent or liquid and restoration of catalytic activity.

The reactor scale maldistribution of the liquid may also cause local hot spots in the zones where liquid may not wet the catalyst under steady state conditions. The reaction rates in this zone of higher temperatures may be higher than in the zone of actively wetted catalyst area, and may result in complete evaporation of the liquid reactant and very high temperatures resulting in catalyst deactivation. This can be prevented or put to productive use in periodic operation by allowing a predetermined rise in the catalyst temperature after which introduction of fresh liquid will bring the temperature down to a lower operating level.

Clearly, for all the reasons cited above, periodic operation via liquid flow modulation, seems promising for gas phase limited reactions.

#### Liquid Limited Reactions

Many industrial trickle bed reactors operate under high pressures, typically 10-20 MPa, at which the extent of gas limitation is no longer significant due to high concentration of the dissolved gaseous reactant (high solubility at high pressure). In fact, they operate under liquid limited conditions at which the extent of external catalyst wetting is tied intimately to reactor performance. In these cases, the higher the wetting the better the performance since the rate of supply of the limiting reactant (liquid) is improved at higher external wetting efficiencies. Due to the nature of the liquid flow, it tends to occur in rivulets, which prefer to flow over externally pre-wetted catalyst areas rather than dry ones. This leads to parts of the catalyst external areas remaining dry or inactively wetted until there is a change in flow (and hence wetting) over that area. Poorly irrigated beds are a cause of concern in industrial reactors due to possible catalyst deactivation and hot spot formation. Distributors type and method of catalyst packing  play a major role in the quality of bed irrigation. The objective is an ideally wetted bed so as to wet every area of the entire catalyst by film flow. This would achieve maximum performance at a given liquid flow rate. It is also desirable to eliminate potential hot spots, by ensuring complete wetting with high liquid flow introduced periodically. This liquid would also open up multiple flow pathways for the rest of the low alternative paths to flow over the catalyst and help achieve better film-like wetting of the catalyst (even at low flow rates) resulting in higher catalyst utilization (i.e., higher conversions could be achieved in shorter bed heights, thus reducing pressure drops) and eliminating hot spots at the same time. A high flow rate slug introduced periodically would also help remove stagnant liquid pockets by supplying fresh reactants and removing products.

It can be seen from the above discussion that under both scenarios of operation it may be advantageous to consider periodic operation of trickle beds to achieve maximum performance in existing reactors as well as to set-up new reactors designed to operate under dynamic conditions. A comprehensive study of these phenomena is not available in literature for a wide range of operating conditions and is necessary to attempt industrial or even pilot scale implementation of unsteady state operation.

## 1.2 Objectives

The main objectives of both parts of this study are outlined below. Details of the implementation of experiments and modeling are discussed in Chapter 3 and Chapter 5, respectively.

### 1.2.1 Comparison of Down-flow (TBR) and Up-flow (PBC) Performance

#### I. Experimental Studies

The objectives of this part of the study are:

- Investigate the comparative performance of laboratory trickle bed (TBR) and up-flow reactors (PBC) under gas and liquid reactant limited conditions using hydrogenation of -methylstyrene to cumene as a test reaction.

- Examine the effect of operating parameters such as pressure, feed concentration liquid-solid contacting, gas velocity, etc., on upflow (PBC) and downflow (TBR) performance.

- Study the effect of bed dilution (with inert fines) on the comparative performance of upflow (PBC) and downflow (TBR) performance.

- Determine and recommend the most suitable mode of operation for scale-up and scale-down studies for trickle bed reactors at different reaction and operating conditions.

- Study the effect of high pressure on intrinsic (slurry) reaction rate and determine kinetic parameters based on these experiments.

#### II. Model Predictions

The objectives determined for this part are:

- Test the experimental data obtained in part I against predictions of the models developed at CREL by El-Hisnawi (1982) and Beaudry et al. (1987) for trickle bed and upflow reactors. Suggest improvements to the models, if necessary.

- Extend reactor and pellet scale models available in literature by incorporating effect of volatile components on the performance.

### 1.2.2 Unsteady State Operation of Trickle Bed Reactors

The goals of this part of the study can be summarized in two sub parts as follows:

#### I. Experimental Study of Periodic Operation

- Experimentally investigate the effect of liquid flow modulation (periodic operation) on the performance of a test reaction under steady state and unsteady state conditions.

- Examine the effect of reactant limitation i.e., gas and liquid limited conditions on performance. Examine the effect of operating pressure and feed concentration on performance at unsteady state conditions.

- Investigate the effect of periodic operation parameters such as total cycle period, cycle split, cycling frequency for the operating conditions at which performance enhancement is observed.

- Examine both ON/OFF and BASE/PEAK flow modulation under some of the conditions chosen on the basis of the above results.

#### II. Model Development and Solution

The objectives of this part of the study are:

- Develop a reaction-transport-flow model for trickle bed reactors which is capable of simulating unsteady state behavior and capturing the phenomena observed in the literature and our experiments on periodic operation. Quantify the enhancement in performance with respect to parameters such as cycle period, cycle split, amplitude and allowable exotherm.

- Investigate the distribution of velocities and liquid holdup during periodic operation using multiphase flow codes (CFDLIB of Los Alamos), and solution of one dimensional momentum equations (for both gas and liquid phase) to illustrate the qualitative picture of the key phenomena and provide a physical basis for the model.

# Chapter 2. Background

## 2.1 Laboratory Reactors – Performance Comparison and Scaleup Issues

Literature studies on scale-up of trickle bed reactors have mainly followed an empirical approach. Preliminary reaction studies with the system of interest are usually conducted in slurry and basket reactors, in the later conditions assuming full wetting of the catalyst are used. This is followed by tests in packed beds with either downflow or upflow with the same size of catalyst to be used in industrial reactors. Simple models have also been used to estimate the effect of the flow pattern and catalyst wetting. Such models account for the effect of mass velocities on transport coefficients and external wetting in evaluating the conversion expected in large scale reactors upon scaleup. No systematic study has been reported which compares the performance of down-flow and up-flow operation over a wide range of operating conditions, particularly reactor pressure. The few studies that are available in the open literature (listed in Table 2-1 and discussed below) do not relate the observed performance to the type of reaction system used (gas-limited or liquid-limited), nor do they conclusively elucidate which is the preferred reactor for scale-up/scale-down.

### 2.1.1 Literature Review: Upflow vs. Downflow Comparison

Goto and Mabuchi (1984) demonstrated that for atmospheric pressure oxidation of ethanol in presence of carbonate, down-flow is superior at low gas and liquid velocities but up-flow should be chosen for high gas and liquid velocities. Beaudry et al. (1987) studied atmospheric pressure hydrogenation of -methylstyrene in liquid solvents at high liquid reactant concentrations and observed the down-flow performance to be better than up-flow except at very high conversion. Mazzarino et al. (1989) observed higher rates in up-flow than in down-flow for ethanol oxidation and attributed the observed phenomenon to better effective wetting in up-flow without considering the type of reaction system (gas or liquid limited). Liquid holdup measurements at elevated pressure using water/glycol as liquid with H2, N2, Ar, CO2 as the gas phase by Larachi et al. (1991) indicate that liquid saturation is much greater in up-flow than in downward flow at all pressures (up to 5.1 MPa). Lara Marquez et al. (1992) studied the effect of pressure on up-flow and down-flow using chemical absorption, and concluded that the interfacial area and the liquid side mass transfer coefficient increase with pressure in both cases. Goto et al. (1993) observed that down-flow is better than up-flow at atmospheric pressure (for hydration of olefins), and noted that the observed rates in down-flow were independent of gas velocity while those in up-flow were slightly dependent on it. Thus, there is no clear guidance as to which reactor will perform better for a given reaction system. A systematic study of the effect of operating conditions is necessary to understand the interplay of factors in the particular reacting system in order to explain why these reactors perform differently and whether up-flow can be used for scale-up of trickle bed reactors. This study provides the rationale behind the results reported in the literature and leads to the rules by which to 'a priori' judge whether an up-flow or down-flow reactor is to be preferred for laboratory testing.

### 2.1.2 Criterion for Gas and Liquid Reactant Limitation

The performance of up-flow and down-flow reactors depends upon the type of reaction, i.e., whether gas (reactant) limited or liquid (reactant) limited. A simple and usable criterion for establishing gas or liquid limitation is needed. In order to obtain such a criterion for the complex processes involved, a step by step comparison of the different transport processes contributing to the observed rate, as illustrated below, is required. For a typical reaction   A(g) + bB(l) = Products(l), the limiting step can be identified by first comparing the estimated rates of mass transfer with the observed reaction rates. The estimated volumetric mass transfer coefficients for the system under study can be evaluated from appropriate correlations in the literature (e.g., (ka)GL from Fukushima and Kusaka (1977), and kLS from Tan and Smith (1982) or Lakota and Levec (1989) listed in Appendix B). The comparison of maximum mass transfer rates, with the experimentally observed rates, (rA)obs as per inequality (2.1), where CA* is the gas solubility at the conditions of interest, confirms that external gas reactant mass transfer does not limit the rate, if the strong inequality (2.1) is satisfied (by strong inequality, we mean that the left hand side is several fold (3-5) larger than the right hand side).

kaGL>>(rA)obsCAkLSaLS>>(rA)obsCA									(2. 3)

The observed rate in the above criterion is the mean rate for the reactor evaluated  from the overall mass balance on the system. For systems where the conversion space-time relationship is highly nonlinear, criterion (2.1) should be applied both at the entrance and at the exit conditions of the reactor. If inequality (2.1) is satisfied, further comparison of the effective diffusion fluxes terms with the observed rate can help in identifying the limiting reactant. This can be achieved by evaluation of the Weisz modulus (fWe= (rA)obs(VP/SX)2/(DeC)), (where DeC is the smaller of the two, DeBCBi/b or DeACA*) (which for our reaction system yielded fWe > 1 (see Table 2.1)). In order to identify the limiting reactant in case of fWe > 1, the diffusion fluxes of the two reactants should be compared (Doraiswamy and Sharma, 1984), whereas for fWe < 1, it is the ratio of the liquid reactant concentration and the dissolved gas reactant concentration that counts. The ratio (g = (DeB CBi ) /b(DeA CA* )) is indicative of  the relative availability of the species at the reaction site. Thus, a value of g >> 1 implies gaseous reactant limitation, while g << 1 indicates liquid reactant limitation for the conditions mentioned above. This criterion, which is again regarded as a strong inequality, is relied on for analyzing our results as well as the literature data.

The limiting reactant in a gas-limited reaction can enter the porous particles through both the actively and inactively wetted surfaces, but it enters at different rates (Mills and Dudukovic, 1980). Accordingly, for a gas limited reaction, the trickle bed reactor is expected to perform better, due to its partially wetted catalyst over which gas reactant has an easy access to the particles, than the up-flow reactor in which the only access of the gaseous reactant to the catalyst is through the liquid film engulfing the catalyst. For a liquid-limited reaction, the liquid reactant can only enter the catalyst particle through its actively wetted surface, leaving the inactively wetted areas unutilized. Liquid limited conditions, therefore, result in a better performance for up-flow, where particles are completely surrounded by liquid, than for down-flow, where particles may be only partially wetted.

The literature data confirm the above assertion and give support to the use of the proposed criterion for identifying the limiting reactant (values of fWe and g are listed in Table 2.1). The experimental data of Goto and Mabuchi (1984) have g values around 300 (approximate estimate using reported concentration and molecular diffusivities), which indicate clearly a gas limited behavior, and so their observation that down-flow performs better than up-flow at low liquid and gas velocities is a forgone conclusion. Beaudry et al. (1987) operated under g values ranging between 20 and 100, and again down-flow outperformed up-flow, except at very high conversion when g was lower than 3 (i.e., of order one) and liquid reactant limitations set in, as explained by Beaudry et al. (1986), in which case up-flow tends to perform close to down-flow. The range of g values (g=0.5 to 17) encountered by Mazzarino et al. (1989), indicate both liquid and gas limited regimes. At one set of conditions (g=0.5) liquid limitation is indicated based on our criterion and the fact that upflow outperforms down-flow can be anticipated. However Mazzarino et al. (1989) report that upflow performs better than downflow even at g=17, which should be a gas limited reaction. It may be noted that their experiments in down-flow (Tukac et al., 1986) and up-flow (Mazzarino et al., 1989), while using presumably the same catalyst, were not performed at the same time. Our experience with active metals on alumina catalyst (Mazzarino et al. used Pt on alumina) is that exactly the same state of catalyst activity is very difficult to reproduce after repeated regeneration. This sheds some doubt whether the two sets of data can be compared. In addition, at the temperature used, ethanol will not behave as a non-volatile reactant and does not satisfy the conditions of our hypothesis. Goto et al. (1993) report down-flow performance superior to up-flow for their reaction system, for which estimates of g yield values of over 8000, which is why any short circuiting of the gas via the dry areas yields higher gas transfer rates to particles and hence higher conversion in down-flow.

Table 2- Identification of the Limiting Reactant for Literature and Present Data

| Authors | Reaction System | Rate(obs) mol/m3.s | CA* mol/m3 | Cbi mol/m3 | Weisz Modulus (fWe) | Gamma (g) | Limiting Reactant |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Goto (1984) | Oxidation of ethanol in presence of carbonate | 1.50E-02 | 0.977 | 600 | 39.5 | 314 | Gas |
| Beaudry (1987) | Hydrogenation of alpha-methylstyrene | 1.7 | 3.76 | 1700 | 21.0 | 92 | Gas |
| Mazzarino (1989) | Ethanol oxidation |  |  |  |  |  |  |
| Extreme I* |  | 5.0E-04 | 0.55 | 0.55 | 1.52 | 0.51 | Liquid |
| Extreme II* |  | 3.0E-02 | 1.66 | 55.31 | 15.3 | 17 | Gas |
| Goto(1993) | Oxidation of ethanol in presence of carbonate | 1.0E-02 | 12.0 | 55000 | 0.04 | 10300 | Gas |
| Khadilkar (1996) Wu (1996) | Hydrogenation of alpha-methylstyrene |  |  |  |  |  |  |
| Extreme I** |  | 1.4 | 14.0 | 520 | 107.7 | 8.8 | Gas |
| Extreme II** |  | 1.1 | 63 | 273 | 21.4 | 0.87 | Liquid |

Extreme I* : Low Liquid Reactant Feed Concentration, Atmospheric Pressure.

Extreme II* : High Liquid Reactant Feed Concentration, Atmospheric Pressure.

Extreme I** : High Liquid Reactant Feed Concentration, Low Pressure.

Extreme II**: Low Liquid Reactant Feed Concentration, High Pressure.

## 2.2 Literature on Unsteady State Operation of Trickle Bed Reactors

The literature dealing with unsteady state behavior in chemical systems was reviewed by Silveston (1990) and classified into two categories: (i) parameter forcing, and (ii) flow reversal. Most of the investigations summarized in his review were parameter forcing studies on stirred tanks, heat exchangers, ion exchangers, and a few flow reversal studies on gas-solid fixed beds, adsorbers and fractionators. On the basis of the observations on these systems, several strategies were suggested for possible performance enhancement in trickle bed reactors (Silveston, 1990), such as modulation of (i) flow, (ii) composition, or (iii) catalyst activity.  Some of these have been considered in the few studies that have appeared in past decade (as summarized in Table 1). However, none of these strategies has been implemented commercially due to a number of factors: (i) lack of knowledge of the transport-kinetic parameters in processes of interest under dynamic conditions, (ii) lack of an established methodology and set of operating parameters for implementation of periodic operation, and (iii) apprehensions about operation and control of large scale continuous reactors under transient conditions. Rigorous experimental and modeling effort is necessary to understand the phenomena underlying unsteady state operation in order to establish guidance for commercial implementation. This study is a step in that direction and represents an attempt to expand the knowledge and understanding of unsteady state behavior of trickle bed reactors.

The investigation of periodic operation in trickle beds is a relatively unexplored area as very few studies reported in the open literature have been conducted to test different strategies with which a performance improvement can be achieved. Most of the reported investigations consist of a few experimental runs of isolated systems. They do not consider quantitatively the extent of improvement achieved in each reaction system, nor do they attribute it to a corresponding quantitative change in the manipulated variables or controlling factors. For that reason we have tried to systematically review the available information and classify it as much as possible. Our efforts in that direction are explained below.

### 2.2.1 Strategies for Unsteady State Operation

Careful examination of the experimentation with periodic operation undertaken in the literature, and the corresponding observed enhancement, leads us to classify the strategies used into three categories.

I. Flow Modulation

a) Isothermal

1. Improvement in wetting and liquid distribution

2. Introduction of dry areas to improve direct access of gaseous reactants.

b) Adiabatic/Non Isothermal

1. Allow a controlled increase in temperature and, hence, enhance rate of reaction

as well as evaporation of external liquid film.

II. Composition Modulation

- This involves pulsing the feed composition by adding another component

which changes reactant ratios to favor one particular reaction (in case of multiple reactions). It can also be used to prevent the reaction from becoming a gas phase reaction by quenching with an inert component or product of the reaction to still operate the reactor under semi-runaway conditions. This is done typically under adiabatic conditions.

III. Activity Modulation

a) Enhance activity by pulse feeding a selected component.

b) Removal of product from catalyst site to prevent deactivation or regenerate

catalyst sites by use of a pulse fed component.

Although the focus of the present study is primarily on strategy I, it is important to consider all the above in some detail. In order to understand the factors causing the actual enhancement, the literature results need to be examined in the light of gas and liquid limited reactions, as introduced earlier. In case of a gaseous reactant limitation, the necessity to obtain complete internal wetting to prevent hot spots and rejuvenation of the liquid in each particle in addition to the need for externally dry areas indicates a contradictory situation for steady state operation. However, this can be achieved using periodic operation by replenishing the liquid reactant via periodic liquid pulses and ensuring internal wetting. Then the liquid flow must be stopped in order to allow for the gaseous reactant to enter the catalyst pores unhindered by external liquid film, which means that the liquid feed pulse should be followed by a zero liquid feed interval. This understanding is crucial to the modeling to be undertaken to achieve realistic predictions for any given reaction system, as well as to obtain an optimal performance out of the complex cause-effect relationships that result in the enhancement of performance. The terminology used in studies of periodic operation must be noted here, i.e. the total time duration between subsequent introduction of liquid is referred to as cycle time (or cycle period, denoted as t) and the part when the liquid flow is active is referred to as the ON part (denoted by t, where  is the fraction of total time corresponding to the ON part) and the rest as the OFF part (corresponding to (1-)t). Reported experimental studies of unsteady state operation in trickle bed reactors are summarized in Table 2.2 and only the key observations are briefly discussed here. Haure et al. (1990) and Lee et al. (1995) studied periodic flow modulation of water in SO2 oxidation on carbon catalyst to obtain concentrated sulfuric acid from dilute SO2 in air stream. They observed an enhancement in supply of SO2 and O2 to the catalyst during the dry cycle, resulting in higher performance (Figure 2-1), and a temperature rise of 10-15 oC (Figure 2-2) during this time interval. Based on intrinsic kinetics of SO2 oxidation, the rate of reaction is first order with respect to the gaseous reactant (O2), and the reaction results in formation of SO3, which remains adsorbed on the catalyst until it is washed by the pulse of water, resulting in sulfuric acid formation as well as restoration of the catalytic activity (strategy IIIb). Lange et al. (1994) experimentally investigated the hydrogenation of cyclohexene, and  the hydrogenation of alpha-methyl styrene on Pd catalysts by manipulation of the liquid feed concentration and feed rate, respectively. They used non-isothermal composition modulation of cyclohexene to control conversion and keep the reaction system from switching from a three phase system to a two phase one, and, designed their total cycle time based on this switching time which was determined experimentally. For the case of hydrogenation of alpha-methylstyrene under isothermal conditions, the authors observed maximum improvement at a cycle period of 8 min. at cycle split of 0.5 (Figure 2-3). The observed improvement (between 2 and 15 %) was attributed to better wetting due to the liquid pulse (and removal of stagnant liquid).  Castellari and Haure (1995) attributed the performance enhancement observed in their hydrogenation experiments to large temperature rise during the OFF part of the cycle. They also observed a gas phase reaction at semi-runaway conditions and a large enhancement resulting from the high gas phase reaction rates.

Table 2- Experimental Studies on Unsteady State Operation in Trickle Beds

| Author(s) | System Studied | Modulation Strategy | Liquid and Gas flow rates | Cycle Period () and Split () | % Enhancement |
| --- | --- | --- | --- | --- | --- |
| Haure et al. (1990) | SO2 oxidation | Flow (Non Isothermal) | VL=0.03-1.75 mm/s VG=1-2 cm/s | =10-80 min (=0.1,0-0.5) | 30-50 % |
| Lange et al. (1994) | Cyclohexene hydrogenation | Composition (Non isothermal) | QL=80-250 ml/h Conc=5-100 % |  = up to 30 min. (=0.2-0.5) | 2-15 % (Temp Rise =30oC ) |
|  | -MS hydrogenation | Liquid Flow (Isothermal) | QL=0-300 ml/h, QG= 20 l/h |  = 1-10 min. (=0.25-0.5) |  |
| Lee et al. (1995) | SO2 oxidation | Adiabatic Flow modulation | VL=0.085-0.212 cm/s, SVG=1000 h-1 |  = up to 60 min (=0.02-0.1) | Temperature Rise = 20 oC |
| Castellari (1996) | -MS hydrogenation | Non Isothermal | QL=2.27 ml/s QG=900 ml/s |  = 5 to 45 min (=0.3-0.5) | 400 % (Temp. Rise = 35oC) |

Most of the studies reported in the open literature are restricted to gas limited conditions with a very limited number of data available for the few studies published. Periodic operation under gas limited conditions can ensure completely internally wetted catalyst pellets, direct access of gaseous reactant to the external catalyst surface, replenishment of catalyst with liquid reactant, periodic removal of products by fresh liquid, and quenching of a predetermined rise in temperature. Under liquid limited conditions, external wetting and liquid supply to the particles is crucial. It is believed that periodic operation can reduce and eliminate liquid maldistribution, ensure a completely irrigated bed, and, quench developing hotspots. Several industrial reactors are operated under liquid limited conditions at high pressure and suffer from liquid maldistribution, which can cause externally dry or even internally completely dry catalyst pellets. At high liquid and gas mass velocities, in the pulsing flow regime, a significant improvement in catalyst wetting and effective removal of hot spots has also been reported (Blok and Drinkenburg, 1982). The use of this regime is not always practical in industrial reactors due to a large pressure drop and little control over the slugging process. Periodic flow modulation with a low base flow and a periodic slug of very high liquid flow referred to as BASE-PEAK flow modulation can improve catalyst utilization even at low mean liquid flows (lower pressure drop) and still achieve temperature and flow control due to artificially induced pulses (or slugs). No study has been reported in the open literature on periodic operation for liquid limited reactions or on translation of lab scale reactor unsteady state performance data to large reactors. Some industrial processes do employ periodic localized quenching of hot spots by injection of cold fluids at several axial locations in the reactor (Yan, 1980).

Figure 2- Time Averaged SO2 Oxidation Rates of Haure et al. (1990)

Figure 2- Experimental and Predicted Temperature Profiles of Haure et al.  (1990)

Figure 2- Enhancement in Periodic Operation Observed by Lange et al. (1993)

## 2.3 Review of Models for TBR Performance

This section briefly reviews trickle bed reactor models for steady and unsteady state performance prediction by discussing their main assumptions and key features. The list of contributors to model development of trickle bed reactors is presented in Table 2-3 and Table 2-4 for steady models and Table 2-5 for unsteady state models. This list is by no means exhaustive and covers only the major contributions since 1980.

### 2.3.1 Steady State Models

Steady state models that are available in the open literature for predicting trickle bed reactor performance were developed primarily for laboratory scale test reaction systems (as listed in Table 2.3). Most of the trickle bed reactor models considered a pseudo-homogeneous approach based on plug flow (Collins et al., 1984; Kheshgi et al., 1992) or a heterogeneous approach based on plug flow for gas and liquid phase (El-Hisnawi et al., 1982; Mills and Dudukovic, 1984; Hekmat and Vortmeyer, 1994; Rajashekharam et al., 1998). Some models accounted for liquid flow non-uniformity and maldistribution by using an axial dispersion model (Chu and Ng, 1986). Most investigations dealt with hydrogenation or oxidation in pure or moderately concentrated organic or aqueous solutions (large excess of liquid reactant) and, hence, mainly considered zero order rate with respect to the liquid reactant and first order with respect to gaseous reactant. Liquid reactants/solvents were assumed to be non-volatile and gas phase assumed to be pure at constant partial pressure of the reacting gas. Thus, the primary model variables of interest have been the liquid phase concentrations of the dissolved gaseous reactant and resulting conversion of the liquid phase reactants. The key effect that was incorporated in most recent models was that of partial wetting and transport of gaseous reactant to externally dry areas of the catalyst resulting in higher rates observed in most of the experimental data (El-Hisnawi et al., 1982; Sicardi, 1984; Berruti et al., 1984; Ruzicka and Hanika, 1993). Some models considered non isothermal effects and used a pseudo-homogeneous energy balance to solve for the temperature at any axial location (Yang and Li, 1992; Harold and Watson, 1993; Rajashekharam et al., 1998). Other variants include a cell model (Brad Sims et al., 1994), a cross-flow model (Tsamatsoulis and Papaynnakos, 1995) , and some others based on liquid flow mal-distribution (Funk et al., 1990) or stagnant liquid zones in the reactor (Rajashekharam et al., 1998).

Pellet scale diffusion with reaction was studied by taking reactant limitation in to account in simpler versions (Beaudry et al., 1987) and, in the general case, by considering both gas and liquid phase reaction zones and solution of the gas-liquid interface by considering liquid imbibition, pore filing and capillary condensation in a partially internally wetted pellet (Harold and Watson, 1993).  Approximate solutions from gas-solid catalyst level equations have also been verified by numerical solution for n-th order as well as Langmuir-Hinshelwood type of kinetics  (Lemcoff et al., 1988).

Table 2- Review of Recent Steady State Reaction Models for Trickle Bed Reactors

| Authors | Model Features | Reaction System | Remarks |
| --- | --- | --- | --- |
| El-Hisnawi et al. (1982) | Heterogeneous, plug flow of each phase, effect of contacting efficiency, Rate enhancement, gas limited conditions | -methylstyrene hydrogenation | Effect of external incomplete contacting on rate enhancement shown |
| Mills and Dudukovic (1984) | Heterogeneous, plug flow of each phase, isothermal, partial wetting effect | -methylstyrene hydrogenation | Comparison of earlier models illustrated |
| Collins et al. (1984) | Isothermal, homogeneous model with gas-liquid at equilibrium | Hydrodesulfurization of benzothiophene | Effect of solvent vaporization considered |
| Sicardi (1984) | Model for volatile reactant |  | Developed correlation for higher rate due to volatile reactant on partially wetted catalyst |
| Berruti et al. (1984) | 2 models developed - Two zone model (wet and dry) - Single zone model (wet) | SO2 oxidation on carbon | Good prediction of experimental rate and liquid velocity effect |
| Ring and Missen (1984) | Multizone external wetting model for catalyst pellet | Hydrodesulfurization of dibenzothiophene | Better predictions than 2 zone model shown and effect of liquid velocity on overall effectiveness predicted. |
| Herskowitz (1985) | Plug flow, isothermal, partial wetting | Xylose hydrogenation | Only gas limited conditions considered |
| Chu and Ng (1986) | Dispersion model for trickling and pulsing flows |  | Dispersion coefficient from Monte Carlo simulation used |

Table 2-3 Review of Recent Steady State Reaction Models for Trickle Bed Reactors (continued)

| Authors | Model Features | Reaction System | Remarks |
| --- | --- | --- | --- |
| Beaudry et al. (1987) | Pellet Scale diffusion effects (multiple wetting conditions) | -methylstyrene hydrogenation | Effect of poor wetting and liquid reactant limitation at low liquid flow considered for a gas limited reaction |
| LaVopa and Satterfield (1988) | Alternate CSTR’s and flash units Completely wetted catalyst | Hydrodeoxygenation of dibenzofuran and hydrogenation of n-butylbenzene | Volatility effects considered, but wetting changes not accounted |
| Lemcoff et al. (1988) | Pellet scale effectiveness factor models | Ethanol Oxidation | Numerical validation of approximate solutions for effectiveness factor |
| Funk et al. (1990) | Computer generated 2-D bed with flow distribution, intraparticle effects |  | Effect of flow mal-distribution effects and bed features |
| Kheshgi et al. (1992) | Pseudo-homogeneous model, non isothermal effects, bed drying | Cyclohexene hydrogenation | Hysteresis and multiplicity due to dry and wet operation predicted |
| Yang and Li (1992) | Multizone reaction model with stochastic flow distribution | Hydro-dewaxing | Effect of feed temperature and flow well predicted |
| Ruzicka and Hanika (1993) | Vertical string of pellets, decoupled hydrodynamics from mass and heat transfer | Cyclohexene hydrogenation | Three possible steady state examined, a wetted, dry and mixed phase reaction considered |
| Harold and Watson (1993) | Pellet scale equations considering partial filling, imbibition, condensation and evaporation in catalyst pore. | Hydrazine decomposition, Alpha-methylstyrene hydrogenation | Partial internal wetting, nonisothermal effects and hysteresis predicted |
| Brad Sims et al. (1994) | Isothermal, two-region cell model with partial wetting | Decomposition of hydrogen peroxide | Flow regime and liquid velocity effects well predicted |

Table 2-3 Review of Recent Steady State Reaction Models for Trickle Bed Reactors (continued)

| Authors | Model Features | Reaction System | Remarks |
| --- | --- | --- | --- |
| Sundmachar and Hoffmann (1994) | Multicomponent distillation model, acid catalyzed reaction on packing | MTBE synthesis | Solid-liquid and gas-liquid transport effects examined |
| Hekmat and Vortmeyer (1994) | Differential mass balance model | Aerobic Oxidation of ethanol and alkylated benzenes | Both gas (oxygen) limitation and substrate limitation well predicted |
| Tsamatsoulis and Papayannakos (1995) | Cross-flow model with parameters obtained from step response tests | Hydrotreating |  |
| Khadilkar et al., Wu et al. (1996) * | Reactor and pellet scale models with pressure dependent intrinsic kinetics | Alpha-methylstyrene hydrogenation | Performance at both gas and liquid limited extremes predicted |
| Khadilkar et al., Jiang et al. (1998) | Reactor scale plug flow model for network of reactions | Hydrogenation Aryl nitro-alcohol | Network kinetic parameters fitted on the basis of reactor data and simulated concentrations |
| Toppinen et al. (1996) | Plug flow, non-isothermal, multicomponent transport | Toluene hydrogenation | Comparison of multicomponent and effective (lumped) transport considered |
| Korsten and Hoffmann (1996) | Film theory based model, wetting effects, LH kinetics | Hydrodesulfurization of vacuum gas oil | Effect of wetting on pilot and industrial performance well predicted |
| Valerius et al. (1996) | Multi-pellet and cylindrical shell model | 3-hydroxypropanol hydrogenation | Comparison of pilot reactor data with bed effectiveness from the 2 models |
| Rajashekharam et al. (1998) | Plug flow, non-isothermal, wetted, dry, and stagnant zones | Hydrogenation of 2,4 dinitrotoluene | Temperature rise, hysteresis behavior well predicted |

* This forms part of the present thesis work.

### 2.3.2 Literature on Models for Volatile Systems

Although literature on trickle-bed reactors is abundant, most of the studies consider isothermal conditions, non-volatile liquid reactants, and constant phase velocities and holdups. These simplifying assumptions are not suitable for the systems considered here. Hence, studies that did not consider volatility, temperature, and related effects have been excluded from the present discussion. Table 1 summarizes the studies of trickle bed reactor with volatile components, each of which is discussed and analyzed further in this section.

In one of the earliest investigations in the field of reactions in trickle bed with volatile components, Hanika et al. (1975, 1976) studied the hydrogenation of cyclohexene using a 3% palladium on charcoal catalyst at atmospheric pressure and observed several interesting phenomena in their steady state experiments. They divided their results into three categories: (1) liquid phase reaction with low conversion and a small temperature rise; (2) gas phase reaction with high conversion and a large temperature rise; and (3) intra-reactor transition from liquid phase to gas phase reaction with hysteresis and steady state rate multiplicity. Hanika et al. (1977) also presented some experimental results for the hydrogenation of 1,5-cyclooctadiene, and examined the thermal behavior under transient conditions when the hot spot and the dry zone was seen to start at the reactor exit and proceed backwards towards the reactor inlet to its steady state location.

Mills and Dudukovic (1980) modeled the effect of contacting efficiency and Biot number for the case of first-order gas or liquid limited reaction with volatile liquid reactant for a slab, cylindrical, and spherical catalyst pellet geometries and obtained series solutions for an overall catalyst effectiveness factor.

Kim and Kim (1981a, 1981b) examined the hydrogenation of cyclohexene using Pt on alumina in liquid and gas filled pellets by exposing catalyst pellets with a vapor containing cyclohexene and studied the influence of hydrogen partial pressure. They observed rate multiplicity corresponding to the following three states of the catalyst: (1) vapor filled micro and macro pores and gas phase reaction; (2) vapor filled macropres and liquid filled micropores (liquid phase reaction); and  (3) a combination of the two states corresponding to an intermediate overall rate.

Collins et al. (1985) studied hydrodesulfurization of benzothiophene at a constant pressure of 68 atm and a temperature range of 546 to 606 K by simulating the global reaction rate in the presence of phase equilibrium effects at each point. The latter was predicted using the Soave-Redlich-Kwong equation of state. They simulated the rate with four solvents with different volatility to study the influence of relative solvent to reactant volatility on the reaction rate, and concluded that liquid phase rates were reduced, unaffected, and increased with lower, almost equal, and higher relative volatility, respectively.

Kocis and Ho (1986) modeled their reaction by assuming a series of plug flow reactors where each was preceded by a vapor-liquid separator where local thermodynamic equilibrium was re-established after the reaction stage. They tested their model for the hydrodesulfurization of dibenzothiophene at 325 oC and 3.15 MPa. They assumed that liquid flow was not affected by the evaporation of solvent and that catalyst particles were completely covered by liquid. These may be valid for their reaction system at dilute liquid reactant concentration, but generally are not observed for situations with higher volatility and high temperatures.

LaVopa and Satterfield (1988) studied the effect of volatility using two test reactions, namely, the hydrodeoxygenation of dibenzofuran and hydrogenation of n-butylbenzene. Their reactor model assumed a series of stirred tanks with both liquid and vapor flows going in and out of each stage analogous to a distillation tray. They examined the effect of changing the gas/liquid feed ratio on the conversion in the trickle flow regime with liquid only. They also studied the complete vapor regime in presence of solvents with different volatility (hexadecane and squalene) when compared to the reactant. They observed an increase in conversion with a decrease in the gas/liquid ratio due to an increase in liquid reactant partial pressure.

Harold (1988) and Harold and Watson (1993) studied the vaporization in a partially wetted and partially filled pellet for a decomposition reaction (hydrazine decomposition) and a bimolecular reaction (cyclohexene or alpha-methylstyrene hydrogenation). The key features of their model involved the solution of reaction diffusion equations for the gas filled and liquid filled part of a catalyst pellet. The gas-liquid interface inside the catalyst was determined by the simultaneous solution of the liquid imbibition velocity equation written as a combination of Darcy’s law and a capillary pressure difference given by the Young-Laplace equation. They determined the catalyst effectiveness factor for their model slab pellet and showed that as catalyst activity is increased, the transition between a mostly liquid filled, low rate state, to a mostly gas filled, high rate state, may be single valued or in form a hysteresis loop. They predicted the existence of multiplicity over a large parameter space (gas/liquid flow ratios, catalyst activity, activation energy etc.,). Their model is the most rigorous model available for prediction of pellet scale phenomena, such as capillary condensation, evaporation, and their influence on gas-liquid interface and reaction rates at the pellet scale. Further discussion of the model equations is included in the model development section of this report. Watson and Harold (1994) also experimentally investigated cyclohexene hydrogenation in a single pellet under gas pre-filled, and liquid pre-filled conditions to examine steady state multiplicity features and proposed bifurcation diagrams for the observed high and low rates observed at different hydrogen feed fraction and catalyst activity.

Bhatia (1988) and Jaguste and Bhatia (1991) theoretically demonstrated the effect of capillary condensation for a vapor mixture containing a volatile and an inert component and examined the possibility of internal recondensation of the volatile reactants and products if the vapor phase is near or at saturation.

Kheshgi et al. (1992) developed a pseudo-homogeneous model to predict conversion and temperature profiles for a gas-liquid mixture. The model was based on fitted rate and wetting coefficients to predict temperature and conversion profiles as well as the ignition and extinction points and multiplicity as observed in Hanika et al.’s (1976) experiments.

Toppinen et al. (1996) used a multicomponent transport approach proposed by Taylor and Krishna (1993) for energy and mass transport to solve a steady state plug flow model for the hydrogenation of toluene. Although their model is rigorous in terms of the equations for mass and energy transport, they have ignored the effects of volatilization on the observed rates and assumed the catalyst pellets to be completely wetted with an apparent rate expression used to solve for liquid-sold interface equations.

Table 2- Summary of Literature on Trickle Bed Reactor Studies with Volatile Components

| Authors | Reaction System | Nature of the Study | Key Features / Observations |
| --- | --- | --- | --- |
| Hanika et al. (1975), Hanika et. al. (1976) | Cyclohexene hydrogenation | Experimental | Dry and wet rates, multiplicity. Measurements on Reactor scale |
| Hanika et al. (1977) | 1-5 Cyclooctadiene hydrogenation | Experimental | Transient thermal behavior and progress of dry zone studied |
| Mills and Dudukovic (1980) | Alpha-methylstyrene | Model | Effect of gas and liquid limited conditions with volatile liquid examined for contacting and Biot number dependence |
| Kim and Kim (1981a), Kim and Kim (1981b) | Cyclohexene hydrogenation | Experimental and Modeling | Pellet scale measurements and model. Three states observed |
| Collins et al. (1985) | Hydrodesulfurization of benzothiophene | Experimental and Modeling | Effect of solvent volatility on rate was shown to be significant |
| Kocis and Ho (1986) | Hydrodesulfurization of benzothiophene | Modeling | Assumed that liquid flow not affected by evaporation |
| LaVopa and Satterfield (1988) | Hydrodeoxygenation of dubenzofuran and hydrogenation of n-butylbenzene | Experimental and Modeling | Effect of solvent and gas/liquid feed ratio was demonstrated. |

Table 2-4 Summary of Literature on Trickle Bed Reactor Studies with Volatile Components (continued)

| Authors | Reaction System | Nature of the Study | Key Features / Observations |
| --- | --- | --- | --- |
| Harold (1988) and Harold and Watson (1993) | Hydrazine decomposition, Alpha-methylstyrene | Modeling | Pellet scale equations considering partial filling, imbibition, condensation and evaporation in catalyst pores. |
| Bhatia (1988) and Jaguste and Bhatia (1991) |  | Experimental and Analysis | Effect of capillary condensation |
| Kheshgi et al. (1992) | Cyclohexene hydrogenation | Modeling | Prediction of Hanika et al. Experimental results. |
| Toppinen et al (1996) | Toluene hydrogenation | Modeling | Multicomponent effects considered, but no volatility effects. |

### 2.3.3 Unsteady State Models for Trickle Bed Reactors

The earliest reported unsteady state modeling approach relied on a plug flow equilibrium assumption for predicting the hotspot formation and movement  during startup of a trickle bed and investigated the introduction of a gas /liquid quench stream at an axial location to quench the developing hotspot (Yan, 1980). Pseudo transient behavior was also modeled by considering similar equations (Warna and Salmi, 1996; Sundmachar and Hoffmann, 1994). Mass transfer terms are considered in extensions of these models to predict periodic variation of temperature and concentration (Haure et al., 1990; Stegasov et al., 1994). Spatial variation terms were dropped in some variations of this model to study time variation of mass transfer coefficients and enhancement in rates and selectivity for model reaction system (Wu et al., 1995), and by addition of catalyst wetting effects during periodic operation (Gabarain et al., 1997). This was done primarily to reduce computational complexity. Activity modulation was incorporated in recent transient models for optimizing performance on the basis of catalyst activity (Yamada and Goto, 1997).

The above models capture the time variation of the reactor temperature fairly well but are lacking in their ability to simulate dynamic concentration variation, multicomponent effects, evaporation, condensation. They also ignore the coupling between mass transfer and hydrodynamics due to the assumption of a specified (or no) variation of holdup and phase velocities in temporal and spatial domains. There is a need to incorporate the factors listed above and develop codes for ‘real’ industrial reaction systems. This development has been limited so far by the lack of computational power sufficient to handle realistic industrial systems especially under dynamic operating conditions.

Table 2- Review of Unsteady State Models for Trickle Bed Reactors

| ##### Authors | Model Features | Reaction System | Remarks |
| --- | --- | --- | --- |
| Yan (1980) | Plug flow, gas-liquid equilibrium, apparent rate on catalyst | Hydrocracking | Temperature profiles with and without quenching predicted |
| Haure et al. (1990) | Non-isothermal plug flow model with liquid ON-OFF, mass transfer controlling the rate | SO2 Oxidation | Thermal waves and temperature rise compared with experimental observations |
| Stegasov et al. (1994) | Non-isothermal plug flow with liquid ON-OFF, mass transfer control | SO2 Oxidation | Temperature and concentration profiles predicted |
| Wu et al. (1995) | Local transient model with multiple reaction, mass transfer variation with high frequency | Three model systems of multiple series and parallel reactions | Rate enhancement with varying frequency examined for natural pulsing |
| Warna and Salmi (1996) | Parabolic PDE’s for gas and liquid phases, pseudo-transient simulation to steady state | Toluene hydrogenation, SO2 Oxidation | Reactor dynamics from start-up to steady state studied |
| Yamada and Goto (1997) |  | Hydrogenation to amino acid | Optimal cycle period for product removal and catalyst regeneration calculated |
| Gabarain et al. (1997) | Three zone model, completely wetted, externally dry and internally dry pellets | Hydrogenation of -methylstyrene | Variation of rate enhancement with cycle period predicted |

The level of complexity and essential features available in the steady state models in the literature are sufficient for evaluation of steady state experiments outlined in the objectives in Section 1.2. These models are still far from mimicking reality in industrial hydrocracking and hydrotreating applications both under steady and dynamic operation due to the following general shortcomings (although some of the literature models have addressed selectively some of the issues mentioned below):

- Most of the models consider single, simple reactions and solve the reaction transport of only the limiting species by keeping other concentrations constant. In contrast, in many industrial reactors, a large set of reactions is typically occurring simultaneously.

- Multicomponent transport between the phases has not been considered in most models. This means that net mass and energy fluxes across the interface such as that occurring in evaporation, condensation etc., (as observed in industrial hydrocrackers) are not accounted in the model calculations.

- Transport coefficients for both mass and energy in most models are based on correlations which are appropriate only at low flux, dilute solution, thermodynamically ideal conditions. Such situations are not observed in most organic solvents where fluxes and concentrations are not always low and behavior is far from an ideal solution.

- Most models for trickle beds assume no effect of interphase transport on the hydrodynamics and assume constant phase velocities and holdups it is seen even though it has been seen in some laboratory scale reactions and many industrial cases that evaporation, condensation etc., can transform a completely liquid phase reaction to a completely vapor phase reaction within the bed. Present models in the literature cannot handle these situations.

Even the state of the art commercial multiphase codes do not inherently handle multicomponent mass transfer and have not been used to investigate the coupling between reaction-transport and hydrodynamics in industrial systems (in the open literature). This motivates us to develop codes which can overcome some of the above mentioned efficiencies which can then be tested on laboratory systems and then on some industrial reactors to get realistic model predictions.

## 2.4 Modeling Multicomponent Effects

The assumptions and simplifications of most of the steady and unsteady state models presented are not valid for most real industrial reactors, as discussed in the previous section, especially for cases where the effect of multiple components transferring across the interface can influence the overall reaction-transport interactions occurring in the system. Conventional models used on laboratory scale test reactors cannot explain several phenomena that are observed in industrial reactors. Krishna (1994) presented a clear analysis of three cases where the multicomponent interaction effects observed in simple experiments were incapable of being explained by single component approaches. Phenomena such as reverse diffusion, where the diffusive flux of a component can occur against its own concentration gradient, osmotic diffusion, where diffusion can occur despite a zero concentration gradient, and, mass transfer barrier where a zero flux of a component is observed despite a non-zero gradient have been demonstrated elegantly to bring forth the importance of multicomponent models. This section reviews the basic equations used in multicomponent modeling of the transport within and between phases required for the model developed in Chapter 5. The use of Stefan-Maxwell equations is discussed briefly as a background for their incorporation in the model equations presented in Chapter 5.

For non-ideal gases and liquids, the fundamental transport equation for a species i in a multicomponent mixture can be written in terms of the balance between the net driving force (di) and the drag (written as the product of the momentum exchange per collision (ui-uj) and the collision frequency (proportional xixj)). This can also be expressed in terms of the chemical potential gradient(∇μi) as the driving force for transport given as

di=−xixj(ui−uj)Dij=xiRT∇μi							(2. 3)

The drag term can be rewritten in terms of the absolute fluxes Ni (using Ni=ciui) or in terms of the relative fluxes Ji (using Ji=Ni - xiNt) for a n component system as

di=j=1n(xiNj−xjNi)ctDij=j=1n(xiJj−xjJi)ctDij						(2. 3)

In order to apply this equation and solve it for the transport fluxes, the chemical potential gradient must be expressed in terms of concentration gradient. This is, however, subject to the restriction imposed by the Gibbs-Duhem equation which states that summation of the total driving forces must be zero. To account for this restriction, the right hand side of  Equation 2-2 is reduced to a n-1 term form as shown below:

di=xiRT∇μi=xiRTj=1n−1∂μi∂xj|T,P,Σ∇xj=xiRTj=1n−1RT∂lnγixi∂xj|T,P,Σ∇xj			(2. 3)

di=j=1n−1δij+xi∂lnγi∂xj|T,P,Σ∇xj=j=1n−1Γij∇xj						(2. 3)

Here ij are elements of the thermodynamic non-ideality matrix [] obtained from an activity coefficient model such as the Wilson, UNIFAC etc. models as shown in Appendix E. The RHS of Equation 2-3 is similarly reduced to a (n-1x n-1) matrix form by assimilation of the terms corresponding to the nth component to equate to the results obtained in Equation 2-5 as:

di=−[B](n−1)x(n−1)(J)(n−1)x1ct								(2. 3)

The multicomponent diffusivity matrix [B] in Equation 2-6 is obtained as outlined in Appendix D. The Equations 2.5 and 2.6 can then be combined to form the set of equations used to obtain the species fluxes in a multicomponent mixture. Using the definition of the absolute flux (Ni = Ji + xiNt) by elimination of di from Equations 2-5 and 2-6, we get:

(N)(n−1)x1=(J)(n−1)x1+(x)(n−1)x1Nt =−ct[B](n−1)x(n−1)−1(Γ)(n−1)x(n−1)(∇x)(n−1)x1+(x)(n−1)x1Nt			(2. 3)

The LHS of Equation 2.7 is a vector of (n-1) unknown fluxes obtained by evaluation of the RHS comprising of (n-1 x n-1) dimensional transport coefficient matrix. Note that RHS contains Nt (=Ni) indicating the implicit nature of the multicomponent transport equations. This set of equations also implies that only n-1 independent equations are available for the n unknown fluxes. An additional condition or restriction is required to make the system of equations complete. This extra condition, referred to as the “bootstrap condition” (Taylor and Krishna, 1993) can, in principle, be obtained by solution of the rigorous momentum equations in the region of interest.  However, this is not feasible in most cases, in particular for interphase transport which is typically occurring in the direction transverse to the main flow direction at interfaces which are extremely difficult to model. In practice, the real hydrodynamics is then replaced by some approximate model such as the film model. This has found immense popularity in engineering applications due to its simplicity which enhances both conceptual understanding and calculations. The film model, however, stipulates the presence of a stagnant film on both sides of the interface, implying that Nt is automatically zero. Nevertheless, this approach has been used to model non-equilibrium separation (Nt  0) by incorporating other conditions based on a priori information available for the particular system under consideration. This bootstrap condition is formulated using a generalized flux equation (Taylor and Krishna, 1993) which can be incorporated in an (n-1) dimensional matrix form to be combined into the Equation 2-7 discussed above. In most cases, this bootstrap conditions is some relation between the fluxes (as illustrated in examples below), which can in general be expressed as:

i=1nνiNi=0										(2. 3)

where i are known as determinacy coefficients values of which are based on the physics of the problem and are known a priori. The use of this form of the bootstrap equation allows its easy conversion to a matrix form to be combined with Equation 2-7. This bootstrap matrix represented by  “”, is obtained (by converting the condition given in Equation 2-8 to n-1 x n-1 dimension form) as

βik=δik−xiΛk									(2. 3)

where

Λk=νk−νnνixi										(2. 3)

Equation 2.7 can then be rewritten (in terms of the mass transfer coefficient if film model is accepted) by using the bootstrap matrix evaluated in Equation (2-7) as

(N)(n−1)x1=ct[ko](n−1)x(n−1)[Γ](n−1)x(n−1)(Δx)(n−1)x1+(x)(n−1)x1Nt =ct[β](n−1)x(n−1)[ko](n−1)x(n−1)[Γ](n−1)x(n−1)(Δx)(n−1)x1			(2. 3)

Here the mass transfer coefficient matrix [ko] consists of the high flux mass transfer coefficient determined from the low flux coefficient matrix [k] by applying an appropriate flux correction as shown in Appendix D. It must be noted again that this correction is also based on flux necessitating implicit calculation even though Equation 2-11 appears explicit.

Special Cases for Bootstrap Condition

Several conditions have been suggested in distillation and absorption literature (Taylor and Krishna, 1993) as an alternative to Equation 2-8 and they all rely upon prior knowledge of the physics of the particular problem. Some of the special cases are as follows:

Case I. Equimolar Counterdiffusion: This is typically assumed (and is approximately valid) in simple distillation to simplify calculations and is expressed either as a zero net flux condition

Equation 2.8 can then be written as

a)  Nt = Ni= 0   set i = n								(2. 3)

A more realistic conditions in case of multicomponent distillation based on a simplified energy balance

b)  i=1nNiDHVAPi=0 set i = HVAPi						(2. 3)

Case II. Stefan Diffusion: This condition is commonly assumed in case of dilute solutions for gas absorption in a stream of an inert component or evaporation/ condensation of a vapor into an inert component. This is written by forcing the nth component flux to be zero.

Nn = 0    set i = 0, n  0								(2. 3)

Case III. Graham Diffusion: The condition for isobaric diffusion or Knudsen diffusion through a porous medium can be expressed by Graham’s law.

i=1nNiM12=0  set i = M1/2							(2. 3)

Case IV. Stoichiometric Diffusion: This condition is employed typically for reaction-diffusion problems, where fluxes are governed by stoichiometry of the reaction. It is also used to relate condensation rates in presence of reactions where the ratio of the fluxes are automatically governed by stoichiometry.

Ni = zi Nt   set i = - 1/ zi , j = 1							(2. 3)

Case V. Simultaneous Energy and Mass Energy Balance: This is used when none of the simpler assumptions above can be applied and is particularly valid for non-isothermal conditions. The condition converted to the  matrix is nothing but the energy balance at the interface given as

qL+i=1nNiLHiL(TL)=qV+i=1nNiVHiv(TV)						(2. 3)

This equation cannot be completely converted to the form given by Equation 2.8, since it has a component due to the qL and qV terms which cannot be converted to a matrix form in terms of the fluxes Ni, which have to be added separately to each flux equation to get the correct total flux (Nt). For the terms containing Ni, we can set:

νi=xi(HiV(@TG)−HiL(@TL))							(2. 3)

Case VI. Zero Net Volumetric Flux: This condition is used typically for transport to confined phases such as bubbles, droplets, or pellets and is used to ensure constant volume of the phase into which mass transfer is occurring by forcing to zero the net volumetric flux to that phase, which can be expressed as

i=1nNiMiρi=0   setνi=Miρi 							(2. 3)

One of the above conditions are used depending upon the physics of the problem to supply the extra condition required to solve for interface fluxes, temperatures and compositions, which are then used to calculate source terms for bulk phase transport equations for the species, energy, and momentum equations discussed in the next section.

## 2.5 Balance Relations for Multiphase Systems

The conservation equations for mass, momentum, and energy in multiphase systems are typically written in the form of microscopic point equations (Bird et al., 1960, Ishii, 1975), which are then expressed in a two-fluid model form and averaged suitably (volume, ensemble, time etc.,) to obtain equations that can be used in simulation of mass, momentum, and energy transfer in multiphase reactors. The point conservation equation for all conserved properties is presented and briefly discussed here (refer to Taylor and Krishna (1993) or Ishii (1975) for details). The two-fluid volume averaged form of this equation, with some simplifying assumptions, is used in the model development in Chapter 5. For any conserved property  (expressed per unit mass of mixture) at any time and position, the conservation equation can be written as

∂(ρtΨ)∂t+∇.ρtΨv+∇.Φ=ζ								(2. 3)

where t is the mass density of the fluid, v is the fluid velocity vector,  is a non convective flux through the surface, ζ is the rate of production per unit volume. The jump balance relation at the interface between two phases a and b can be written in general as:

ξ.Φa+ρtaΨa(va−vI)−(Φb+ρtbΨb(vb−vI))=ζI					(2. 3)

where va, vb, and, vI are the velocities of the phases a, b, and the interface, respectively,  is the unit normal directed from a to b phase, ζIis the rate of production of quantity  at the interface. Equation 2-20 can be used to express total mass, species mass, momentum, and energy conservation by appropriately defining the quantities , , and ζ(assumed to be zero in mass and species balance for simplicity, implying no reaction).

- For species balance equation, choose =xi, =ji, and ζ=0

- For total mass conservation, choose =1, =0, ζ= 0, ζI=0

- For momentum conservation, choose =v, =p, and ζ= F

- For Energy balance, choose =U+1/2 v.v, =q + Hiji + p.v, and ζ=0

The Equation (2-21) for mass transfer across the interface can also be written in terms of concentrations and velocities in the bulk and at the interface (assuming no interfacial reaction) as:

ξ.cia(va−vI)=ξ.cib(vb−vI)								(2. 3)

which can be written in terms of interfacial fluxes as discussed in Section 2.4

ξ.Nia=ξ.Nib										(2. 3)

The evaluation of Ni required for substitution in Equation 2.23 and then into Equation 2.20 is done by solution of the flux equation vector for multi-component transport (Equation 2.7) as demonstrated in Section 2.4. These equations form the basis of reaction transport modeling presented in Chapter 5, where all the four conservation equations are used in form suitable for presence of multiple phases and in the volume averaged form.

# Chapter 3. Experimental Facility

## 3.1 High Pressure Trickle Bed Setup

Our first objective was to experimentally investigate the comparative performance (under identical external operating conditions) of trickle bed (TBR) and up-flow (PBC) reactors. This involved collecting data in up-flow and down-flow reactors for a hydrogenation reaction over a range of pressures and liquid mass velocities in order to create the data bank required for comparison of the two reactors. Identical experimental investigation in the presence of fines was also outlined in the objectives. Both these objectives required a versatile reactor setup in which the reaction could be carried out with the same batch of a catalyst in both trickle bed and packed bubble column configuration without removing and reactivating the catalyst bed. This necessitated the modification of several components of the high pressure trickle bed reactor facility previously set up by Al-Dahhan (1993) for high pressure hydrodynamic studies. These are discussed in the following subsection. Several modifications were also done to the system for the unsteady state operation conducted in the second part of the study and are presented subsequently.

### 3.1.1 Reactor and Distributors for Upflow and Downflow

Two reactors were designed for the two parts of this study, each consisting of a 60 cm long stainless steel tube with a 2.54 cm outer diameter and 2 mm wall thickness enough to withstand 500 psig pressure. One of the reactors was designed to run at near isothermal conditions by adding an external cooling jacket (5.08 cm outer diameter), while the other was designed for adiabatic operation and the cooling jacket was substituted by a fiber glass insulation with 3 ports for temperature and pressure drop measurements. Connections between the reactor and distributor sections are made with flanges on both the bottom and the top of the reactor. Liquid and gas are fed to the reactor through ¼ inch stainless steel lines equipped with three way valves to allow for both down-flow and up-flow configurations by merely switching the valve positions (Figure 3.1). For trickle bed operation, the valves V1, V2, V3, and V4 were all switched to position #1, while for packed bubble column (upflow) operation, all of them are switched to position #2 (as shown in Figure 3.1). This allows switching between the two modes of operation without discharging the catalyst and enables performance comparison at identical catalyst activity. Both gas and liquid distribution are critical in downflow operation, while gas distribution is important in upflow operation. The downflow distributor (Figure 3.2) consists of 21 stainless steel tubes (1.6 mm inside diameter) to distribute the liquid phase through a perforated plate welded inside the distributor. The gas phase enters the space surrounding these tubes and leaves through 3.55 mm holes around the liquid tubes as well as through the extra holes (0.76 mm in diameter) provided in between. The gas distributor at the bottom of the reactor is used for upflow operation and is designed in the form of a U-shaped stainless steel box welded to a ¼ inch stainless steel line. On top of the box are 7 holes (0.2 mm in diameter) to distribute the gas uniformly in the reactor (Figure 3.2). The liquid line for this distributor is just a ½ inch stainless steel tube concentric to the gas delivery line and feeds the liquid all around the gas distributor box. The distributor was tested using a clear acrylic reactor to ensure that no channeling of gas occurs during up-flow operation.

### 3.1.2 Gas-Liquid Separator and Level Control

The gas-liquid separator was designed and constructed from a 7 inch diameter optically clear acrylic rod sealed at both ends by stainless steel flanges (Figure 3.3). The top flange consists of a liquid level measurement device at the center with connections for the gas-liquid mixture feed to the separator and gas exit from the separator on the sides, while the bottom flange has a liquid exit line and a sampling line. The funnel shape originally chosen by Al-Dahhan (1993) in order to minimize the liquid volume is also used in this case. The small cylindrical section below it is used as a reservoir for the paddle of a liquid level measurement device (Dynatrol, E-103C), which measures vibrations in the liquid and transmits the liquid level signal to a controller (Omega 7600). The controller signal is then converted to a pneumatic signal using an I/P transducer and then used to operate a pneumatic valve controlling the liquid leaving the separator. Liquid level control in the separator is crucial to maintaining a stable pressure in the reactor as well as to prevent hydrogen escaping to the liquid exit line. A stainless steel mesh demister is also placed in the top flange to trap the mist from the effluent gas stream.

### 3.1.4 Liquid and Gas Delivery System

The liquid reactant and solvent is delivered by a set of two high pressure metering pumps (Calgon HP-15 and Eldex B-100) either to a high pressure feed tank or directly to a small tank used to damp the pulsation due to the pump and then sent on to the reactor. The liquid feed rate is controlled by the pump head setting and also by a recycle line from the small damper tank back to the pump inlet. Direct liquid supply from the pump is used in steady state experiments, but this is not suitable for unsteady state experiments. A pulsation free liquid supply is obtained by using the high pressure feed tank, which is independently pressurized by nitrogen supplied from a high pressure cylinder. The liquid supplied either from the high pressure tank or from the damper is sent through two sets of parallel lines through needle valves to a combination of three way and solenoid valves which are used to switch between steady flow and ON/OFF or BASE/PEAK flow required for experiments operated under unsteady state conditions (Figure 3.3). The solenoid valves are controlled by an electronic timer to which the cycle period and split data are supplied.

The liquid outlet stream from the gas liquid separator is split into a sampling stream and a waste stream. The sampling stream is connected to a needle valve for withdrawal of samples analyzed by the gas chromatograph (Gow Mac Model 550). The waste stream is passed through two liquid back pressure regulators used to drop the liquid pressure from the operating pressure to atmospheric. The liquid is collected in a 110 liter stainless steel waste tank. Liquid flow measurement is also done by a rotameter placed before the waste tank or manually by collecting the exit liquid over several time intervals. Good level control is important in obtaining reproducible flow rate data.

The gas delivery system consists of high pressure hydrogen gas cylinders connected to the gas delivery line via a pressure regulator. Before the gas is delivered to the reactor, it is bubbled through two small tanks (similar to the damper tanks) full of solvent in order to prevent excessive evaporation of solvent in the catalyst bed. The gas is then supplied to the distributor via a needle valve used to control its feed rate. The gas outlet stream from the gas-liquid separator passes through a series of back pressure regulators used to drop the pressure from the reactor pressure to a pressure of 8 psig at which it flows through the rotameter which is pre-calibrated to determine the flow rate at that pressure. This is then corrected to calculate the actual superficial velocity at the operating pressure. The exit gas from this arrangement is then vented to the atmosphere via a copper vent tube connected to the hood above the laboratory. Gas pressure indicators and safety valves are mounted at several locations to prevent pressure buildup in the gas delivery and exit stream.

Figure 3. 1 Reactor and Gas-Liquid Separator

Figure 3. 2 Down-flow and Up-flow Distributor for TBR and PBC Operation

Figure 3. 3 Experimental Setup for Unsteady State Flow Modulation Experiments

### 3.1.5 Data Acquisition and Analysis

Pressure drop measurements were conducted using a high pressure differential pressure transducer (Viatran Model 374, 0-20 “ WCD), the output of which is connected to the terminal block of a multiplexed data acquisition unit (National Instruments SCXI-1100) which in turn is connected to a data acquisition board (National Instruments-AT MIO-16) of a personal computer (Dell 486) as shown in Figure 3.4. Transducer outputs from the thermocouples (type-K) are also connected similarly to the SCXI-1100 unit. The software Labview (National Instruments V4.0) is utilized to develop the required virtual instruments (VI’s) for both temperature and pressure drop measurements.

Figure 3. 4 Data Acquisition System

The reactant concentration data obtained from the gas chromatograph (Gow Mac Model 550) is also sent to the same computer using a separate data acquisition board and Peaksimple-II software which can detect the peaks of the species and evaluate area percent values which are then converted to concentrations using a predetermined calibration curve.

## 3.2 Operating Procedures and Conditions

### 3.2.1 Steady State TBR-PBC Comparison Experiments

Prior to experimentation in the trickle bed reactor, stirred basket reaction tests were performed to establish a reproducible catalyst activation procedure and to test the stability of the catalyst before undertaking the packed bed reactor studies. Preheating in an inert (Nitrogen) atmosphere to 140oC followed by activation with Hydrogen at reaction temperature (~24oC) yielded consistent activity for over 40 hours of operation (after an initial 9 hours of stabilization runs).

Figure 3. 5 Basket Reactor  Catalyst Stability Test for Palladium on Alumina Catalyst

Catalyst (2.5% Pd on alumina extrudate (properties as listed in Table 3.1)) supplied by Engelhard was packed to a height of 27.5 cm with glass beads (3 mm diameter) packed on both sides to a total reactor length of 59 cm. The bed was activated by first preheating it in a chromatographic-type oven at 140 oC for 4-5 hours with a low flow rate of nitrogen passed through the bed to remove any moisture present in the catalyst. The bed was cooled and activated by reducing it with a low flow of hydrogen at near room temperature for 6-8 hours. The bed was then stored in solvent (hexane) under a nitrogen atmosphere to maintain the activity and keep the catalyst wetted at all times. a-methylstyrene (Eastman Chemicals) was pre-purified by passing over a bed of alumina to remove the polymerization inhibitor. This was then diluted in hexane (ACS grade, 99.9% purity) and was used as the liquid phase delivered to the reactor by a precalibrated high pressure pump or the pressurized feed tank as mentioned in Section 3.1.4. Pure hydrogen (prepurified, analytical grade) bubbled through a hexane saturator (to prevent evaporation of hexane in the reactor) was used as the gas phase. The reactor was kept isothermal at room temperature by using a cooling jacket (24oC with maximum 2oC temperature rise) easily due to dilute liquid reactant (maximum concentration < 8 % v/v). At these conditions very little loss of liquid can occur due to volatilization. Typically a high liquid flow rate was chosen for the first experiment and then the flow was reduced for subsequent experiments and so on. This was done to eliminate the possibility of hysteresis seen when flow rates are increased from lower values to higher ones. Liquid samples were drawn from the gas-liquid separator after steady state was reached at each liquid flow rate (about 20-25 minutes, after which no further change in exit concentration of a-methylstyrene was observed). The samples were analyzed on a gas chromatograph (Gow Mac Series 550, with a thermal conductivity detector) from which the conversion of -methylstyrene was determined. Pressure drop across the bed was measured using a differential pressure transducer as shown in Figure 3.1. Experiments were conducted at a fixed gas flow rate by varying liquid flow rate for each  mode of operation. The stability of the catalyst and the reproducibility of the data over the duration of the experiments was verified by taking intermittent samples at identical operating conditions. The reproducibility of the data was observed to be within 3%.  The range of operating conditions investigated in this study are presented in Table 3.2. These operating conditions were selected on the basis of three considerations: i) liquid mass velocities were chosen so as to cover a range of wetting conditions from partially wetted to completely wetted catalyst, ii) liquid feed concentrations and pressures were picked to encompass both gas and liquid limited conditions (g values range from 0.7 to 10), iii) liquid reactant concentrations were kept low by dilution with solvent (n-hexane) to keep the system nearly isothermal.

### 3.2.2 Bed Dilution and Experiments with Fines

Upon completion of the entire set of experiments without fines, the catalyst was discharged and repacked with inert fines (Silicon Carbide) using a reproducible packing procedure developed at CREL by Al-Dahhan et al. (1995). Several methods (such as the dry and wet method) have been suggested in literature with different proportions of fines to catalyst and have been shown by Al-Dahhan et al. (1995) to give different and somewhat non-reproducible holdup and pressure drop characteristics. The procedure developed at CREL and followed in the present experiments was to use the dry method of packing, whereby the volume of the fines added was calculated on the basis of the bed void volume in the undiluted condition. The bed was packed in two portions (halves) in the present study. The catalyst was packed to half the bed height first followed by addition of the predetermined volume of fines. These were packed in the interstices of the catalyst by using a vibrating device on the bed until the top layer of catalyst was visible. This was then followed by the second half of the catalyst, which was similarly packed, and reactor vibrated until the top layer of catalyst was visible. The increase in the overall volume of the diluted bed over the undiluted bed was kept to within 2 % to ensure that the structure of the original bed is not significantly disturbed by the addition of fines. Experimental runs similar to those done previously were conducted now with the diluted bed to study the effect of fines on the performance of down-flow and up-flow modes of operation.

Table 3. 1 Catalyst and Reactor Properties for Steady State Experiments

| Catalyst Properties |  |  | Reactor Properties |  |
| --- | --- | --- | --- | --- |
| Active metal | 2.5% Pd. |  | Total length | 59 cm |
| Catalyst support | Alumina |  | Catalyst length | 27.5 cm |
| Packing shape | Cylinder |  | Diameter | 2.2 cm |
| Packing dimensions (cm) | 0.13 x 0.56 |  |  |  |
|  |  |  | Fines |  |
| Surface area (m2 / g) | 184 |  | Material | Silicon Carbide |
| Total pore volume (cm3 / g) | 0.481 |  | Diameter | 0.02 cm |
| Pellet density P (g / cm3) | 1.222 |  |  |  |
| True pellet density t (g / cm3) | 2.965 |  |  |  |
| Pellet porosity P | 0.5878 |  |  |  |

Table 3. 2 Range of Operating Conditions for Steady State Experiments

| Temperature | 24oC |
| --- | --- |
| Pressure | 30 - 200 psig (3 -15 atm.) |
| Concentration of -methylstyrene | 3.1 - 7.8 %(v/v) (230-600 mol/m3) |
| Superficial gas velocity (mass velocity) | 3.8 - 14.4 cm/s (3.3x10-3-12.8x10-3 kg/m2 s) |
| Superficial liquid velocity (mass velocity) | 0.09-0.5 cm/s (0.63 - 3.85 kg/m2 s) |

### 3.2.3 Unsteady State Experiments

The reaction chosen for this study was hydrogenation of alpha-methylstyrene to iso-propyl benzene (cumene) which is the same system that was used for experiments in section 3.2.1. The high pressure packed bed reactor facility described in section 3.1 was used in this study (with the non-jacketed reactor). The liquid delivery system was modified for unsteady state experiments by adding a set of solenoid valves and a timer (as shown in Figure 3.5) to obtain liquid ON-OFF flow, liquid BASE-PEAK flow, and steady liquid flow as desired. The catalyst used for these experiments, 0.5% Pd on alumina spheres (different from that used in the earlier steady state experiments) was packed to a height of 26 cm (with glass beads on both sides to a total height of 59 cm) and was activated by reducing in situ (since this reactor did not have an external jacket, it was easy to pre-heat, cool, and activate in situ). The reaction was run in this activated bed for several hours at steady state until a constant catalyst activity was obtained. Since the activity varied slightly between runs, steady state experiments were performed before and after each set of unsteady state runs to ensure reproducibility of the catalyst activity within each set. -methylstyrene (99.9% purity and prepurified over alumina to remove the polymerization inhibitor) in hexane (ACS grade, 99.9% purity) was used as the liquid phase. Pure hydrogen (pre-purified, industrial grade) was used as the gas phase. The reactor was operated under adiabatic conditions. Liquid samples were drawn from the gas-liquid separator after steady state was reached at each liquid flow rate. The samples were analyzed by gas chromatography (Gow Mac Series 550, with thermal conductivity detector) from which the steady state conversion of -methylstyrene was determined. Unsteady state conversion was determined by evaluating concentration of a liquid sample collected over multiple cycles to get the flow average concentration. The reproducibility of the data was observed to be within 3%. The ranges of operating conditions investigated are presented in Table 3.4. The feed concentrations and operating pressures were chosen so as to examine both gas limited and liquid limited conditions (as evaluated approximately by the  criterion as discussed in section 2.1.2). The liquid mass velocities were chosen so as to cover partial to complete external wetting of the catalyst. Both liquid ON-OFF and BASE-PEAK flow modulation were studied over a range of liquid mass velocities (Table 3.4) for each set of experiments as illustrated in Figure 3.6 below.

Figure 3.6 Schematic of Flow Modulation: Connections and Cycling Strategy

Table 3. 3 Catalyst and Reactor Properties for Unsteady State Conditions

| Catalyst Properties | Reactor Properties |
| --- | --- |
| Active metal 0.5 % Pd | Total Length 59 cm |
| Catalyst support Alumina | Catalyst Length 26 cm |
| Packing shape Sphere | Diameter 2.2 cm |
| Packing dimensions 3.1 mm |  |

Table 3. 4 Reaction and Operating Conditions for Unsteady State Experiments

| Superficial liquid mass velocity | 0.05-2.5 kg/m2s |
| --- | --- |
| Superficial gas mass velocity | 3.3x10-3-15x10-3 kg/m2s |
| Operating pressure | 30 -200 psig (3-15 atm) |
| Feed concentration | 2.5 - 30 % (200-2400 mol/m3) |
| Feed temperature | 20-25 oC |
| Cycle time,  (Total Period) | 5-500 s |
| Cycle split,  (ON Flow Fraction) | 0.1-0.6 |
| Max. allowed temperature rise | 25 oC |

# Chapter 4. Experimental Results

## 4.1 Steady State Experiments on Trickle Bed Reactor and Packed Bubble Column

Comparison of the two reactors was achieved by studying the conversion at identical  nominal space times (defined as reactor length/ superficial liquid velocity) and identical reactant feed concentration. This is the proper scale-up variable, (space time = 3600/LHSV) when the beds for upflow and downflow are identically packed (i.e., bed voidage = constant) and the reaction rate is based per unit volume of the catalyst.

### 4.1.1 Effect of Reactant Limitation on Comparative Performance of TBR and PBC

At low pressure (30 psig) and high feed concentration of -methylstyrene (CBi= 7.8 %v/v), the reaction is gas limited (g = 8.8). In this case downflow performed better than upflow reactor as shown in Figure 4.1. This is due to the nature of the hydrogenation reactions which are typically  hydrogen (gas reactant) limited at low pressure (at or just above atmospheric) and high -methylstyrene concentrations (Beaudry et al., 1987). It is obvious that this is due to low hydrogen solubility at these pressures reducing external transport rates of hydrogen. In downflow mode of operation, the catalyst particles are not fully wetted at the liquid flow rates used (Figure 4.11 shows contacting efficiency calculated using the correlation of Al-Dahhan and Dudukovic (1995)). This facilitates the access of the gas reactant to the pores of the catalyst on the externally dry parts, and reduces the extent of gas limitation compared to fully wetted pellets in upflow reactor. The result is a higher conversion in downflow than in the upflow mode of operation. In case of upflow, since the catalyst is almost completely wetted, the access of gaseous reactant to the catalyst site is limited to that through liquid film only. This provides an additional resistance for the gaseous reactant (especially at high space time i.e., low liquid flow rate) and resulted in conversion lower than that obtained in downflow. This effect is more prominent at higher liquid reactant feed concentrations, due to the larger extent of gas limitation at such conditions (higher g values). As liquid mass velocity increases (space time decreases), the downflow performance approaches that of upflow, due to catalyst wetting efficiency approaching that of upflow (contacting efficiency approaches 1 as seen in Figure 4.11).

As the reactor pressure increases and the feed concentration of -methylstyrene decreases, the value of g decreases and the reaction approaches liquid limited behavior as postulated earlier. This is reflected in a complete reversal in performance at higher pressures and at low -methylstyrene concentration (Figure 4.2), where the performance of upflow becomes better than downflow. This is because  under these conditions the catalyst in downflow is still partially wetted (since at the operating gas velocities and gas densities (hydrogen), high pressure only slightly improves wetting in downflow (Figure 4.11 based on Al-Dahhan and Dudukovic, 1995) while catalyst is fully wetted in upflow. In a liquid limited reaction, the conversion is governed by the degree of catalyst wetting, and since upflow has higher wetting (100 %) than downflow, it outperforms downflow (Figure 4.2). As the liquid mass velocity increases, and the contacting efficiency of downflow approaches 100 %, the performance of the two reactors approaches each other, as evident in Figure 4.2 at low space times. Thus, as pressure is increased from 30 to 200 psig, and feed concentration of a-methylstyrene is decreased from 7.8% to 3.1%(v/v), the reaction is transformed from a gas-limited (g = 8.8) to a liquid-limited regime (g = 0.8). The criterion (g) is dependent on two factors (apart from the diffusivity ratio), pressure (hydrogen solubility) and feed concentration of the liquid reactant (-methylstyrene) (as discussed in section 2.1.2). Further insight into the gas and liquid limitation can be obtained by investigating these two contributions individually for the set of operating conditions examined.

Figure 4. 1 Trickle Bed and Up-flow Performance at CBi=7.8%(v/v) and Ug =4.4 cm/s at 30 psig.

Figure 4. 2 Comparison of Down-flow and Up-flow Performance at CBi=3.1%(v/v) at 200 psig.

### 4.1.2 Effect of Reactor Pressure on Individual Mode of Operation

As reactor pressure increases, the performance of both upflow and downflow improves due to increase in gas solubility, which both helps the rate of transport to the wetted catalyst (in both modes) and improves the driving force for gas to catalyst mass transfer to the inactively wetted catalyst in the downflow mode. At low feed concentration of the liquid reactant (-methylstyrene (3.1%v/v)) and at high pressure (>100 psig), the reaction  becomes liquid reactant limited (or liquid reactant affected) as can be seen from Figures 4.3 and 4.4 where no further enhancement is observed when pressure is increased from 100 to 200 psig (where g drops from 1.5 at 100 psig to 0.8 at 200 psig). This means that any further increase in the reactor pressure and hence liquid phase hydrogen concentration, will have minimal effect since hydrogen is not the limiting reactant anymore.

To confirm the above observation the reaction was studied at higher feed concentration of -methylstyrene (4.8 %v/v) in order to determine whether gas limited behavior is observed at higher g values. The performance indeed improves when pressure is increased from 100 to 200 psig (Figures 4.5) implying that the reaction is not completely liquid limited at this feed concentration at 100 psig operating pressure (g=2.44). It  becomes liquid limited at pressures above 200 psig (g =1.3) at this feed concentration, whereas it is liquid limited at lower -methylstyrene concentration (3.1%v/v) even at lower pressures as noted previously in Figures 4.3 and 4.4. Both upflow and downflow conversion increases with increasing pressure, primarily due to increase in solubility of the gaseous reactant as the pressure increases. A significant improvement in performance (conversion) occurs when pressure is changed from 30 to 100 psig as compared to the change in conversion when pressure changes from 100 to 200 psig. This confirms that the effect of pressure diminishes when liquid limitation is approached (as g approaches 1.0 from above (Figure 4.5)).

### 4.1.3 Effect of Feed Concentration of -methylstyrene on Individual Mode of Operation

Atmospheric pressure hydrogenation of  a-methylstyrene has been known to be a zero order reaction with respect to a-methylstyrene and first order with respect to hydrogen (El-Hisnawi et al., 1982; Beaudry et. al, 1986). Our observations confirm this observation at 30 psig as well as at 100 psig, the reaction is zero order with respect to -methylstyrene as shown in Figures 4.6 and 4.7 for upflow and downflow, respectively. An inverse proportionality of conversion with liquid reactant feed concentration (typical of zero order behavior) is observed especially at higher liquid flow rates (lower space times). At lower liquid flow rates, at 100 psig the zero order dependence appears to vanish and a first order dependence (due to -methylstyrene transport or intrinsic rate limitations), i.e., conversion independent of feed concentration, is observed. Beaudry et al. (1987) have also observed positive order dependence at low liquid velocities (much higher space times) due to alpha-methylstyrene affecting the rate. This shift in feed concentration dependence is confirmed by data at higher pressure (200 psig, Figures 4.8 and 4.9). When liquid limitation is observed there is no effect of feed concentration on the conversion in either mode of operation, as can be seen in Figure 4.8 and 4.9. This is a consequence of the intrinsic rate limitation that shows up as a first order dependence making conversion independent of feed concentration (see also Appendix A for high pressure intrinsic rate data).

Figure 4. 3 Effect of Pressure at Low -methylstyrene Feed Concentration on Upflow Reactor Performance.

Figure 4. 4 Effect of Pressure at Low -methylstyrene Feed Concentration (3.1% v/v) on Downflow Performance.

Figure 4. 5 Effect of Pressure at Higher -methylstyrene Feed Concentration on Downflow Performance.

Figure 4. 6 Effect of -methylstyrene Feed Concentration at 100 psig on Upflow Performance.

Figure 4. 7 Effect of -methylstyrene Feed Concentration at 100 psig on  Downflow Performance.

Figure 4. 8 Effect of -methylstyrene Feed Concentration at 200 psig on Downflow Performance.

Figure 4. 9 Effect of -methylstyrene Feed Concentration at 200 psig on Upflow Performance.

### 4.1.5 Effect of Gas Velocity and Liquid-Solid Contacting Efficiency

Effect of gas velocity on reactor Performance :

At low gas and liquid mass velocities, the level of interaction between the gas and liquid phases is expected to be minimal in downflow. In case of upflow, the effect of gas velocity on gas-liquid mass transfer is expected due to changing interfacial area for transport with changing gas velocity. This would however be influential in determining the performance if the gas-liquid mass transfer were limiting the overall reaction. The influence of gas velocity on the performance of both upflow and downflow reactors is shown in Figure 4.10. The effect of gas velocity on reactor performance was also examined for both upflow and downflow reactors. A significant effect was not observed in the range of the gas velocities studied (3.8-14.4 cm/s, i.e., gas Reynolds number in the range of 6-25) on either downflow or upflow performances at all the feed concentrations and pressures tested. This is in agreement with earlier observations of Goto et. al (1993). Experimental pressure drop measurements were also made for both modes of operation during the reaction runs. The data obtained (shown in Figure 4.11) indicates higher pressure drops for upflow at both ends of the pressure range covered (30 and 200 psig) than for downflow, which is in agreement with expectation and the  pressure drop data reported in the literature. The higher downflow performance (conversion) at 30 psig, despite lower pressure drop, confirms that poor contacting (Figure 4.11) does yield better conversion due to reaction being gas limited, which seems contrary to the notion that higher transport always involves higher pressure drop (which is observed to be true here in case of liquid limited reaction at 200 psig).

Figure 4. 10 Effect of  Gas Velocity on Reactor Performance at 100 psig.

Figure 4. 11 Pressure Drop in Downflow and Upflow Reactors and Contacting Efficiency for Downflow Reactor at 30 and 200 psig.

## 4.2 Comparison of Down-flow (TBR) and Up-flow (PBC) Reactors with Fines

Fines (nonporous inert particles, order of magnitude smaller than catalyst pellets packed only in the voids of the catalyst) were used to investigate the performance of the two modes of operation using the same reaction in an attempt to demonstrate the decoupling of hydrodynamic and kinetic effects. A way to establish this decoupling is to use the upflow and downflow modes, which are intrinsically hydrodynamically different (as discussed earlier), and asses whether fines can indeed yield the "true" kinetic behavior (more properly called "apparent" rates on catalyst pellets of interest, i.e., rates unmasked by external transport resistances and hydrodynamic effects). The two extreme cases discussed before, i.e., gas limitation (downflow performance better than upflow, Figure 4.1), and liquid limitation (upflow performance better than downflow, Figure 4.2) are now conducted in the presence of fines. Figures 4.12 and 4.13 show the performance of both reactors when the bed is diluted with fines. It can be seen by comparing Figure 4.12 with Figure 4.1 and Figure 4.13 with Figure 4.2 that fines have eliminated the disparities between the two modes of operation even in the extreme cases of reactant limitation. This is primarily due to the fact that fines improve liquid spreading considerably and achieve comparable (and almost complete) wetting in both modes of operation. It must be noted that Figures 4.1 and 4.12, or Figures 4.2 and 4.13, could not be directly superimposed due to slightly different catalyst activity obtained after repacking the bed with fines and catalyst and reactivating it. Nevertheless, fines have successfully decoupled the hydrodynamics and apparent kinetics, and the data with fines reflect the kinetics in the packed bed under "ideal" liquid distribution conditions. It can be observed in Figure 4.12 that at low liquid flow rate and low pressure (gas limited reaction), a trickle bed with fines still performs slightly better than upflow with fines, which indicates that the degree of wetting is still not complete in downflow resulting in some direct exposure of the internally wetted but externally dry catalyst to the gas . This may be due to the fact that at low liquid flow rate even with fines , the catalyst is not completely externally wetted (Al-Dahhan and Dudukovic, 1995). At high pressure (liquid limited reaction) Figure 4.12 reveals identical performance of both reactors where complete wetting is achieved in both modes.

Since we studied the impact of the two factors, pressure and feed concentration on the performance without fines, the same study was conducted for the bed diluted with fines.

Figure 4. 12 Effect of Fines on Low Pressure Down-flow Versus Up-flow Performance

Figure 4. 13 Effect of Fines on High Pressure Down-flow Versus Up-flow Performance

### 4.2.1 Effect of Pressure in Diluted Bed on Individual Mode of Operation

The effect of pressure on the performance of both modes of operation in beds with fines is illustrated in Figures 4.14 and 4.15. At higher pressure the performance of both upflow and downflow is better than that at low pressure. This observation is also consistent with the data obtained without fines. The pressure dependence is observed as expected by increase in gas solubility based on the hydrogen concentration dependence reported by Beaudry et al. (1987) as well as that observed in slurry experiment at both pressures as discussed in Appendix A and reported by El-Hisnawi et al. (1982) when inverse dependence of conversion with respect to feed concentration. Beaudry et al. (1987) observed some liquid limitation effects at low liquid mass velocity (high space time) these were not seen at the lower space times examined here.

### 4.2.2 Effect of Feed Concentration in Diluted Bed on Individual Mode of Operation

At 30 psig, the conversion is higher at lower feed concentration of  methyl styrene (lower 2 curves for downflow (Figure 4.14) and upflow (Figure 4.15). At higher reactor pressure, there is no effect of feed concentration (upper 2 curves, Figure 4.14  and 4.15). This was also observed for the reactors without fines and explained on the basis of liquid limitation in the previous section. The fact that it is observed with fines confirms the feed concentration dependence (of performance) in case of gas and liquid limited reaction.

Fines have been shown to successfully decoupled the hydrodynamics and reaction effects, and can yield true apparent kinetic data in the packed bed under "ideal" liquid distribution conditions. Both gas and liquid limited conditions were investigated and identical performance was shown under all conditions studied, implying that using fines is the recommended strategy to be used for scale-up or scale-down.

Figure 4. 14 Effect of -methylstyrene Feed Concentration at Different Pressures on Performance of Downflow with Fines.

Figure 4. 15 Effect of -methylstyrene Feed Concentration at Different Pressures on Performance of Upflow with Fines.

## 4.3 Unsteady State Experiments in TBR

The objectives outlined in Chapter 1 for the study of unsteady state operation were to conduct experiments to study the effect of gas and liquid reactant limitation as well as cycling parameters such as total cycle period, cycle split, and cycling frequency (as described in Section 3.2.3).  Comparison of the data obtained (as listed in Appendix F) is done with the few data available in literature on similar systems.

### 4.3.1 Performance Comparison for Liquid Flow Modulation under Gas and Liquid Limited Conditions

Performance comparisons were done at the two cases of interest, (i) gas reactant limitation, and (ii) liquid reactant limitation, obtained by changing operating pressure and feed concentration. Performance under unsteady state operation was obtained by evaluating the flow averaged conversion over several cycles of operation according to the procedure outlines in Section 3.2.3. Based on several trial runs, a total cycle time of 60 s and a cycle split of 0.5 was chosen for this set of experiments with liquid ON/OFF flow (see Figure 4.26 for liquid mass velocities corresponding to the space times investigated).  Under near liquid-limited conditions (i.e., high pressure and low feed concentrations) no enhancement is observed with this modulation strategy, except at very low liquid mass velocities (high mean space times (= VR/QL(mean)). At high liquid mass velocity, the catalyst is well irrigated and any advantage due to better wetting under unsteady state is not feasible. This can be qualitatively seen by examining the convex shape of the contacting efficiency curve (Figure 4.26) which would yield better catalyst wetting under steady state conditions and hence better performance under unsteady state conditions. This analysis cannot however be applied at low liquid mass velocity where the unsteady state wetting and replenishment of stagnant liquid pockets with fresh liquid reactants can   make enhancement possible. Under these conditions, the bed is poorly irrigated and the disadvantage due to liquid maldistribution (not seen in steady state contacting efficiency plot as shown in Figure 4.26) can be overcome by the high flow rate liquid (Figure 4.16,  < 4).

At low space times (high mass velocity, Figure 4.16), performance enhancement is not seen under laboratory conditions due to several factors such as small reactor diameter, good distributor, resulting in good catalyst wetting (as seen in Figure 4.26). The conditions investigated in the present experiments correspond to fairly high liquid hourly space velocities (LHSV) in comparison with industrial trickle beds, where this maldistribution effect may be seen to be more pronounced. LHSV varied from 3 to 15 in our experiments as compared to 1.5 to 10 used in industrial reactors. In case of gas limited conditions (i.e., at low operating pressures and high feed concentration), it can be seen that unsteady state performance (conversion) is significantly higher than that under steady state conditions at all space times (Figure 4.17,  ~ 25).  This case illustrates the conditions of a liquid reactant full catalyst and enhanced supply of the gaseous reactant leading to better performance. This enhancement improves as the extent of partial wetting is increased as seen at higher space times (lower liquid mass velocities). This also corroborates the findings of Lange et al. (1994) and Castellari and Haure (1995) under severe gas limitation (due to 50 % (v/v) and 100 % pure liquid reactants in their studies respectively) enhancement is feasible. Castellari and Haure (1995) explored this enhancement further by increasing total cycle time to allow complete internal drying and corresponding large temperature increase and semirunaway conditions. This was not feasible in the present study and enhancement due to higher gas supply was expected to increase by increasing the gaseous reactant supply and lower liquid ON times. A small exothermic contribution was also observed during unsteady state operation here with maximum bed temperatures reaching ~ 6 oC higher than steady state temperatures.

Figure 4. 16 Comparison of Steady and Unsteady State Performance under Liquid Limited Conditions ( < 4)

Figure 4. 17 Comparison of Steady and Unsteady State Performance under Gas Limited Conditions ( ~25)

### 4.3.2 Effect of Modulation Parameters (Cycle Period and Cycle Split) on Unsteady State TBR Performance

To explore whether further performance enhancement is achievable for the gas reactant limited case by increasing gaseous reactant supply to the catalyst, a constant liquid mean flow was chosen (Space time = 660 s, L= 0.24 kg/m2s) and cycle split () was varied (to vary the ratio of the gas to liquid access times). It can be seen that further enhancement was indeed possible as cycle split was lowered from steady state ( = 1) to a split of  = 0.25, the performance improved by as much as 60% over steady state at the same mean liquid mass velocity (Figure 4.18).  This improvement continues up to the point where liquid limitation sets in at very low cycle split (indicating that the liquid is completely consumed in a time duration less than the OFF time of the cycle), beyond which the performance will be controlled by liquid reactant supply. This implies that performance improvement can be maximized by choosing an appropriate cycle split (at a given liquid mass velocity and total cycle period).

The effect of the total cycle period was investigated at the cycle split () value of 0.33, where performance enhancement was observed to be significant (Figure 4.18). The performance enhancement is seen to increase with total cycle period up to a maximum, after which it drops to near (or below) steady state values. Similar maximum was observed by both Lange et al. (1994) and Haure et al. (1990) for different reaction systems. A qualitative explanation for this phenomena can be discerned on the basis of their observations and the present data. At low cycle periods the liquid reactants are supplied at shorter intervals than can be consumed by the gaseous reactants leading to aggravation of the gas limitation (or gas reactant starvation). At large cycle periods (for the same cycle split), opposite behavior is seen due to longer gas access time than is necessary to consume liquid reactants (liquid reactant starvation). Gabarain et al. (1997) have examined the effect of very large cycle times (up to 40 minutes) and seen that the temperature rise initially observed due to large reaction rates drops off as liquid starvation is seen at the end of the long cycle. The have attempted to find an optimum total cycle period based on this maximum temperature rise (maximum rate due to gas phase reaction on completely externally and then internally dry catalyst). Due to the competition between gas reactant starvation at the lowest cycle periods and liquid reactant starvation at the higher ones, a feasibility envelope can be determined (Figure 4.19) based upon which the performance enhancement can be optimized. The location of maximum enhancement is seen to be dependent upon the liquid reactant concentration as seen in the experiments of Lange et al. (1994) (cycle period ~ 7.5 minutes for ~ 50 % v/v of alpha-methylstyrene) under similar operating conditions. This maximum could be quantified by a complex function of an effective  parameter (under dynamic conditions) and the effect of cycle split and liquid mass velocity if transient variation of concentration is known accurately. The above effects of cycle split and total cycle period were examined at a constant liquid mass velocity. Due to the strong dependence of catalyst wetting and reaction rate (both steady and unsteady), the dependence of liquid mass velocity (which can be referred to as amplitude of the flow modulation) is important from the point of view of commercial scale application. This is examined in the section.

Figure 4. 18 Effect of  Cycle Split () on Unsteady State Performance under Gas Limited Conditions

Figure 4. 19 Effect of Total Cycle Period () on Unsteady State Performance under Gas Limited Conditions

### 4.3.3 Effect of Amplitude (Liquid Mass Velocity) on Unsteady State TBR Performance

The feasibility envelope (region where performance enhancement is possible,  Figure 4.19) as discussed in Section 4.3.2 and observed in literature is strongly dependent on the relative supply of gaseous and liquid reactants. Gaseous reactant access is governed by the extent of wetting (depending on liquid flow) and the formation of dry areas (depending on evaporation). Castellari and Haure (1995) examined catalyst drying in their experiments which is attributed to complete external and internal evaporation of liquid (until liquid reactant is completely consumed). However, all of their experiments were conducted at the same mean liquid mass velocity (~ 10 times higher than in the present study) at which the steady state wetting effect is complete. The change in catalyst wetting due to evaporation in the present study is not large (as compared to Castellari and Haure (1995)) as compared to that due to change in flow with time, it is important to explore whether the feasibility region can be altered with changing liquid mass velocity. Two mean liquid mass velocities where wetting would have the most effect (lowest mass velocity possible, see Figure 4.26) were examined (at a cycle split of 0.25). The results presented in Figure 4.20 compare normalized enhancement (conversion at unsteady state over that at steady state). A significantly higher enhancement is observed by lowering the mean liquid mass velocity. At a mass velocity of 0.137 kg/m2s, a similar feasibility envelope is seen as in Figure 4.19 which ends in degradation of performance to below steady state values at higher cycle periods due to depletion of the liquid reactants (liquid starvation as discussed in the previous section). The lower liquid mass velocity allows more time for liquid reactant supply (higher mean space times) to the catalyst. This is reflected in the shift in the liquid starvation to even higher total cycle periods (Figure 4.20), while the gas starvation side remains unchanged (similar to that observed in Figure 4.19).  The higher maximum enhancement at lower mass velocity (Figure 4.20) clearly indicates the trend towards the limit of maximum possible enhancement (the ideal case of zero flow of liquid reactant and complete conversion).

Figure 4. 20 Effect of Liquid Mass Velocity on Unsteady State Performance under Gas Limited Conditions

### 4.3.4 Effect of Liquid Reactant Concentration and Pressure on Performance

Two key parameters, which decide the extent of gas or liquid reactant limitation are liquid reactant feed concentration and operating pressure. The combined effect of these was discussed in Section 4.3.1 for two sets of steady and unsteady state performance data corresponding to the gas limited and liquid limited extremes. It was shown that with the ON/OFF liquid flow modulation strategy, performance enhancement was possible under gas limited conditions (Figure 4.17). The effect of individual contributions of pressure and liquid reactant feed concentration under gas limited conditions needs to be carefully examined to determine exact cause-effect relationships in performance enhancement. The effect of liquid reactant feed concentration was examined under gas limited conditions by evaluating enhancement at different cycle splits. With increase in gas limitation due to higher liquid reactant feed concentration, we would expect higher enhancement due to flow modulation at higher liquid reactant feed concentration. However, this is not observed as expected (Figure 4.21). Since the absolute value of the conversion at higher feed concentrations is lower (due to gas reactant limitation), the enhancement seen is not as high even if lower mean liquid mass velocity was used. Mass velocity used at the higher liquid reactant feed concentration was 0.1 kg/m2s as compared to 0.24 kg/m2s at the lower feed concentration. The effect of operating pressure was examined at constant gas velocity of 5.4 cm/s and liquid mass velocity of 0.085 kg/m2s. Under gas limited conditions, both steady and unsteady performance improves with increase in pressure as expected (due to enhanced solubility at elevated pressures). This enhancement diminishes as liquid limited conditions are approached at even further higher pressures, especially at liquid mass velocities where the bed irrigation is complete (under laboratory conditions). In the present experiment the liquid mass velocity used is fairly low, hence the performance enhancement is seen even at the highest pressure studied (Figure 4.22).

Figure 4. 21 Effect of Liquid Reactant Feed Concentration on Unsteady State Performance under Gas Limited Conditions

Figure 4. 22 Effect of Operating Pressure on Unsteady State Performance under Gas Limited Conditions

### 4.3.5 Effect of Cycling Frequency on Unsteady State Performance

Liquid ON/OFF flow modulation can be considered as square wave cycling about the mean flow for the case with a cycle split of 0.5. Conversion under periodic conditions can then be used to examine the dominant time scales affected by induced flow modulation (IFM) by looking at the frequency () dependence of flow averaged conversion. Both Figures 4.23 and 4.24 show performance enhancement as a function of the IFM frequency with similar trends seen at different pressures, feed concentrations, and even for a case of non-square wave pulsing ( = 0.2). The performance in both cases shows degeneration of the enhancement at low frequencies tending to the steady state operation at zero frequency. But as frequency is increased, the conversion reaches a clear maximum improvement point. Ritter and Douglas (1970) observed similar frequency dependence (in dynamic experiments on stirred tanks) and attributed the maximum to the resonance frequency of the rate controlling process. All transport processes typically have a natural frequency corresponding to their characteristic time scale. Gas-liquid transport in trickle beds corresponds to 0.2 to 0.8 Hz (at low pressures), liquid-solid transport corresponds to 0.5 to 2 Hz, whereas catalyst level reaction-diffusion processes correspond typically to much lower frequency (larger time scale) depending upon intrinsic rates in pellets. Typical industrial reactions in trickle bed reactors have been reported to correspond to a frequency range of 0.01 to 0.1 Hz (Wu et al., 1995). Natural pulsing occurs in trickle beds at high liquid flows and displays frequencies of 1 to 10 Hz, at which external transport is significantly improved (Blok and Drinkenberg, 1982; Wu et al., 1995). The present IFM frequencies are much lower than those observed under natural pulsing. The frequency dependence of the performance (Figure 4.23 and 4.24) shows that the highest effect of IFM can be observed at low frequencies ( ~0.1 Hz) at which catalyst level processes could be predominantly affected to obtain the observed enhancement. Some effect on external transport processes can also be seen (in Figures 4.23 and 4.24) with some enhancement observed corresponding to their natural frequencies. This opens up the possibility of selectivity enhancement and control for complex reaction schemes by controlling reactant supply by the proper choice of the IFM frequency for the desired reactant (Wu et al., 1995). The low enhancement seen at both ends ( 0 and  ) can be explained on the basis of the frequency analysis similar to that done by Lee and Bailey (1974). The very low IFM frequency ( 0) corresponds to an equilibrium state or pseudo steady state, where the reaction transport processes have time to catch up with the modulated variable (flow in this case) and the overall system behaves as a combination of discrete steady states. On the other hand at high IFM frequency ( ), the input fluctuations are so rapid that none of the reaction-transport processes in the system can respond to the IFM and any gain in performance due to the modulated variable is again not feasible.

Figure 4. 23 Effect of Cycling Frequency on Unsteady State Performance under Gas Limited Conditions

Figure 4. 24 Effect of Cycling Frequency on Unsteady State Performance

### 4.3.6 Effect of Base-Peak Flow Modulation on Performance

For the case of liquid limited conditions (i.e., at high pressure and low feed concentrations) the use of complete absence of liquid during the OFF part of the cycle is not recommended based on the discussion in Section 4.3.1 and observed in experiments of Lange et al. (1994). A low base flow similar to the mean operating flow has been recommended instead (Gupta, 1985; Lange et al., 1994) with a periodic high flow slug introduced to improve liquid distribution and open up multiple flow pathways for the liquid to flow during the rest of the cycle.  The cycle split () here is the fraction of the cycle period for which the high flow rate slug is on (typically chosen to be very short). Tests were conducted at a cycle split of 0.1 and cycle times varying from 30 to 200 s at an operating pressure of 150 psig and low liquid reactant feed concentration (C(AMS) feed = 784 mol/m3) to ensure liquid limited conditions. This strategy is shown to yield some improvement over steady state performance (Figure 4.25,  < 1.2), although this is not as high as observed under gas limitation (maximum enhancement observed here was 12 % as against 60 % in case of gas limited conditions). This is primarily due to intrinsically better flow distribution in small laboratory reactors, which would not be the case in typical pilot or industrial reactors where much higher enhancement can be anticipated.

L (peak)

L (mean) =  L (peak)+(1-) L (base)

, (sec)



(1-)

L (base)

L (mean)



Figure 4. 25 Unsteady State Performance with BASE-PEAK Flow Modulation under Liquid Limited Conditions

Figure 4. 26 Effect of Liquid Mass Velocity on Steady State Liquid-Solid Contacting Efficiency

# Chapter 5. Modeling Of Trickle Bed Reactors

## 5.1 Evaluation of Steady State Models for TBR and PBC

The qualitative analysis and arguments made in the discussion of the experimental comparison in section 4.1 on the basis of reactant limitation, liquid-solid contacting, and effect of pressure on kinetics were verified by comparison with model predictions of some of the existing models as discussed in this section. A history of the model development of trickle bed reactors was presented in Chapter 2 and salient features of each presented in Table 2.3. Based on the discussion therein, two models (developed at CREL), one with reactor scale equations (El-Hisnawi, 1982) and other with pellet scale equations (Beaudry, 1987) were chosen to compare predictions to experimental data. The key parameters of distinction between downflow and upflow are the hydrodynamics and catalyst wetting efficiency, and hence involved the solution of partially wetted pellet performance for downflow and fully wetted pellets performance for upflow. The effect of liquid mass velocity, and, gas-liquid and liquid-solid transport in both reactors was required for the model. These were evaluated on correlations proposed in literature for the reactor under consideration. The intrinsic kinetics required for this was obtained by slurry experimentation as discussed in section 4.1 and Appendix A.

### 5.1.1 Reactor Scale Model (El-Hisnawi et al., 1982)

The El-Hisnawi et al. (1982) model was originally developed for a low pressure trickle bed reactor to account for rate enhancement due to externally inactively wetted areas. The model was proposed in the form of heterogeneous plug flow equations for the limiting reactant and solution of surface rate of the limiting reactant and substitution of this expression into the plug flow equation of the non-limiting reactant (Table 5.1). For example, when A (gaseous reactant, hydrogen) is the limiting reactant, its surface concentration is solved for, and, rate evaluated on its basis is substituted in the plug flow equation for concentration of B (liquid reactant, alpha-methylstyrene) to obtain conversion of B at each velocity specified. Analytical solutions derived for first order kinetics for the equations at low pressures. At high pressure, however, the reaction was observed to be B limited with non-linear kinetics (given in the Langmuir–Hinshelwood form in Appendix A) and surface concentration of B was solved from the non-linear equation (in Table 5.1) to get the rate equation and then solved numerically to get the concentration profile of B and reactor exit conversion at each space time. The pellet effectiveness factor can be determined from the thiele modulus but was used here as a fitting parameter at one superficial velocity and used for all other cases. This was done due to the uncertainty in the catalyst activity (rate constant) and the effective diffusivity evaluation at different pressures. The liquid-solid contacting efficiency was determined at low pressure by correlation’s developed by El-Hisnawi (1981) and at high pressure using the correlation of Al-Dahhan (1993). The upflow reactor was assumed to have completely wetted catalyst in all cases. Mass transfer coefficients were determined for downflow by correlations for gas-liquid mass transfer coefficient from Fukushima and Kusaka (1977), for liquid-solid mass transfer coefficient from Tan and Smith (1980), and gas-solid mass transfer coefficient from Dwiwedi and Upadhyay (1977). For upflow prediction, the gas-liquid mass transfer coefficients were obtained from the correlation by Reiss (1967), and, liquid-solid mass transfer coefficients from correlation by Spechhia (1978). The variation of the mass transfer coefficients calculated from the above correlations with space time is shown in Figure 5.5.  The predictions of the El-Hisnawi model at low pressure (gas limited) compare well with the downflow experimental data as shown in Figure 5.1. For the case of upflow performance however, the model over predicts the experimental data at low space time. This implies that the effect of external mass transfer is felt in case of upflow, which typically predicts higher mass transfer coefficient at higher liquid velocity (lower space time) resulting in the higher predicted conversion at low space times. At high pressure, liquid limited conditions, however, El-Hisnawi model predictions compare well with the experimental data for upflow and downflow as shown in Figure 5.2. The effect of mass transfer is not felt as much at high pressure and predictions are qualitatively and quantitatively able to capture the observed experimental behavior of both reactors.

### 5.1.2 Pellet Scale Model (Beaudry et al., 1987).

The Beaudry model considered the evaluation of catalyst pellet effectiveness subject to different wetting conditions and substitution of the overall effectiveness into a simple plug flow equation to evaluate reactor conversion. The catalyst pellets were modeled in the form of infinite slabs with the two sides exposed on both sides, either to gas or liquid, or a half-wetted pellet exposed to gas and liquid on either side. At low pressure (gas limited conditions), the gaseous reactant supplied from both sides of the pellet depletes to almost zero within a short distance depending upon the extent of the reactant limitation. Hence, the solution of pellet effectiveness for downflow involved solution of both the dry and wetted side for a half wetted pellet, and solution of the completely wetted pellet for the gas limited case (as shown Table 5.2). For the completely dry pellet the effectiveness for this case was zero since no liquid reactant could be supplied to this pellet.  The analytical solutions to this case for the first order in hydrogen are available in Beaudry et al. (1987) and were used to obtain the overall effectiveness factor as a weighted average of the contacting and the effectiveness of each type of pellet as shown in Table 5.2). At high pressure (liquid limited conditions) the solution is much more complicated due to the non-linear reaction rate which demands the solution of the reaction diffusion equations for the externally wetted pellets on both sides and the half wetted pellet only on the wetted side. Here, the value of  is the point where the liquid reactant depletes completely and is the boundary for the liquid reactant concentration solution. This solution needs to be evaluated at each point in the reactor to get a local effectiveness factor corresponding to the local concentration of the liquid reactant. Instead of doing this as a coupled system of equation both on the pellet and reactor scale, the pellet scale equations were solved at different bulk liquid reactant concentrations and then fitted as a polynomial of pellet effectiveness as a function of surface concentration. This polynomial is then used to solve the reactor scale equations numerically to obtain conversion at each space time. Although this approach does not require any fitted parameters as needed in the El-Hisnawi model, the rate constant was similarly fitted to match the conversion at one space-time and used to compare with the experimental data at all other space times. As can be seen from Figure 5.1 and 5.2, this model predicts the observed data for down flow at low pressure and at high pressure well, but not so well for up-flow especially at low pressure and high feed concentration. The reason may be due to mass transfer correlations used which may predict a lower performance (than observed experimentally) at high space times in the upflow operating mode.

The Beaudry et al. (1987) model predictions are also shown on the Figure 5.1 and 5.2 for low and high pressure respectively. As can be seen, the Beaudry (1987) model predicts downflow performance exactly as El-Hisnawi's model does, but under-predicts upflow performance at higher space times (low liquid velocities) due to some effect of estimation of mass transfer can be seen at high space times due to the correlations used. For high pressure, on the other hand, Beaudry's models predicts experimental data quite well both for downflow and upflow, since the effect of mass transfer is not seen as much as that at low pressure.

The effect of the feed concentration on predictions of both the models was also examined for both downflow and upflow reactors as shown in Figures 5.3 and 5.4 respectively. As mentioned earlier in the discussion, the predictions are almost identical for both models for downflow and agree with experimental data. The predictions for the upflow experimental data for both models differs slightly due to the effect of mass transfer coefficient for both the models, as seen in Figure 5.4. In both cases, the inverse relationship of liquid feed concentration with conversion typical of low pressure gas limited operation seen in the experiments is predicted correctly.

The predictions of the reactor scale and pellet scale models are satisfactory for current conditions although there is a need for high pressure correlation for mass transfer coefficient and interfacial area in order to predict performance with greater certainty, especially in cases where the rate is affected significantly by external mass transfer. The predicted performance of upflow and downflow for both the models presented and discussed in this section is seen to be strongly dependent on the reaction system i. e., whether the reaction is gas or liquid limited under the conditions of investigation. The laboratory reactors are operated in the range of partially to fully wetted catalyst to demonstrate that the influence of wetting can be detrimental or beneficial, depending upon the reactant limitation. Models that account for these two effects (such as the ones chosen here) can predict the performance over the entire range of operating conditions. The intrinsic kinetics of the reaction studied at different pressures is also was also seen to be important in obtaining good predictions. Hence in any given reaction it is recommended to study the slurry kinetics at the specific operating pressure before any scale up or modeling is attempted. A rate expression with different rate constants at each of the discrete pressures (as used here) can be used to predict the trickle bed reactor data at the same pressures most accurately, rather than using a general rate form which cannot fit all the data obtained at different pressures. It must be mentioned that the reactor scale model failed to predict experimental data well at the intermediate conditions (100 psig,  ~ 1) when the reaction is neither completely gas limited nor liquid limited (or switches from gas limited to liquid limited at some location in the reactor) because the model assumptions were for the extreme conditions of one reactant being limiting. Rigorous solution of the reactor and pellet scale equations presented in the next section would be able to cover any general case without the assumptions made here.

Table 5. 1 Governing Equations for El-Hisnawi (1982) Model

Original model equations (gas limited conditions)

And

Boundary conditions:

Equilibrium feed Non-Equilibrium feed and CB(z)¦z=0=CBi

Model equations at high pressure (liquid limited conditions) (boundary conditions are the same as at low pressure)

−uSLdCA,Ldz+(ka)gL[CA,e−CA,L]−ηkvs(1−εB)ηCECA,LCB,LS(1+K1CB,LS+K2CC,LS)β=0

kLSaLS(CB,L−CB,LS)=ηkvs(1−εB)ηCECA,LSCB,LS(1+K1CB,LS+K2CC,LS)β (for liquid reactant limitation)

Table 5. 2 Governing Equations for Beaudry (1987) Model

Pellet Scale Equations:

a) Low Pressure (Gas Reactant Limited) with rate first order in A.

Boundary conditions (Pellet):

(=0 for m<1) (=0 for m<1)

Boundary conditions (Reactor):

Where,

Table 5.2 (contd..)

- High Pressure (Liquid Reactant (diffusional) Limitation for Langmuir-Hinshelwood rate form)

Boundary conditions:

For both case a) and b) above, the reactant conversion in general is given by

where (at low pressure) (at high pressure)

and overall effectiveness factor is given by

Figure 5. 1 Upflow and Downflow Performance at Low Pressure (gas limited condition): Experimental data and model predictions

Figure 5. 2 Upflow and Downflow Performance at High Pressure (liquid limited condition): Experimental data and model predictions

Figure 5. 3 Effect of Feed Concentration on Downflow Performance

Figure 5. 4 Effect of Feed Concentration on Predicted Upflow Performance

Figure 5. 5 Estimates of volumetric mass transfer coefficients in the range of operation from published correlations (G-L (downflow) Fukushima and Kusaka (1977), L-S (downflow) Tan and Smith (1980), G-L (upflow) Reiss (1967), L-S (upflow) Spechhia (1978)).

## 5.2 Unsteady State Model for Performance of Trickle Bed Reactors in Periodic Operation

This section discusses the development of a generalized model, which can account for phenomena occurring in trickle bed reactors under unsteady state operating conditions as presented in experimental results and discussion in Section 4.3. The models available in literature on unsteady state operation are discussed in Section 2.3.2 and form the basis of the model equations presented in this section. The following steps can help understand the phenomena occurring during unsteady state flow modulation presented in section 4.2. A typical cycle period can be considered in terms of the steps outlined below.

- Supply of liquid reactants by a slug of liquid to the catalyst with almost complete wetting when the slug passes (Figure 5.6(a)). Mass transfer rates of reactants to and products from the catalyst are enhanced in this step.

- This is followed by a liquid OFF (or a low liquid flow as described in Section 4.3.6) period (Figure 5.6 (b) and (c)) in which supply of gaseous reactants to the catalyst is enhanced leading to high reaction rate and consumption of the liquid reactant..

- The reaction may occur isothermally or non-isothermally depending upon concentration of the liquid reactant and heat removal rate. Temperature rises and, hence, higher rates are achieved during the OFF part of the cycle (Figure 5.6 (c)). The effect of increased temperature affects the evaporation of liquid reactant. In case of a gas reactant limited reactions, no liquid flow is allowed in the OFF part of the cycle(Figure 5.6 (c)). In case of liquid reactant limited reaction a low flow of liquid is used in the OFF (referred to as BASE flow in section 4.3.6) part following the slug of  higher liquid flow rate (Figure 5.6 (b)) This is followed by re-introduction of another slug of liquid which quenches the heat and replenishes the liquid reactant (Figure 5.6 (a)).

These steps also confirm the explanation of Gabarain et al. (1997) and their simplified model for periodic flow modulation in terms of three different zones, (i) completely externally wetted pellets, (ii) partially externally wetted pellets and completely externally and (iii)internally dry pellets.

Figure 5. 6 Phenomena Occurring in Trickle Bed under Periodic Operation

Scope of the Model:

The phenomena occurring in periodic operation as described above can be summarized and converted to the required equations as follows

- Dynamic variation in the gas and liquid phase species concentrations is obtained by solving species mass balance equations for the gas and liquid phase.

- Gas-liquid, liquid-solid, and gas-solid mass transfer is modeled by the Maxwell-Stefan equations for multicomponent transfer.

- Variation of holdup with time and axial position, during and after the pulse of liquid/gas, is obtained by solving the continuity equations for the liquid and the gas phase.

- Variation of interstitial velocity with time and axial position is accounted for by solving the liquid and gas phase momentum balance equations.

- Possible evaporation, condensation, and heat of reaction are incorporated by solution of separate energy balance equations for gas, liquid, and solid phase. The interface energy balance is coupled with multicomponent mass transfer to solve for interphase fluxes and temperatures.

- Allowance for the reaction to continue due to static holdup during the OFF pulse part, and for the exchange of species during both parts of the cycle, is made by inserting catalyst level accumulation terms (and Maxwell-Stefan diffusion flux terms) for reactant and product species.

- Allowance for variation of parameters such as contacting efficiency and mass transfer coefficients with axial position and time is made by dynamic evaluation of the parameters at each time instant at all the axial locations.

The model equations proposed here are based on the assumptions mentioned below. A list of the number of variables and corresponding equations necessary to solve the model is given in Table 5.8. It must be emphasized here that the use of Maxwell-Stefan formulation is made in order to account for concentration effects, thermodynamic non-idealities and bulk transport of heat and mass across the interfaces as discussed in Section 2.4.

Assumptions :

- Variation of temperature, concentration, velocity and holdup in radial direction is negligible as compared to axial direction.

- All the parameter values are equal to the cross sectionally averaged values and vary only with axial location and time. These are determined at every time instant for the different axial location and used to solve for pertinent variables at the next time instant.

- The heat of reaction is released only to the solid and then transferred to other phases through interphase heat transfer.

- No temperature gradients exist inside the catalyst pellet.

- The catalyst pellets are modeled as slabs of three types: fully externally wetted, half wetted and fully dry (different approaches to solution of catalyst level equations are discussed in a separate section).

### 5.2.1 Reactor Scale Transport Model and Simulation

As discussed in the background Section (2.5), the point equations for mass, energy and momentum can be written by converting to two-fluid volume-averaged one dimensional form (based on the assumptions above) as follows

Bulk phase equations for species and energy for gas, liquid and solid

Equations for the bulk phase species concentrations can be written in terms of the convection, accumulation and interphase fluxes as given below. The expressions and models for the fluxes are discussed in detail in the next section and play a key role as far as solution of these equations is concerned.

Liquid Phase Species Balance Equations

The generalized 1-D equation for flowing liquid phase species concentrations can be written as a balance of convection, interphase (gas-liquid, liquid-solid) mass transfer and net accumulation terms (nc-1 equations) (where nc is the number of  species).

¶¶t(eLCiL)+¶¶zuILeLCiL=NiGLaGL−NiLSaLS					(5. 6)

Gas Phase Species Balance Equations

These are written as a balance between convection and interphase transport to get accumulation term for any given species. Thus, nc-1 equations can be written (for a total of nc species) as:

¶¶t(eGCiG)+¶¶zuIGeGCiG=-NiGLaGL−NiGSaGS					(5. 6)

The catalyst level species balance equations are discussed in a separate subsection due to several strategies used for their formulation and solution.

Energy Balance Equations

A three temperature model with catalyst, liquid and gas having different temperatures is considered to be consistent with solution of interphase energy balance equations

Gas Phase Energy Balance

This incorporates convection, liquid to gas phase heat transfer (conductive and bulk transport such as evaporation and condensation), heat loss to ambient and accumulation written in terms of gas enthalpy. All other contributions (such as due to viscous dissipation, pressure effect and mechanical work etc.) are not considered in the equations.   The terms for heat loss to ambient has been put in for sake of generality, but solution will primarily focus on adiabatic conditions where this are omitted.

¶(ρGeGHG)¶t+¶(eGuIGHG)¶z=-EGLaGL−EGSaGS−EGAaGA				(5. 6)

where the overall enthalpy HG can be written in terms of molar enthalpy of each species HiG as

HG=CiGHiGρG 									(5. 6)

Liquid Phase Energy Balance

Similarly, writing balances for liquid phase in terms of convection, liquid to gas heat transfer, solid to liquid heat transfer, heat loss to ambient and accumulation, we have

¶(ρLeLHL)¶t+¶(eLuILρLHL)¶z=EGLaGL−ELSaSL−ELAaLA				(5. 6)

where the overall enthalpy HL can be written in terms of molar enthalpy of each species HiL as

HL=CiLHiLρL									(5. 6)

Catalyst (Solid Phase) Energy Balance

Catalyst is assumed to be fully internally wetted and to have no internal temperature gradients. Axial temperature variation is obtained by

∂(1−εB)HCP∂t=+ELSaLS+EGSaGS							(5. 6)

where the overall enthalpy HCP can be written as a combination of the solid and occluded liquid phase enthalpy (in terms of molar enthalpy of each species HiCP ) as

HCP=(1−εP)CPC(TCP−Tref)+εPCiCPHiCP					(5. 6)

The interphase mass and heat transfer fluxes are evaluated as discussed later in this chapter. The species enthalpy shown in equations 5.4, 5.6, and 5.8 are obtained as a combination of heat of formation and a heat capacity x temperature term which gives the phase temperature. Several approaches that have been suggested in literature for solution of these types of equations were considered as alternatives. A moving boundary approach suggested by Finlayson (1990) was not found to be suitable due to different phases present and moving at different velocity (i.e., a stationary solid phase, a pulsing liquid phase and a constant velocity gas phase). The other popular approach is the use of orthogonal collocation on finite elements (OCFEM) (Villadsen and Michelsen, 1978; Laura Gardini et al., 1985) which is known to work well for non-steep concentration and temperature profiles, which was not the case in the present problem. The large set of non-linear algebraic equations to be solved for the interphase fluxes, temperatures and compositions (as discussed subsequently) prevented usage of any Ordinary Differential Equation solvers (LSODE and DDASSL with method of lines, PDECOL etc.). Matrix solution of the interphase transport could not be handled by other packages such as the Integral, Partial Differential and Algebraic Equation (IPDAE) solvers such as gPROMS (Oh and Panetelides, 1995). A finite difference approach with semi-implicit solution of the differential equations (using explicit source terms) followed by separate solution of algebraic equations was also attempted with limited success. Finally, the equations 5.1, 5.2, 5.3, 5.5, and 5.7 are solved at each time and axial location by marching in space using a semi-implicit predictor step (for all concentrations and temperatures) which is used as the guess value for the fully implicit corrector step which solves the interphase equations (for source terms to the above mentioned set of equations) and simultaneously corrects the above variables. The source terms involve evaluation of interphase mass and energy fluxes, compositions and temperatures are evaluated by solution of the non-linear equations at the interface (discussed in subsequent sections) by using a globally convergent multivariable Newton solver with a line search algorithm incorporated into it (Press et al., 1992).

All the concentrations, temperatures and other scalar variables are defined at the cell centers of a staggered grid employed here for convenience in solution of pressure and continuity in the flow solver discussed in the next section. Although it is convenient in many cases to non-dimensionalize the equations using characteristic values of each variable to obtain the familiar dimensionless groups, it was found to be inappropriate in this case due to two reasons 1) large variations in the variables during each periodic cycle yielding no single characteristic value for non-dimensionalization, and 2) The transport coefficients differ for each pair of components and are written in matrix form, which would result in a matrix of dimensionless Sherwood or Stanton numbers resulting again in no single dimensionless group to analyze effects.  However, non-dimensionalizing the variables was deemed necessary for the successful usage of the numerical routines used. This was done by considering the guess vector at each point to be the characteristic set of values of the variables and was used to non-dimensionalize the variables before the Newton solver was called. This dynamic non-dimensionalization improved the effectiveness of the non-linear solver considerably.

### 5.2.2 Flow Model Equations

Continuity Equation for the flowing liquid and gas

The liquid present in the reactor is divided into flowing liquid and stagnant (intra-catalyst) liquid for the sake of convenience in modeling. The continuity equation for the flowing liquid and gas can be given in terms of the accumulation and convection terms balanced by the total mass transferred to and from the other phase (written in terms of interphase fluxes for gas-liquid, liquid-solid and gas-solid equations, each discussed in the subsequent section).

∂ρLεL∂t+∂ρLuILεL∂z=+NiGLaGLMi−NiLSaLSMi				(5. 6)

∂ρGεG∂t+∂ρGuIGεG∂z=−NiGLaGLMi−NiGSaGSMi				(5. 6)

The momentum equations for liquid and gas can be written in terms of accumulation, convection terms on the left hand side and the gravity, pressure, drag due to the packed phase, gas-liquid interphase momentum exchange (with exchange coefficient K) and momentum gain due to mass exchange (assuming it is added at the interfacial velocities for liquid and gas phase, uIIL and uIIG respectively) terms on the right hand side as

∂∂t(ρLuILεL)+∂∂z(ρLεLuILuIL)=εLρLg−εL∂P∂z|L−FD,Liq+K(uIG−uIL)+uILINiGLaGLMi−NiLSaLSMi)		(5. 6)

∂∂t(ρGuIGεG)+∂∂z(ρGεGuIGuIG)=εGρGg−εG∂P∂z|G−FD,Gas+K(uIL−uIG)+uIGI−NiGLaGLMi−NiGSaGSMi		(5. 6)

Expanding and simplifying using continuity equations with the assumption of identical velocity of bulk and interface in each of the phases (i.e., uIG= uIIG, uIL= uIIL)

ρLeL¶uIL¶t+ρLeLuIL¶uIL¶z=eLρLg−eL¶P¶z|L−FD,Liq+K(uIG−uIL)			(5. 6)

ρGeG¶uIG¶t+ρGeGuIG¶uIG¶z=eGρGg−eG¶P¶z|G−FD,Gas+K(uIL−uIG)		(5. 6)

Here K is the momentum exchange coefficient between the gas and liquid (suitably defined or dropped for low interaction between gas and liquid) and FD is the drag term is obtained from Holub's three zone approach (Holub, 1990) (with zone selection based on the local value of liquid holdup obtained from the continuity equation and Reynolds numbers calculated at each axial location denoted by subscripts R, F, and D for rivulet, film and dry zones respectively). The expression for the film zone is the most general form and can be used as an alternative to switching between the zones if numerical problems are encountered. The drag between liquid-solid and gas-solid phases is given for the film zone as

FD,Gas=eGρGgee−eL3E1ReG,FGaG+E2ReG,F2GaG					(5. 6)

FD,Liq=eLρLgeeL3E1ReL,FGaL+E2ReL,F2GaL						(5. 6)

Solution of Continuity and Momentum Balance Equations

The method of solution of the continuity and momentum equations is the modified MAC method (Harlow and Welch, 1965) which is a semi-implicit method, i.e., it is implicit in pressure and exchange terms and explicit in all other terms such as gravity, drag and convection. A staggered grid is used for scalars i.e., pressure and phase holdups for which values are evaluated at the midpoint of the computational cell, whereas vectors such as velocity, drag, etc. are evaluated at the cell faces themselves following the approach used in typical Computational Fluid Dynamic (CFD) computations (Patankar, 1980; Versteeg and Malalsekhara, 1996). A simplification was made here for executing this code is that both the phases are incompressible (which is discussed in the subsequent discussion section).

The algorithm thus consists of three main steps:

- Evaluation of the explicit terms to obtain an intermediate velocity u*IL and u*IG (which does not satisfy continuity).

- Substituting these to evaluate pressure at the next time instant by solution of Poisson equation for pressure.

- Evaluating the velocities and holdups at the new time instant based on the newly evaluated pressure (which ensures that continuity is satisfied).

The calculations at each of the above steps are as described below (here n represents the current time values and n+1 represents the next time instant values). To begin the calculations at each time, interstitial velocities are needed for the reactor inlet. These are calculated by estimating liquid holdup using any of the steady state correlations available in literature. The superficial velocities are then corrected to obtain inlet interstitial velocities. The computations at each step can then be summarized as follows:

Step 1. Evaluation of intermediate velocities from explicit equations. The momentum equation is evaluated explicitly (at nth time instant) to obtain an estimate of the velocities at the next time (n+1). Since these velocities will not satisfy continuity, they are considered only as intermediate value for further calculations. The phase holdups used here are also the nth level values to be updated later.

εLρLuiL=εLρLuiLn+Δt(−εLρLuiLn∂uiLn∂z+εLρLg−εL∂pn∂z−FD,Liqn+K(uiGn−uiLn))(5. 6)

eGρGuiG=eGρGuiGn+Dt(−eGρGuiGn¶uiGn¶z+eGρGg−eG¶pn¶z−FD,Gasn−K(uiGn−uiGn))(5. 6)

Step 2. Similarly we can write implicit equations for the velocities at the next time instant with all the terms evaluated implicitly. Subtracting above equations from the implicit equations, we can get a relationship between the intermediate values and the actual velocity values (at the n+1th time step).

εLρLuILn+1=εLρLuIL+Δt(−εL∂(pn+1−pn)∂z+K((uIGn+1−uIG)−(uILn+1−uIL)))		(5. 6)

εGρGuIGn+1=εGρGuIG+Δt(−εG∂(pn+1−pn)∂z−K((uIGn+1−uIG)−(uILn+1−uIL)))		(5. 6)

In order to calculate the pressure at the next time step, we need to eliminate the   velocities of gas and liquid at the next time instant (n+1). This is done by substituting the above equations into the continuity equations (and eliminating the time derivatives by addition) to obtain the Poisson equation for pressure.

This is done differently in two cases as follows:

a) Constant Density (Volume Fraction Addition) Method: This is done in two stages, first by simplifying the RHS of above equations to eliminate non-primary intermediate variables (u*IG in liquid equation and u*IL in gas equation), to get

uILn+1=uIL−Δt((1−α)ρL+αρG)∂(δp)∂z							(5. 6)

uIGn+1=uIG−Δt((1−β)ρG+βρL)∂(δp)∂z							(5. 6)

where

dp=pn+1−pn									(5. 6)

and α=KρGeGDt(ρGeGρLeL+(ρLeL+ρGeG)KDt)    				(5. 6)

β=KρLeLDt(ρLeLρGeG+(ρLeL+ρGeG)KDt)					(5. 6)

Substituting these in the continuity equation and eliminating n+1 level velocities of gas and liquid, we have

NiGLaGLMi(1ρG−1ρL)+NiGSaGSMiρG+NiLSaLSMiρL+∂(εLuIL)∂z+∂(εLuIL)∂z=Δt∂∂zω∂(δp)∂zwhere w=eL1−αρL−αρG+eG1−βρG−βρL					(5. 6)

This is the familiar Poisson equation for pressure, written here for the pressure difference in time (dp=pn+1−pn). This has to be solved for the entire z domain and can be arranged in the form of a tri-diagonal matrix of equations (when written for all z). The boundary conditions for pressure used are that the pressure gradients in time (p) just outside the inlet and the exit are zero, which is true based on the physics of the problem. It must be noted here that the pressure values are evaluated at the center of the grid over which velocity is calculated. The tri-diagonal system is then solved efficiently by the Thomas algorithm (Press et al., 1992).

b) Variable Density Correction (Rigorous Algorithm): Here Equations 5.22 and 5.23 are substituted in their respective continuity equations by pre-multiplying them with the corresponding density and phase holdups. Equation 5.27 is written on elimination of the time derivatives of holdup (which get eliminated on summation of the continuity equations) as

NiGLaGLMi(1ρG−1ρL)+NiGSaGSMiρG+NiLSaLSMiρL+1ρL∂(ρLεLuIL)∂z+1ρG∂(ρGεGuIG)∂z+εL∂lnρL∂t+εG∂lnρG∂t=Δt1ρL∂∂zωL∂(δp)∂z+1ρG∂∂zωG∂(δp)∂z

(5. 6)

where ωL=ρLεL1−αρL−αρG and ωG=ρGεG1−βρG−βρL. 			 (5. 6)

Here the derivatives of density may be neglected for simplicity.

Step 3. The pressure obtained from the solution of the above set of equations is used to update the velocities (using the above equations for the n+1th time instant). These velocities are in turn used to update holdup at the next instant using the continuity equations.

ρLεLn+1=ρLεLn+Δt∂ρLuILεL∂z+NiGLaGLMi−NiLSaLSMi			(5. 6)

ρGeGn+1=ρGeGn+DtρG¶uIGeG¶z−NiGLaGLMi−NiGSaGSMi			(5. 6)

This procedure for pressure calculation and velocity correction is continued until velocity and pressure converges. Adaptive time stepping is used in the flow solution due to the explicit part of the procedure. Solution for holdup and velocity over the entire reactor length calculated using a full and two half time steps are compared to check for convergence as shown in the flow sheet in Appendix B.

### 5.2.3 Multicomponent Transport at the Interface

Interphase Mass and Energy Transport using Maxwell-Stefan equations

The interphase fluxes for the three transport processes must be rigorously modeled for periodic operation due to the fact that assumptions for conventional isothermal, steady state dilute solution transport of single species do not apply in this case. Since flow rates are changing with time and axial position by a large magnitude, any assumption of simple (or equilibrium) mass or heat transfer controlling steps will not hold for the entire simulation. Hence the use of Stefan-Maxwell equations to model interphase fluxes is made in the simulations. The important factor in modeling Stefan-Maxwell processes is that an extra equation connecting the relative flux equations to the total (or some reference) flux is necessary. This reference relation (referred to as “bootstrap”) is discussed for each process after the individual solution sections.

Solution of Stefan-Maxwell Equations at the Gas-Liquid Interface:

The number of unknown variables in these relations is nc fluxes, nc interphase mole fractions for liquid and vapor interface each and interface temperature (total = 3nc+1). The number of equations available for solution are the liquid and gas phase absolute flux equations (nc-1 each), the equilibrium relations at the interface (nc), the mole fraction  summations (to unity) of the gas and liquid interface compositions (2) and the energy balance relation to obtain gas-liquid interface temperature. Several alternatives are available for use as bootstrap as discussed in Section 2.4. However, at unsteady state, the only condition applicable is the energy balance relation itself. Using this condition the gas-liquid flux can be written for each component as follows:

NiGL−ctl[βL][kLo][Γ]i,j(xjI−xjL)−xi(Δq)λx 					(5. 6)

where kLo is the liquid side (high flux) mass transfer coefficient matrix, βL is the bootstrap matrix ( x denotes the mean heat of vaporization of the liquid). The non-matrizable contribution due to q is due to the net conductive heat flux at the interface is evaluated as Dq=hL(TGLI−TL)−hG(TG−TGLI).  Similar equations can be written for the gas side using kGo, the gas side mass transfer coefficient matrix as well as the gas side bootstrap matrix based on y (the mean heat of vaporization based on gas side mole fractions) (Table 5.3). The actual calculation of the [G] matrix is given in Appendix D. It has been assumed here that the interface reaches equilibrium instantaneously, and that there is no accumulation at the interface from which one can write NiG=NiL=NiGL (which is used in further reference to the gas-liquid flux).

The equilibrium at the gas-liquid interface also provides with relations for interface concentrations given for each species as

(5. 6)

The equilibrium constants are determined as given in Appendix D. Other conditions are the summation of mole fractions on the gas and liquid side at the gas-liquid interface. These are coupled simultaneous non-linear equations, expressed as a combined vector of the functions to be solved as in Table 5.3. The solution of this matrix of non-linear equations is accomplished by a modified multi-variable Newton’s method (as mentioned earlier) for which the Jacobian evaluation is done either analytically (ignoring composition dependence of [kG], [kL] and, []) or numerically.

Table 5. 3 Typical Equation Vector for the Stefan-Maxwell Solution of Gas-Liquid Interface

FGL(z,t)=N1GL−ctlj[βL][kLo][Γ]1,j(xjI−xjL)−x1(Δq)λxN2GL−ctlj[βL][kLo][Γ]2,j(xjI−xjL)−x2(Δq)λx...Nnc−1GL−ctlj[βL][kLo][Γ]nc−1,j(xjI−xjL)−xnc−1(Δq)λxN1GL−ctgj[βG][kGo]1,j(yjV−yjI)−y1(Δq)λyN2GL−ctgj[βG][kGo]2,j(yjV−yjI)−y2(Δq)λy...Nnc−1GL−ctgj[βG][kGo]nc−1,j(yjV−yjI)−ync−1(Δq)λyy1I−K1x1Iy2I−K2x21I.yncI−KncxncIx1I+x2I+x3I+...xncI−1y1I+y2I+y3I+...yncI−1qV−qL+i=1nNiGL(HiV−HiL)3nc+1,1

### 5.2.4 Catalyst Level Rigorous and Apparent Rate Solution

Combined Solution of Stefan-Maxwell Equations for Liquid-solid and Gas-Solid Interface

The total number of unknowns at the liquid-solid interface of the catalyst is 2nc+1 (nc liquid solid fluxes, nc liquid-solid interface compositions and 1 liquid-solid interface temperature). The liquid solid transport equations can be formulated in the Stefan-Maxwell scheme by writing flux equations of the general form

NLS=ct[βLS][kLS.]Dx								(5. 6)

The number of independent equations that can be written for the liquid solid system are only 2nc  (Table 5.4, of which nc-1 are the mass transfer flux equations on the liquid side, nc-1 are the catalyst wetted side fluxes from the reaction diffusion equations, one equations from the interface mole fraction summation and one from the energy flux equation). The additional equation required for complete definition of the problem is used to ensure  constant liquid volume in the pellet by ensuring zero net volumetric flux condition (as discussed in Section 2.4). The bootstrap condition on the liquid side is still the energy balance equation, while on the solid side the above mentioned zero volumetric flux condition is implemented (as presented in Appendix D).

Similar equations are written for the (externally) dry side of the catalyst. The number of unknowns in this case are 3nc+1(nc gas-solid fluxes, nc gas side and solid side interface compositions each, and one gas-solid interface temperature). The number of independent equations available are also 3nc+1 (nc-1 flux equations on the gas side, nc-1 catalyst side flux equations from reaction diffusion equations, nc equilibrium relations between the gas and solid (wetted) interface compositions, 1 equation each for the interface mole fraction summation on the gas and solid side, and 1 equation for the energy flux).  The two sets of equations are coupled through the catalyst level reaction-diffusion equations (they incorporate the boundary conditions on the pellet scale equations).  The bootstrap condition used here is the same as that used on the liquid side (in order to make the entire set of equations consistent). This condition is simply expressed as

(NiMiρLi)L−S=(NiMiρLi)G−S						(5. 6)

This completes the set of equations that are necessary to solve for all the transport processes on the liquid-solid and gas-solid interfaces (5nc+2 total equations). This is in addition to the catalyst level equations (which are discussed in the next section). The system of equations for the liquid-solid and gas-solid boundary of a typical half-wetted pellet is presented in Table 5.4.

Table 5. 4 Typical Stefan Maxwell Equation Vector for a Half Wetted Pellet

FGLS(z,t)=N1LS−ctlj[βL][kLo][Γ]1,j(xjL−xjI)−x1(Δq)λxN2LS−ctlj[βL][kLo][Γ]2,j(xjL−xjI)−x2(Δq)λx...Nnc−1LS−ctlj[βL][kLo][Γ]nc−1,j(xjL−xjI)−xnc−1(Δq)λxN1CP−ctCPj[β][B]−1[Γ]1,j(∇x|LSI)N2CP−ctCPj[β][B]−1[Γ]2,j(∇x|LSI)...Nnc−1CP−ctCPj[β][[B]−1[Γ]nc−1,j(∇x|LSI)N1GS−ctGj[βG][kGo]1,j(yjG−yjI)−y1(Δq)λyN2GS−ctGj[βG][kGo]2,j(yjG−yjI)−y2(Δq)λyNnc−1GS−ctGj[βG][kGo]nc−1,j(yjG−yjI)−ync−1(Δq)λyN1CP−ctCPj[β][B]−1[Γ]1,j(∇x|GSI)N2CP−ctCPj[β][B]−1[Γ]2,j(∇x|GSI)Nn−1CP−ctCPj[β][[B]−1[Γ]n−1,j(∇x|GSI)(y1I−K1x1I)GS(y2I−K2x21I)GS.(yncI−KncxncI)GS(x1I+x2I+x3I+...xncI)GS−1(x1I+x2I+x3I+...xncI)LS−1(y1I+y2I+y3I+...yncI)GS−1qG−qS+i=1ncNi(HiG−HiS)qL−qS+i=1ncNi(HiL−HiS)(NiMiρLi)LSI=(NiMiρLi)GSI5nc+2,1

Inspection of the origin of some of the variables listed in Table 5.4 such as intra-catalyst concentrations shows that the catalyst flux equations need to be solved simultaneously with the above (since neither the flux nor the interface concentrations at the next time step are known until the Stefan Maxwell solution is converged).

Energy Flux Relations for Interphase Transport by Stefan Maxwell Approach

For gas-liquid, liquid-solid and gas-solid transport, the interphase energy flux equations can be written based on the Maxwell-Stefan formulation (as described in section 2.4) as a combination of the conductive (without mass exchange) and bulk transport (such as with evaporation, condensation etc.,) flux for bulk to interface transport for each phase and then for continuity of the flux at the interface as shown in Table 5.5 for energy transport flux from bulk to the interface.

Table 5. 5 Energy Flux Equations for Gas-Liquid, Liquid-Solid, and Gas-Solid Interfaces

___________________________________________________________________

Gas-Liquid Interface

EG=hG.(TG−TGLI)+i=1nNiGLHiG(TG)

EL=hL.(TGLI−TL)+i=1nNiGLHiL(TL)

Interface Equilibrium 		EL=EG=EGL

Liquid-Solid Interface

ES=hS.(TLSI−TS)+i=1nNiLSHiS(TS)

EL=hL.(TL−TLSI)+i=1nNiLSHiL(TL)

Interface Equilibrium  	EL=ES=ELS

Gas-Solid Interface

EG=hG.(TG−TGSI)+i=1nNiGSHiG(TG)

ES=hS.(TGSI−TS)+i=1nNiGSHiS(TS)

Interface Equilibrium  	EG=ES=EGS

___________________________________________________________________

Solid Phase Species Balance Equations

Catalyst level equations need to be solved at each axial location separately at each time and are typically of the reaction-diffusion type. The catalyst pellets are considered to be in the form of infinite slabs with the two sides exposed to the flowing phases similar to those assumed by Beaudry et al. (1987). Three approaches are considered as alternatives to solve the catalyst level equations and are discussed below (with pros and cons of each approach). The general equation in all the approaches is as given below

∂CiCP∂t+∂NiCP∂x'=νiR									(5. 6) where x’ is the intra-catalyst spatial dimension

#### i) Rigorous Single Half Wetted Pellet Approach

This formulation considers a single half-wetted pellet exposed to flowing liquid on one side and flowing gas on the other (Figure 5.7(a)). For the intra-catalyst calculations, the flux can be expressed in terms of the Stefan Maxwell diffusion matrix as a product of the inverse of the diffusion matrix and the non-ideality matrix ([B]-1[] as discussed in Section 2.4). But this is a function of the concentration and location thus complicating the solution of the reaction-diffusion equations. This is simplified by using and average diffusivity matrix or by evaluating the diffusivity matrix at each location. This simplification is still not as unrealistic as the usage of the film model for the extracatalyst equation as discussed in Section 2.4. This leads to the set of equations (written for nc-1 species) in terms of central finite difference for the spatial derivatives (as shown in Table 5.6). This approach, however, has several disadvantages, 1) it assumes a half wetted pellet at all times and axial locations, which is not the case in the system of our interest, where the flow modulation causes all three cases i.e., fully externally wetted, half wetted and fully externally dry pellets, 2) this approach assumes constant wetting conditions and is typically suitable for steady state simulation, and, 3) the total number of equations  ((nc-1)x(ncat-1)) is prohibitively large to be solved at each axial location and each time instant due to the use of finite difference (where the discretization points, ncat, required to capture the concentration profiles of all species can be quite large).

Table 5. 6 Equation Set for Single Pellet Model

________________________________________________________________________

FCAT(z,t)=ctCPx1,zcnt+1−x1,zcntdt−ctCPj[β][B]−1[Γ]1,j(xj,zc+1nt+1−2xj,zcnt+1+xj,zc−1nt+1)(Δzc)2+ν1Rzcnt+1ctCPx2,zcnt+1−x21,zcntdt−ctCPj[β][B]−1[Γ]2,j(xj,zc+1nt+1−2xj,zcnt+1+xj,zc−1nt+1)(Δzc)2+ν2Rzcnt+1...ctCPxnc−1,zcnt+1−xnc−1,zcntdt−ctCPj[β][B]−1[Γ]nc−1,j(xj,zc+1nt+1−2xj,zcnt+1+xj,zc−1nt+1)(Δzc)2+νnc−1Rzcnt+1nc−1,ncat−1________________________________________________________________________

ii) Apparent Rate Three Pellet Form

This approach considers all the possible wetting conditions for a slab type of catalyst, i.e., it considers three types of pellets which can have (i) both its side wetted, (ii) a half wetted pellet, and (iii) a pellet with both sides externally dry (Figure 5.7 (b)). But this necessitates the use of three times the number of equations and a correspondingly large computational effort. This is bypassed by using apparent rates with locally calculated effectiveness factors (by assuming that the reaction occurs at the apparent rate at the interface, or at the intrinsic rate in the bulk of the catalyst). This assumption is realistic only at very high thiele modulus (reaction near the surface) and very low thiele modulus (reaction inside the catalyst), but is not suitable for intermediate values of thiele modulii. Nevertheless, it is used due to its advantage of allowing multiple wetting conditions and accumulation of the non-limiting reactant in the catalyst, which can then be consumed during some part of the transient cycle. The governing equations for this case are given for a half-wetted pellet (with sides 1a and 1b).

ctCPxint+1−xintdt−j=1a,1bNiLSajLS−j=1a,1bNiGSajGS−ν1RAppnt+1−ν1Rbulknt+1=0			(5. 6)

Figure 5. 7  Representation of the Catalyst Level Solution

iii) Rigorous Three Pellet Form (Collocation Solution)

The disadvantages of both the approaches discussed above i.e., large computational requirement for rigorous finite difference solution three pellet model (approach ii) and the inability of modeling multiple wetting (in approach (i)) are overcome by using a collocation solution (Villadsen and Michelsen, 1978) for the spatial derivative which requires less number of computational points and hence less number of equations to be solved. This approach does not have the shortcomings of the approach ii, but does require extra effort in solution of the catalyst level equations, particularly since both the multicomponent and the collocation solution involve matrices for diffusivity ([][B]-1[]) and spatial second derivatives [Bcol]. The set of equations thus can be written as shown in Table 5.7

Table 5. 7 Catalyst Level Equations for Rigorous Three Pellet Model

_______________________________________________________________

FCAT(z,t)=c1,icnt+1−c1,icntdt−isp=1nc−1jj=1ncol[β][B]−1[Γ]1,isp[Bcol]ic,jjcisp,jj−ν1Ricnt+1 . .(ic=1,2,....ncol for species 1, pellet I)c2,icnt+1−c2,icntdt−isp=1nc−1jj=1ncol[β][B]−1[Γ]2,isp[Bcol]ic,jjcisp,jj−ν2Ricnt+1 . .(ic=1,2,....ncol for species 2, pellet I)cnc−1,icnt+1−cnc−1,icntdt−isp=1nc−1jj=1ncol[β][B]−1[Γ]2,isp[Bcol]ic,jjcisp,jj−νnc−1Ricnt+1 . . (ic=1,2,....ncol for species 2, pellet I). . .ncol∗(nc−1) equations for each pellet3(nc−1)ncol

_______________________________________________________________

Table 5. 8 List of Model Variables and Equations

(component i, c = total number of components)

| Variable | Symbol | # of variables | Equation basis | # of Equations | Equation / Table |
| --- | --- | --- | --- | --- | --- |
| Liquid phase concentration | CiL | c-1 | Liquid phase mass balance | c-1 | Equation 5.1 |
| Gas phase concentration | CiG | c-1 | Gas phase mass balance | c-1 | Equation 5.2 |
| Gas temperature | TG | 1 | Gas phase energy balance | 1 | Equation 5.3 |
| Liquid temperature | TL | 1 | Liquid phase energy balance | 1 | Equation 5.5 |
| Temperature of Catalyst | TS | 1 | Solid phase energy balance | 1 | Equation 5.7 |
| Liquid holdup | eL | 1 | Liquid phase continuity | 1 | Equation 5.9 |
| Gas holdup | eG | 1 | Gas phase continuity | 1 | Equation 5.10 |
| Velocity of Gas | UG | 1 | Gas phase momentum | 1 | Equation 5.14 |
| Velocity of Liquid | UL | 1 | Liquid phase momentum | 1 | Equation 5.13 |
| Gas density | rG | 1 | Gas Equation of State | 1 | Appendix E |
| Liquid density | rL | 1 | Liquid density equation | 1 | Appendix E |
| Mass transfer of i across gas-liquid interface | NiGL | c | Gas phase multicomponent transfer | c-1 | Table 5.3 |
|  |  |  | Liquid phase multicomponent transfer | c-1 | Table 5.3 |
| Interface vapor mole fraction of i | YiI | c | G-L Interface equilibrium | c | Table 5.3 |
| Interface liquid mole fraction of I | XiI | c |  |  |  |
| Interface (G-L) temperature | TIGL | 1 | V-L Interface energy Flux Balance | 1 | Table 5.5 |
|  |  |  | V-L interface vapor mole fraction sum | 1 | Table 5.3 |
|  |  |  | V-L interface liquid mole fraction sum | 1 | Table 5.3 |
| Liquid-solid mass transfer flux | NiLS | c | Liquid solid mass transfer | c-1 | Table 5.4 |
| Liquid-solid interface concentration | xiLS | c | Catalyst side flux relations | c-1 | Table 5.4 |
|  |  |  | L-S interface mole fraction sum | 1 | Table 5.4 |
|  |  |  | Zero net volumetric flux | 1 | Equation 5.35 |
| Temperature of Catalyst | TILS | 1 | Liquid-Solid energy flux balance | 1 | Table 5.5 |
| Gas-solid mass transfer rate | NiGS | c | Gas-solid mass transfer | c-1 | Table 5.4 |
| Gas-Solid interface compositions | xiGS | c | Catalyst side flux relations | c-1 | Table 5.4 |
| Gas-Solid interface compositions | yiLS |  | Equilibrium equations on catalyst surface | c | Table 5.4 |
|  |  |  | G-S interface mole fraction sum | 1 | Table 5.4 |
|  |  |  | G-S interface mole fraction sum | 1 | Table 5.4 |
|  |  |  | Liquid-Solid energy flux balance | 1 | Table 5.5 |
| Intra-catalyst concentrations |  |  | Approach (i) |  | Table 5.6 |
|  |  |  | Approach (ii) |  | Equation 5.38 |
|  |  |  | Approach (iii) |  | Table 5.7 |

DISCUSSION OF RESULTS

A four component system is considered in the simulation of hydrogenation of Alpha-methylstyrene to Cumene in n-hexane solvent. The conditions of the bed, catalyst activity, rate form, flow rates etc. are as in section 4.3. The transient simulation of the apparent rate three pellet approach (approach ii) yields a set of equations ranging from 92 to 116 depending upon the wetting conditions which are solved using an implicit finite difference approach with simultaneous solution of the non-linear system of equations resulting from the Maxwell-Stefan equations using either the multivariable Newton method with linear search as discussed earlier in this chapter. For the pseudo-transient simulation to steady state, the velocity and holdup of each phase is supplied to the code, and concentration, flux, and temperature profiles are simulated. Liquid inlet flow at the feed concentration and temperature is specified and the reaction transport equations are solved at each time at all the axial locations to follow the development of concentration profiles in the gas, bulk liquid, and catalyst phase until steady state is achieved.

Figures 2 and 3 show the alpha-methylstyrene concentration profile development  at different locations in the reactor with time. As can be seen in the Figures 2 and 3 the alpha-methylstyrene concentration reaches steady state relatively early compared to the cumene concentration profiles which are shown (Figure 4). Since this is the case of gas-limited conditions with zero order rate with respect to alpha-methylstyrene, linear concentration profiles for steady state profiles are observed as expected. Figures 4 and 5 show the buildup of liquid phase cumene and hydrogen concentration at different axial positions in time. The liquid phase hydrogen concentration is seen to build to saturation at the exit where low alpha-methylstyrene concentration is present, whereas at the inlet it is consumed faster than transferred from the gas phase and results in much lower concentrations as indicated by the lower curves in Figure 5.

Figure 5. 8: Transient Alpha-methylstyrene (-MS) Concentration Profiles at Different Axial Locations

Figure 5. 9 Transient Alpha-methylstyrene Concentration Profile Development with Time (shown in seconds in the legend table).

Figure 5. 10 Axial Profiles of Cumene Concentration at Different Simulation Times (shown in seconds in the legend table)

Figure 5. 11 Transient Hydrogen Concentration Profiles at Different Axial Locations

Flow Modulation : Velocity and Holdup Simulation

A typical set of simulated velocity and holdup profiles for a total cycle time of 80 seconds and LIQUID ON time of 15 seconds are shown in Figure 5.12 and 5.13. The liquid flow rate used is 42 ml/min (1.4 kg/m2s) during LIQUID ON time and 2 ml/min (0.067 kg/m2s) during the rest of the cycle with gas flow of 400 cc/min (0.0192 kg/m2s). The inlet pulse is seen to be sharp with significant spreading as the liquid pulse travels down the reactor. This is observed even with an order of magnitude reduction in spatial and temporal step size and corroborates experimental observations.

Figure 5. 12 Transient Liquid Holdup Profiles at Different Axial Locations in Periodic Flow

Figure 5. 13 Transient Liquid Velocity Profiles at Different Axial Locations in Periodic Flow

Periodic Flow: Reaction-Transport Simulation

This involves the solution of the transient reaction transport equations (described in section 5._) when coupled with the flow simulation (described in section 5._) for reaching a steady state and then with periodic flow modulation. The coupling between evaporative fluxes and their influence on holdup is solved in an explicit manner at each step for the momentum equations. The converged velocity and holdup profile is then used to do the rest of the reaction transport calculations implicitly. Reaction rates at steady state and time averaged rates under periodic flow conditions are then compared to evaluate performance under unsteady state conditions. For a test case with liquid flow cycling for 20 seconds each  with liquid ON flow mass velocity of 1.01 kg/m2s and OFF (Base) flow mass velocity of 0.05 kg/m2s with a constant gas mass velocity of 0.017 kg/m2s, typical product (cumene) concentration in the bulk liquid phase is shown in Figure 8. It can be observed that the access of gas and liquid and hence the rate of product (cumene) formation is enhanced during the periodic flow modulation cycle. Figures 9 and 10 show intra-catalyst concentration of hydrogen (in a previously wetted pellet with very small reaction rate) and concentration of alpha-methylstyrene in a previously dry pellet during the entire 40 second cycle. This demonstrates the access (to the catalyst) of gaseous reactant during the OFF cycle and that of the liquid reactant during the ON cycle, which can cause enhancement in overall reactor performance.

Figure 5. 14 Cumene Concentration Profiles during Periodic Flow Modulation

Figure 5. 15 Intra-catalyst Hydrogen Concentration Profiles during Flow Modulation for a Previously Externally Wetted Catalyst Pellet at Different Axial Locations.

Figure 5. 16 Intra-Catalyst Alpha-methylstyrene Concentration Profiles during Flow Modulation for a Previously Externally Dry Catalyst Pellet at Different Axial Locations.

# Chapter 6. Conclusions

Performance (conversion) comparison of downflow and upflow modes of operation of laboratory packed bed reactors over a range of identical feed and pressure conditions yields two conclusions. Gas limited conditions imply better downflow performance (due to easier access of  the gas to the partially externally wetted catalyst areas in downflow), while liquid limited conditions result in better upflow performance over downflow. Conversion is seen to improve with pressure in the gas limited domain and remains fairly constant once liquid limitation sets in. Zero order behavior with respect to liquid reactant feed concentration is exhibited at the gas limited end, and first order behavior with respect to the liquid reactant (either transport or kinetic) at the liquid limited end. Gas velocity has no significant effect on the performance of either mode of operation in the range of velocities examined.

Studies of the downflow and upflow mode of operation in bed diluted with fines under identical conditions show that fines do indeed neutralize the differences between the two modes of operation for both gas and liquid limited conditions and successfully decouple hydrodynamics (flow pattern and wetting) and "apparent" kinetics. This implies that the performance of the diluted bed is not dependent on the reactant limitation and flow mode used. This conclusion is important in establishing the use of fines in laboratory scale reactors as an effective and viable scale-up tool possibly to be preferred to upflow reactors. While one can argue that upflow reactors also provide rates on completely wetted particles and, hence, to some extent decouple the kinetics and hydrodynamics, they do so only at the expense of increased liquid holdup which can alter the relative amount of homogeneous and heterogeneous reactions occurring in more complex systems and should be avoided. In addition, upflow can lead to more dispersion and flow non-idealities than encountered in beds with fines. Finally, holdup and flow regimes in diluted beds are closer to those in trickle beds than in upflow.

Further understanding and quantitative comparison of the performance with and without fines, under upflow and downflow conditions, requires detailed modeling which is currently under investigation.

The performance of upflow and downflow is reaction system dependent i. e., whether the reaction is gas or liquid limited under the conditions of investigation. The laboratory reactors are operated in the range of partially to fully wetted catalyst to demonstrate that the influence of wetting can be detrimental or beneficial, depending upon the reactant limitation. Models that account for these two effects can predict the performance over the entire range of operating conditions.

The intrinsic kinetics of the reaction studied is different at different pressures and hence it is recommended to study slurry kinetics at the operating pressure before any scale up or modeling is attempted. A rate expression with different rate constants at different pressures can be used to predict the trickle bed reactor data well.

The predictions of the reactor scale and pellet scale models are satisfactory for current conditions although there is a need for high pressure correlation for mass transfer coefficient and interfacial area in order to predict performance with greater certainty, especially in cases where the rate is affected significantly by external mass transfer. Unsteady state liquid flow modulation (periodic operation) has been investigated in this study using hydrogenation of alpha-methylstyrene to cumene in a hexane solvent over 0.5% Pd on alumina spheres as a test reaction for gas and liquid limited conditions. Performance enhancement has been observed for a wide range of operating conditions with ON/OFF flow modulation under gas reactant limitation, but seen to be strongly dependent upon the extent of catalyst wetting under liquid limited conditions and observed with BASE/PEAK flow modulation. The enhancement feasibility has been shown to be dependent upon the extent of reactant limitation and mean liquid mass velocity.  Performance enhancement has shown to be dependent upon the induced flow modulation (IFM) frequency and discussed in relation to the natural frequency of the performance governing process. Further experiments are necessary to investigate this effect to obtain insight into the cause of enhancement under a wide range of operating conditions, particularly when the rate controlling processes are different. Test simulation results show possibility of describing dynamic access of gaseous and liquid reactants and has to be tested for the entire set of experimental conditions presented here and in literature.

## 6.1 Recommendations for Future Work

# Appendix A. Slurry Experiments: Intrinsic Rate  At High Pressure

Generation of intrinsic kinetic data was necessary before any trickle bed model predictions could be attempted. Hence, the reaction rate in a slurry reactor at different pressures and over a range of initial liquid reactant concentrations was investigated. This was done in a high pressure slurry reactor using crushed catalyst (< 50 m size). Sufficient agitation was maintained to ensure no external transport limitations in the system, along with a supply of fresh hydrogen to replace whatever was consumed and, hence, maintain saturation concentration of hydrogen in the liquid phase.

Slurry reactions at low pressure (30 psig) showed the expected dependence (El-Hisnawi, 1981) of zero order with respect to a-methylstyrene concentration. A peculiar behavior, encountered sometimes only at very high pressure (>1000 psig), was observed in our high pressure experiments (max. pressure 300 psig) as shown in the Figure 15 below. 	The rate for a system with approximately the same liquid reactant concentration increased when pressure was raised from 30 to 100 psig but decreased when pressure was increased further from 100 to 200 and further to 300 psig. This was further augmented by the slight inhibiting effect observed at high pressure which could be attributed due to the presence of cumene, particularly at higher conversions when 8-10% (volume cumene/volume liquid solution) cumene is present in the system. This pressure dependence, and product inhibition, has been observed at pressures over 1000 psig in commercial hydrogenation and hydrodesulphurizations (Satterfield and Roberts, 1968).  A mechanism based single rate form was first used to fit the data. Different single and dual site adsorption-reaction mechanisms were tried by considering a shift in the rate limiting step. No simple mechanism was capable of representing this reaction at all pressures, and since the primary objective was to study and predict trickle bed and upflow performance at the same discrete pressure values, separate fits were used at each pressure for the general Langmuir-Hinshelwood rate expression as shown below. The hydrogen concentration dependence was lumped in the numerator in the rate constant and the adsorption equilibrium constants were obtained by a constrained non-linear fit to at least 30 points of data at each pressure. The fitted parameters at different pressures are presented in Table 7 (Khadilkar et al., 1996).

Table A. 1 Rate Constants Obtained from Slurry Data at Different Pressures

| Pressure (psig) | kvs (m3liq./m3cat./s) *(mol/m3 liq)m'-1 | K1 | K2 | b |
| --- | --- | --- | --- | --- |
| 30 | 0.0814 | 0 | 0 | 0 |
| 100 | 1.14 | 4.41 | 11.48 | 1 |
| 200 | 0.022 | 2.73x10-2 | 2.1x10-2 | 2 |

Figure A. 1 Slurry conversion versus time at different pressures

Figure A. 2 Comparison of the Model Fitted Alpha-methylstyrene concentrations to experimental values

# Appendix B. Correlations Used In Model Evaluation

Downflow correlations

El-Hisnawi (1982) ηce=1.104ReL0.331+[ΔPZ)ρLg]GaL19	Al-Dahhan and Dudukovic (1995)

Tan and Smith (1980)

Fukushima and Kusaka (1977)

Fukushima and Kusaka (1977)

Dwivedi and Upadhyah (1977)

Lakota and Levec (1989)

Upflow Correlations

Specchia (1978)

Reiss (1967)

# Appendix C. Flow Charts For The Unsteady State Simulation Algorithm

# Appendix D Maxwell-Stefan Equations For Multicomponent Transport

Solution of simultaneous mass and energy transfer equations:

For a system with n components, the number of unknowns in the problem are 2n mole fractions at the interface (xiI and yiI), n molar fluxes (incorporating NiL=NiV=NiVL ) and an interface temperature (TI). Since energy equilibrium at the interface is also considered, it acts as a bootstrap to relate the total flux to the absolute quantities without needing any further assumptions (of inert flux being zero or stoichiometric fluxes etc.). Thus the total number of variables are 3n+1. The number of equations relating these variables is

a) n-1 rate equations for the liquid phase given by

(D. 1)

where     (in matrix form, (n-1)x(n-1)) 		(D. 2)

and Lk=(lk-ln)/lx  , Dq=h.L(TI-TL)-h.V(TG-TI)					(D. 3)

(   and ,   )					(D. 4)

b) n-1 rate equations for the vapor(gas) phase given by

(D. 5)

where      (in matrix form, (n-1)x(n-1))		(D. 6)

c) n equilibrium relations    						(D. 7)

d) Interfacial energy balance

(D. 8)

e) Two more equations are necessary to complete the set and are given by mole fraction sum for both gas and liquid compositions at the interface.

(D. 9)

(D. 10)

This completes the set of equations required to solve for all the fluxes, compositions and temperatures. The mass and heat transfer coefficients are the corrected values at the fluxes and can be revised based on this flux calculation until converged values are obtained. Details of iteration for finite flux transfer coefficients can be found in Taylor and Krishna (1993).

- The “Bootstrap” matrix  for interphase transport

- Bootstrap matrix for gas transport is evaluated based on interphase energy flux for the gas -liquid interface

[βi,k]G=dik−yiLk									(D. 11)

where Lik=(λk−λnc)λy								(D. 12)

and λy=iyiλi , λi=yi(HiV(@TG)−HiL(@TL))					(D. 13)

For the gas-solid interface similar equations are used except the enthalpy is now  of the liquid in the solid (liquid at solid temperature).

- Bootstrap matrix for the liquid phase is also based on the energy flux term and given by

[βi,k]L=dik−xiLk									(D. 14)

where Lik=(λk−λnc)λx								(D. 15)

and λx=ixiλi , λi=xi(HiV(@TG)−HiL(@TL))					(D. 16)

- Catalyst level equations have unsymmetric boundary conditions due to gas and liquid bulk flow streams on either side of partially wetted pellets and suitable boundary conditions have to be incorporated into the bootstrap matrix to ensure that mass balance is satisfied in the pellets. Several bootstraps are possible in this case such as stoichiometric fluxes, zero flux for inert component, and net zero flux for zero net mole production. But these are not deemed suitable for the present problem. Hence the most likely  condition would be that net volumetric flux entering on the wetted side is equal to net mass flux leaving on the dry side. This can be incorporated directly in the fully implicit beta evaluation as

(NiMiρLi)L−S=(NiMiρLi)G−S						(D. 17)

- or in general since beta is not evaluated at each intra-catalyst location, it can be       represented as

[βi,k]CP=dik−xciLk									(D. 18)

where Lk=(Mk−Mnc)λmx							(D. 19)

and λmx=ixiMi 									(D. 20)

- B) The transport matrices for interphase mass and energy flux

- 1. For the gas phase the low flux mass transfer coefficient matrix can be calculated as

[k]G=[B]−1[G]									(D. 21)

where [Bi,j]=yiki,nc+kykki,k  (for i = j )						(D. 22)

[Bi,j]=-yi(1ki,j−1ki,nc) (for i  j)							(D. 23)

- Here [ki,j]=Di,jl where the diffusivity coefficients are calculated from infinite dilution diffusivities (from kinetic theory) using mole fraction weighting factors.  The [] matrix is the non-ideality coefficient matrix, which is considered to be identity for the gas side.

- The liquid phase mass transfer coefficient matrix are calculated similarly as

[k]L=[B]−1[G]									(D. 24)

where [Bi,j]=yiki,nc+kykki,k  (for i = j )						(D. 25)

[Bi,j]=-yi(1ki,j−1ki,nc) (for i  j)							(D. 26)

- Here [ki,j]=Di,jl where the diffusivity coefficients are calculated from infinite dilution diffusivities (using the Wilke-Chang correlation as given in Appendix E) using mole fraction weighting factors.  The [] matrix is the non-ideality coefficient matrix, which is considered to be identity for the gas side.

C. High flux correction

- High flux correction is important for the gas side mass and heat transfer coefficients and has been incorporated as below.

Determine [] matrix based on the calculated fluxes

Fii=Nictkin+Nkctkik									(D. 27)

Fii=-Ni(1ctkij−1ctkin)								(D. 28)

correct the low flux mass transfer coefficient matrix by the correction as

[k]highflux=[k]lowflux[F][exp([F])−[I]]−1						(D. 29)

where exp[] is evaluated by a Pade approximation as

exp[F]=[I]+[F]2[I]−[F]2								(D. 30)

Similarly, heat transfer coefficient is also corrected by

Fht=NiCpih									(D. 31)

hhighflux=hlowfluxFht((exp(Fht)−1)							(D. 32)

# Appendix E Evaluation Of Parameters For Unsteady State Model

Liquid phase infinite dilution diffusivities are determined using the Wilke and Chang (1955) correlation as

Dij0=7.4x10−12(φjMj)12TμjVi0.6								(E. 1)

Gas phase infinite dilution diffusivities are determined as given by Reid et al. (1987)

Dij0=1.013x10−2((Mj+Mj)MiMj)12T1.75P(Vi13+Vi13)2						(E. 2)

These are corrected for concentration effect using the Vignes (1966) correlation as recommended by taylor and Krishna (1993)

Dij=(Dij0)(1−xi)(Dji0)(xi)								(E. 3)

The film thickness for gas and liquid phase is determined using the Sherwood correlation by using the particle diameter as the characteristic dimension as

dPℓ=0.023Re0.83Sc0.44								(E. 4)

The Schmidt number used in the above correlation is different for each species and can give multiple values of film thickness according to the species used in the calculation. This dilemma is resolved using either an average value of diffusivity or the diffusivity of the gas phase key reactant.

Heat transfer coefficients for gas and liquid phase are determined using either the Nusselt (E.5 ) number or the Chilton-Colburn analogy (E.6) as recommended by Taylor and Krishna (1993)

hαdPkα=Pr13(2.14Re12+0.99)							(E. 5)

hα=ctαCPαkαmt(PrSc)−23								(E. 6)

The interfacial areas required for interphase transport (gas-liquid, liquid-solid, and gas-solid) are evaluated using the following correlations. ,

aGL=3.9x10−4(1−εLεB)(1dP)ReL0.4(dPDR)−2						(E. 7)

aLS=(1−εB)(VPSX)ηCE									(E. 8)

aGS=(1−εB)(VPSX)(1−ηCE)								(E. 9)

The modeling of liquid phase non-ideality effects can be done by using any of the several approaches available (such as simple 2 parameter models such as the Margules, Van-Laar, or the Wilson equation or complex rigorous methods such as the UNIQUAC or UNIFAC). In the present study, the Wilson equation was used to obtain the equilibrium constants as well as the activity matrix ([]) as given by the equations below.

For the Wilson equation, the excess energy Q is given by

Q=−i=1nxiln(Si)									(E. 10)

where

Si=−j=1nxjΛij										(E. 11)

Λij=VjViexp(−(μij−μii)RT)								(E. 12)

First and second derivatives of Q w.r.t. compositions i and j are then given by

Qi=−ln(Si)−k=1nxkΛkiSk								(E. 13)

Qij=−ΛijSi−ΛjiSj+k=1nxkΛkiΛkjSk2						(E. 14)

The activity coefficients needed for calculating the equilibrium constants are then given directly by

γi=exp(χ+Qi)									(E. 15)

The thermodynamic factors (ij) required for the nonideality matrix are given by

Γij=δij+xi(Qij−Qin)								(E. 16)

# Appendix F. Experimental Data From Steady And Unsteady Experiments

Table F.1 Performance of Downflow and Upflow Reactors under Gas and Liquid Limited Conditions

| Mode | Space time (s) | Pressure (psig) | Feed Concentration (% v/v) | Conversion |
| --- | --- | --- | --- | --- |
| Upflow | 32 | 30 | 7.8 | 0.036 |
|  | 53 | 30 | 7.8 | 0.065 |
|  | 80 | 30 | 7.8 | 0.079 |
|  | 155 | 30 | 7.8 | 0.169 |
|  | 297 | 30 | 7.8 | 0.447 |
| Downflow | 32 | 30 | 7.8 | 0.054 |
|  | 53 | 30 | 7.8 | 0.088 |
|  | 80 | 30 | 7.8 | 0.133 |
|  | 155 | 30 | 7.8 | 0.28 |
|  | 297 | 30 | 7.8 | 0.638 |
| Upflow | 155 | 200 | 3.1 | 0.729 |
|  | 105 | 200 | 3.1 | 0.510 |
|  | 64 | 200 | 3.1 | 0.319 |
|  | 46 | 200 | 3.1 | 0.247 |
| Downflow | 155 | 200 | 3.1 | 0.63 |
|  | 105 | 200 | 3.1 | 0.428 |
|  | 64 | 200 | 3.1 | 0.289 |
|  | 46 | 200 | 3.1 | 0.231 |
| Upflow | 190 | 30 | 3.1 | -- |
|  | 155 | 30 | 3.1 | 0.404 |
|  | 105 | 30 | 3.1 | 0.265 |
|  | 64 | 30 | 3.1 | 0.188 |
| Downflow | 190 | 30 | 3.1 | 0.534 |
|  | 155 | 30 | 3.1 | 0.397 |
|  | 105 | 30 | 3.1 | 0.254 |
|  | 64 | 30 | 3.1 | 0.183 |

Table F.1 Performance of Downflow and Upflow Reactors under Gas and Liquid Limited Conditions (continued)

| Upflow | 190 | 200 | 4.8 | -- |
| --- | --- | --- | --- | --- |
|  | 155.4 | 200 | 4.8 | 0.750 |
|  | 105.2 | 200 | 4.8 | 0.505 |
|  | 64.5 | 200 | 4.8 | 0.325 |
|  | 45.6 | 200 | 4.8 | 0.252 |
| Downflow | 190 | 200 | 4.8 | 0.801 |
|  | 155.4 | 200 | 4.8 | 0.735 |
|  | 105.2 | 200 | 4.8 | 0.448 |
|  | 64.5 | 200 | 4.8 | 0.290 |
|  | 45.6 | 200 | 4.8 | -- |

Table F.2 Pressure Effects on Upflow and Downflow Performance

| Mode | Space time (s) | Pressure (psig) | Feed Concentration (% v/v) | Conversion |
| --- | --- | --- | --- | --- |
| Upflow | 190 | 30 | 3.1 | 0.462 |
|  | 155 | 30 | 3.1 | 0.405 |
|  | 105 | 30 | 3.1 | 0.266 |
|  | 64 | 30 | 3.1 | 0.188 |
|  | 46 | 30 | 3.1 | -- |
|  | 190 | 100 | 3.1 | 0.874 |
|  | 155 | 100 | 3.1 | 0.711 |
|  | 105 | 100 | 3.1 | 0.497 |
|  | 64 | 100 | 3.1 | 0.325 |
|  | 46 | 100 | 3.1 | -- |
|  | 190 | 200 | 3.1 | -- |
|  | 155 | 200 | 3.1 | 0.729 |
|  | 105 | 200 | 3.1 | 0.51 |
|  | 64 | 200 | 3.1 | 0.319 |
|  | 46 | 200 | 3.1 | 0.247 |
| Downflow | 190 | 30 | 3.1 | 0.534 |
|  | 155 | 30 | 3.1 | 0.397 |
|  | 105 | 30 | 3.1 | 0.254 |
|  | 64 | 30 | 3.1 | 0.183 |
|  | 46 | 30 | 3.1 | -- |
|  | 190 | 100 | 3.1 | 0.648 |
|  | 155 | 100 | 3.1 | 0.599 |
|  | 105 | 100 | 3.1 | 0.42 |
|  | 64 | 100 | 3.1 | 0.279 |
|  | 46 | 100 | 3.1 | -- |
|  | 190 | 200 | 3.1 | -- |
|  | 155 | 200 | 3.1 | 0.632 |
|  | 105 | 200 | 3.1 | 0.429 |
|  | 64 | 200 | 3.1 | 0.289 |
|  | 46 | 200 | 3.1 | 0.231 |

Table F.2 Pressure Effects on Upflow and Downflow Performance (continued)

| Downflow | 214 | 30 | 4.8 | -- |
| --- | --- | --- | --- | --- |
|  | 190 | 30 | 4.8 | 0.357 |
|  | 155 | 30 | 4.8 | 0.262 |
|  | 105 | 30 | 4.8 | 0.15 |
|  | 65 | 30 | 4.8 | 0.112 |
|  | 214 | 100 | 4.8 | 0.741 |
|  | 190 | 100 | 4.8 | -- |
|  | 155 | 100 | 4.8 | 0.55 |
|  | 105 | 100 | 4.8 | 0.363 |
|  | 65 | 100 | 4.8 | 0.236 |
|  | 214 | 200 | 4.8 | -- |
|  | 190 | 200 | 4.8 | 0.801 |
|  | 155 | 200 | 4.8 | 0.735 |
|  | 105 | 200 | 4.8 | 0.448 |
|  | 65 | 200 | 4.8 | 0.29 |
| Upflow | 214 | 30 | 4.8 | -- |
|  | 190 | 30 | 4.8 | 0.385 |
|  | 155 | 30 | 4.8 | 0.243 |
|  | 105 | 30 | 4.8 | 0.173 |
|  | 65 | 30 | 4.8 | 0.12 |
|  | 46 | 30 | 4.8 | -- |
|  | 214 | 100 | 4.8 | 1 |
|  | 190 | 100 | 4.8 | -- |
|  | 155 | 100 | 4.8 | 0.746 |
|  | 105 | 100 | 4.8 | 0.411 |
|  | 65 | 100 | 4.8 | 0.256 |
|  | 46 | 100 | 4.8 | -- |
|  | 214 | 200 | 4.8 | -- |
|  | 190 | 200 | 4.8 | -- |
|  | 155 | 200 | 4.8 | 0.75 |
|  | 105 | 200 | 4.8 | 0.505 |
|  | 65 | 200 | 4.8 | 0.325 |
|  | 46 | 200 | 4.8 | 0.252 |

Table F.3 Alpha-methylstyrene Feed Concentration Effect on Upflow and Downflow Performance

| Mode | Space time (s) | Pressure (psig) | Feed Concentration (% v/v) | Conversion |
| --- | --- | --- | --- | --- |
| Downflow | 214 | 100 | 3.1 | -- |
|  | 190 | 100 | 3.1 | -- |
|  | 155 | 100 | 3.1 | 0.41 |
|  | 105 | 100 | 3.1 | 0.238 |
|  | 80 | 100 | 3.1 | 0.198 |
|  | 65 | 100 | 3.1 | 0.158 |
|  | 214 | 100 | 4.8 | 0.741 |
|  | 190 | 100 | 4.8 | -- |
|  | 155 | 100 | 4.8 | 0.55 |
|  | 105 | 100 | 4.8 | 0.363 |
|  | 80 | 100 | 4.8 | -- |
|  | 65 | 100 | 4.8 | 0.236 |
|  | 214 | 100 | 6.8 | -- |
|  | 190 | 100 | 6.8 | 0.648 |
|  | 155 | 100 | 6.8 | 0.599 |
|  | 105 | 100 | 6.8 | 0.42 |
|  | 80 | 100 | 6.8 | -- |
|  | 65 | 100 | 6.8 | 0.279 |
| Upflow | 214 | 100 | 3.1 | -- |
|  | 190 | 100 | 3.1 | 0.874 |
|  | 155 | 100 | 3.1 | 0.711 |
|  | 105 | 100 | 3.1 | 0.497 |
|  | 80 | 100 | 3.1 | -- |
|  | 65 | 100 | 3.1 | 0.328 |
|  | 214 | 100 | 4.8 | 1 |
|  | 190 | 100 | 4.8 | -- |
|  | 155 | 100 | 4.8 | 0.746 |
|  | 105 | 100 | 4.8 | 0.411 |
|  | 80 | 100 | 4.8 | -- |
|  | 65 | 100 | 4.8 | 0.256 |
|  | 214 | 100 | 6.8 | -- |

### Table F.3 Alpha-methylstyrene Feed Concentration Effect on Upflow and Downflow Performance (continued)

|  | 190 | 100 | 6.8 | -- |
| --- | --- | --- | --- | --- |
|  | 155 | 100 | 6.8 | 0.465 |
|  | 105 | 100 | 6.8 | 0.267 |
|  | 80 | 100 | 6.8 | 0.215 |
|  | 65 | 100 | 6.8 | 0.175 |
| Downflow | 190 | 200 | 3.1 | -- |
|  | 155 | 200 | 3.1 | 0.632 |
|  | 105 | 200 | 3.1 | 0.429 |
|  | 64 | 200 | 3.1 | 0.289 |
|  | 46 | 200 | 3.1 | 0.231 |
|  | 190 | 200 | 4.8 | 0.801 |
|  | 155 | 200 | 4.8 | 0.735 |
|  | 105 | 200 | 4.8 | 0.448 |
|  | 64 | 200 | 4.8 | 0.29 |
|  | 46 | 200 | 4.8 | -- |
| Upflow | 155 | 200 | 3.1 | 0.729 |
|  | 105 | 200 | 3.1 | 0.51 |
|  | 64 | 200 | 3.1 | 0.319 |
|  | 46 | 200 | 3.1 | 0.247 |
|  | 155 | 200 | 4.8 | 0.75 |
|  | 105 | 200 | 4.8 | 0.505 |
|  | 64 | 200 | 4.8 | 0.325 |
|  | 46 | 200 | 4.8 | 0.252 |

Table F. 4 Performance Comparison with Bed Dilution

| Mode | Space time (s) | Pressure (psig) | Feed Concentration (% v/v) | Conversion |
| --- | --- | --- | --- | --- |
| Downflow | 190 | 30 | 6.7 | 0.319 |
|  | 155 | 30 | 6.7 | 0.247 |
|  | 105 | 30 | 6.7 | 0.153 |
|  | 64 | 30 | 6.7 | 0.103 |
| Upflow | 190 | 30 | 6.7 | 0.283 |
|  | 155 | 30 | 6.7 | 0.224 |
|  | 105 | 30 | 6.7 | 0.152 |
|  | 64 | 30 | 6.7 | 0.102 |
| Downflow | 155 | 200 | 3.1 | 0.715 |
|  | 105 | 200 | 3.1 | 0.501 |
|  | 64 | 200 | 3.1 | 0.312 |
|  | 46 | 200 | 3.1 | 0.241 |
| Upflow | 155 | 200 | 3.1 | 0.72 |
|  | 105 | 200 | 3.1 | 0.482 |
|  | 64 | 200 | 3.1 | 0.302 |
|  | 46 | 200 | 3.1 | 0.237 |
| Downflow | 190 | 30 | 3.18 | 0.651 |
|  | 155 | 30 | 3.18 | 0.451 |
|  | 105 | 30 | 3.18 | 0.251 |
|  | 64 | 30 | 3.18 | 0.182 |
|  | 46 | 30 | 3.18 | -- |
|  | 190 | 30 | 6.7 | 0.32 |
|  | 155 | 30 | 6.7 | 0.248 |
|  | 105 | 30 | 6.7 | 0.154 |
|  | 64 | 30 | 6.7 | 0.104 |
|  | 46 | 30 | 6.7 | -- |
|  | 190 | 200 | 3.18 | 0.816 |
|  | 155 | 200 | 3.18 | 0.501 |
|  | 105 | 200 | 3.18 | 0.313 |
|  | 64 | 200 | 3.18 | 0.241 |

Table F.4 Performance Comparison with Bed Dilution

|  | 46 | 200 | 3.18 | -- |
| --- | --- | --- | --- | --- |
|  | 190 | 200 | 6.7 | 0.916 |
|  | 155 | 200 | 6.7 | 0.786 |
|  | 105 | 200 | 6.7 | 0.504 |
|  | 64 | 200 | 6.7 | -- |
|  | 46 | 200 | 6.7 | 0.228 |
| Upflow | 190 | 30 | 3.18 | 0.618 |
|  | 155 | 30 | 3.18 | 0.418 |
|  | 105 | 30 | 3.18 | 0.257 |
|  | 64 | 30 | 3.18 | 0.156 |
|  | 46 | 30 | 3.18 | -- |
|  | 190 | 30 | 6.7 | 0.284 |
|  | 155 | 30 | 6.7 | 0.225 |
|  | 105 | 30 | 6.7 | 0.152 |
|  | 64 | 30 | 6.7 | 0.102 |
|  | 46 | 30 | 6.7 | -- |
|  | 190 | 200 | 3.18 | -- |
|  | 155 | 200 | 3.18 | 0.729 |
|  | 105 | 200 | 3.18 | 0.482 |
|  | 64 | 200 | 3.18 | 0.303 |
|  | 46 | 200 | 3.18 | 0.241 |
|  | 190 | 200 | 6.7 | -- |
|  | 155 | 200 | 6.7 | 0.827 |
|  | 105 | 200 | 6.7 | 0.487 |
|  | 64 | 200 | 6.7 | 0.307 |
|  | 46 | 200 | 6.7 | 0.233 |

Table F. 5 Gas Velocity Effect on Upflow and Downflow Performance

| Mode | Space time (s) | Pressure (psig) | Feed Concentration (% v/v) | Conversion |
| --- | --- | --- | --- | --- |
| Downflow | 1.39 | 100 | 4.1 | 0.287 |
|  | 2.78 | 100 | 4.1 | 0.298 |
|  | 4.18 | 100 | 4.1 | 0.290 |
|  | 5.57 | 100 | 4.1 | 0.277 |
|  | 6.97 | 100 | 4.1 | 0.282 |
|  | 8.36 | 100 | 4.1 | 0.279 |
| Upflow | 1.39 | 100 | 4.1 | 0.305 |
|  | 2.78 | 100 | 4.1 | 0.301 |
|  | 4.18 | 100 | 4.1 | 0.298 |
|  | 5.57 | 100 | 4.1 | 0.290 |
|  | 6.97 | 100 | 4.1 | 0.302 |
|  | 8.36 | 100 | 4.1 | 0.293 |

Table F. 6 Experimental Data from High Pressure Slurry Experiments

| Time (min) | Alpha-methylstyrene (% v/v) | Cumene (% v/v) | Hydrogen Concentration (mol/m3) | Alpha-methylstyrene Concentration (mol/m3) | Cumene Concentration (mol/m3) |
| --- | --- | --- | --- | --- | --- |
| 0 | 4.89 | 6.56 | 14.08 | 377.17 | 471.34 |
| 20 | 4.71 | 6.73 | 14.08 | 363.88 | 483.52 |
| 40 | 4.48 | 7.12 | 14.08 | 345.60 | 511.70 |
| 60 | 4.33 | 7.34 | 14.08 | 333.97 | 526.93 |
| 90 | 4.11 | 7.44 | 14.08 | 317.36 | 534.54 |
| 110 | 3.64 | 7.69 | 14.08 | 280.80 | 552.05 |
| 140 | 3.25 | 8.22 | 14.08 | 250.89 | 590.13 |
| 170 | 3.10 | 8.63 | 14.08 | 239.26 | 619.82 |
| 210 | 2.88 | 8.82 | 14.08 | 222.65 | 633.53 |
| 0 | 4.20 | 3.75 | 14.08 | 324.00 | 269.56 |
| 10 | 3.96 | 3.90 | 14.08 | 305.73 | 280.22 |
| 20 | 3.73 | 3.88 | 14.08 | 288.28 | 278.69 |
| 40 | 3.64 | 4.06 | 14.08 | 280.80 | 291.64 |
| 60 | 3.49 | 4.40 | 14.08 | 269.17 | 316.00 |
| 90 | 3.13 | 4.67 | 14.08 | 241.76 | 335.80 |
| 110 | 2.93 | 4.78 | 14.08 | 225.97 | 343.42 |
| 140 | 2.63 | 5.12 | 14.08 | 202.71 | 367.78 |
| 170 | 2.41 | 5.37 | 14.08 | 186.09 | 386.06 |
| 210 | 2.14 | 5.65 | 14.08 | 165.32 | 405.86 |
| 240 | 1.89 | 5.96 | 14.08 | 146.22 | 427.94 |
| 0 | 4.21 | 2.22 | 14.08 | 324.83 | 159.14 |
| 10 | 4.13 | 2.26 | 14.08 | 319.02 | 162.19 |
| 20 | 3.97 | 2.47 | 14.08 | 306.56 | 177.42 |
| 30 | 3.74 | 2.64 | 14.08 | 289.11 | 189.60 |
| 50 | 3.57 | 2.82 | 14.08 | 275.82 | 202.55 |
| 70 | 3.36 | 3.34 | 14.08 | 259.20 | 239.86 |
| 120 | 2.81 | 3.66 | 14.08 | 216.83 | 262.70 |
| 150 | 2.38 | 4.14 | 14.08 | 183.60 | 297.73 |
| 240 | 1.81 | 4.66 | 14.08 | 139.57 | 335.04 |

## Table F.6 Experimental Data from High Pressure Slurry Experiments (continued)

| 0 | 4.90 | 2.32 | 35.76 | 378.00 | 166.76 |
| --- | --- | --- | --- | --- | --- |
| 10 | 4.49 | 2.63 | 35.76 | 346.43 | 188.84 |
| 20 | 3.59 | 3.38 | 35.76 | 277.48 | 242.90 |
| 40 | 3.02 | 4.14 | 35.76 | 233.45 | 297.73 |
| 70 | 2.13 | 4.90 | 35.76 | 164.49 | 351.79 |
| 100 | 1.39 | 5.78 | 35.76 | 107.17 | 414.99 |
| 0 | 3.97 | 2.53 | 35.76 | 306.56 | 181.99 |
| 10 | 3.48 | 2.75 | 35.76 | 268.34 | 197.22 |
| 20 | 2.88 | 3.28 | 35.76 | 222.65 | 235.29 |
| 40 | 2.43 | 3.83 | 35.76 | 187.76 | 274.89 |
| 70 | 1.82 | 4.57 | 35.76 | 140.40 | 328.19 |
| 100 | 1.27 | 4.82 | 35.76 | 98.03 | 346.46 |
| 0 | 7.17 | 1.96 | 35.76 | 553.30 | 140.87 |
| 10 | 6.80 | 2.20 | 35.76 | 525.05 | 158.38 |
| 20 | 6.26 | 2.78 | 35.76 | 483.51 | 199.50 |
| 30 | 5.91 | 3.02 | 35.76 | 456.10 | 217.01 |
| 50 | 5.68 | 3.29 | 35.76 | 438.65 | 236.05 |
| 70 | 5.43 | 3.35 | 35.76 | 419.54 | 240.62 |
| 100 | 5.15 | 3.71 | 35.76 | 397.94 | 266.51 |
| 140 | 4.98 | 3.92 | 35.76 | 384.65 | 281.74 |
| 180 | 4.82 | 4.22 | 35.76 | 372.19 | 303.06 |
| 220 | 4.48 | 4.58 | 35.76 | 345.60 | 328.95 |
| 0 | 4.29 | 2.06 | 35.76 | 331.48 | 147.72 |
| 10 | 3.94 | 2.28 | 35.76 | 304.06 | 163.71 |
| 20 | 3.40 | 2.84 | 35.76 | 262.53 | 204.07 |
| 40 | 2.75 | 3.50 | 35.76 | 212.68 | 251.28 |
| 70 | 2.13 | 4.08 | 35.76 | 164.49 | 293.16 |
| 100 | 1.56 | 4.93 | 35.76 | 120.46 | 354.08 |
| 0 | 2.12 | 1.35 | 35.76 | 163.66 | 96.70 |
| 10 | 1.87 | 1.53 | 35.76 | 144.56 | 109.65 |
| 20 | 1.62 | 1.98 | 35.76 | 125.45 | 142.39 |
| 30 | 1.40 | 2.04 | 35.76 | 108.00 | 146.20 |
| 50 | 1.13 | 2.13 | 35.76 | 87.23 | 153.05 |
| 70 | 1.04 | 2.35 | 35.76 | 80.59 | 169.04 |
| 100 | 0.52 | 2.55 | 35.76 | 39.88 | 183.51 |

Table F.6 Experimental Data from High Pressure Slurry Experiments (continued)

| 0 | 5.43 | 1.89 | 64.90 | 419.54 | 135.54 |
| --- | --- | --- | --- | --- | --- |
| 10 | 4.91 | 2.33 | 64.90 | 378.83 | 167.52 |
| 20 | 4.18 | 3.14 | 64.90 | 322.34 | 225.39 |
| 30 | 3.76 | 3.60 | 64.90 | 289.94 | 258.89 |
| 50 | 3.28 | 3.90 | 64.90 | 253.39 | 280.22 |
| 70 | 2.94 | 4.53 | 64.90 | 226.80 | 325.14 |
| 90 | 2.71 | 4.62 | 64.90 | 209.36 | 331.99 |
| 120 | 2.52 | 4.71 | 64.90 | 194.40 | 338.09 |
| 150 | 2.39 | 5.02 | 64.90 | 184.43 | 360.93 |
| 0 | 4.30 | 2.00 | 64.90 | 332.31 | 143.91 |
| 10 | 4.01 | 2.55 | 64.90 | 309.88 | 183.51 |
| 20 | 3.33 | 3.02 | 64.90 | 256.71 | 217.01 |
| 30 | 3.16 | 3.24 | 64.90 | 244.25 | 233.01 |
| 50 | 2.98 | 3.45 | 64.90 | 230.13 | 247.47 |
| 70 | 2.65 | 3.68 | 64.90 | 204.37 | 264.22 |
| 90 | 2.49 | 3.99 | 64.90 | 191.91 | 286.31 |
| 120 | 2.31 | 4.19 | 64.90 | 178.62 | 300.77 |
| 150 | 2.01 | 4.39 | 64.90 | 155.36 | 315.24 |
| 0 | 7.56 | 1.83 | 64.90 | 584.04 | 131.73 |
| 10 | 7.41 | 2.04 | 64.90 | 572.41 | 146.20 |
| 20 | 7.05 | 2.51 | 64.90 | 544.16 | 180.46 |
| 30 | 6.84 | 2.58 | 64.90 | 528.37 | 185.03 |
| 70 | 6.49 | 2.95 | 64.90 | 500.96 | 211.68 |
| 100 | 6.38 | 3.13 | 64.90 | 492.65 | 224.63 |
| 140 | 6.04 | 3.55 | 64.90 | 466.07 | 255.09 |
| 180 | 5.89 | 3.61 | 64.90 | 454.44 | 259.66 |
| 0 | 1.90 | 2.08 | 64.90 | 147.05 | 149.25 |
| 5 | 1.49 | 2.34 | 64.90 | 114.65 | 168.28 |
| 10 | 1.18 | 2.62 | 64.90 | 91.39 | 188.08 |
| 20 | 1.04 | 2.81 | 64.90 | 80.59 | 201.79 |
| 50 | 0.81 | 3.07 | 64.90 | 62.31 | 220.82 |
| 70 | 0.56 | 2.97 | 64.90 | 43.20 | 213.21 |
| 100 | 0.50 | 3.10 | 64.90 | 38.22 | 222.34 |

Table F.6 Experimental Data from High Pressure Slurry Experiments (continued)

| 0 | 3.71 | 2.16 | 86.80 | 286.62 | 155.34 |
| --- | --- | --- | --- | --- | --- |
| 5 | 3.51 | 2.51 | 86.80 | 270.83 | 180.46 |
| 10 | 3.31 | 2.68 | 86.80 | 255.88 | 192.65 |
| 20 | 3.26 | 3.01 | 86.80 | 251.73 | 216.25 |
| 50 | 2.82 | 3.37 | 86.80 | 217.66 | 242.14 |
| 70 | 2.74 | 3.34 | 86.80 | 211.85 | 239.86 |
| 100 | 2.63 | 3.53 | 86.80 | 202.71 | 253.56 |
| 140 | 2.34 | 3.81 | 86.80 | 180.28 | 273.36 |
| 180 | 2.18 | 4.07 | 86.80 | 168.65 | 292.40 |

Table F.7 Unsteady State Performance Comparison under Gas and Liquid Limited Conditions

| Pressure (psig) | Feed Concentration (mol/m3) | Flow Rate, Qmean (ml/min) | Space time (s) | Cycle Time (s) | Cycle Split | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| 30 | 1484 | 4.5 | 1199 | Steady | Steady | 0.2548 |
| 30 | 1484 | 7.0 | 771 | Steady | Steady | 0.23 |
| 30 | 1484 | 12.0 | 449 | Steady | Steady | 0.1613 |
| 30 | 1484 | 3.6 | 1499 | 60 | 0.5 | 0.4365 |
| 30 | 1484 | 7.0 | 771.08 | 60 | 0.5 | 0.2857 |
| 60 | 690 | 3.2 | 1686.7 | Steady | Steady | 0.755 |
| 60 | 690 | 7.0 | 771.08 | Steady | Steady | 0.703 |
| 60 | 690 | 12.5 | 431.80 | Steady | Steady | 0.473 |
| 60 | 690 | 19 | 284.08 | Steady | Steady | 0.3045 |
| 60 | 690 | 2.1 | 2570.2 | 60 | 0.5 | 0.79 |
| 60 | 690 | 4.5 | 1199.4 | 60 | 0.5 | 0.621 |
| 60 | 690 | 10.0 | 539.7 | 60 | 0.5 | 0.4205 |
| 60 | 690 | 12.0 | 449.8 | 60 | 0.5 | 0.38 |

Table F.8 Effect of Cycle Split on Performance under Gas Limited Conditions

| Pressure (psig) | Feed Concentration (mol/m3) | Flow Rate, Qmean (ml/min) | Space time (s) | Cycle Time (s) | Cycle Split | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| 30 | 1484 | 7.0 | 771.0 | Steady | Steady | 0.23 |
| 30 | 1484 | 7.0 | 771.0 | 60 | 0.5 | 0.2936 |
| 30 | 1484 | 7.0 | 771.0 | 60 | 0.33 | 0.3422 |
| 30 | 1484 | 7.0 | 771.0 | 60 | 0.25 | 0.3577 |

Table F. 9 Total Cycle Period / Liquid Mass Velocity Effect (Liquid Solid Contacting)

| Pressure (psig) | Feed Concentration (mol/m3) | Flow Rate, Qmean (ml/min) | Space time (s) | Cycle Time (s) | Cycle Split | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| 30 | 1553 | 7.0 | 771.0 | Steady | Steady | 0.2059 |
| 30 | 1553 | 7.0 | 771.0 | 18 | 0.33 | 0.2188 |
| 30 | 1553 | 7.0 | 771.0 | 30 | 0.33 | 0.2605 |
| 30 | 1553 | 7.0 | 771.0 | 60 | 0.33 | 0.2922 |
| 30 | 1553 | 7.0 | 771.0 | 120 | 0.33 | 0.2334 |
| 30 | 1579 | 4.0 | 1344.21 | Steady | Steady | 0.2426 |
| 30 | 1579 | 4.0 | 1344.21 | 20 | 0.25 | 0.2609 |
| 30 | 1579 | 4.0 | 1344.21 | 40 | 0.25 | 0.3059 |
| 30 | 1579 | 4.0 | 1344.21 | 60 | 0.25 | 0.28419 |
| 30 | 1579 | 4.0 | 1344.21 | 160 | 0.25 | 0.2355 |
| 30 | 1579 | 4.0 | 1344.21 | 320 | 0.25 | 0.1637 |
| 30 | 1579 | 2.0 | 2688.4 | Steady | Steady | 0.2858 |
| 30 | 1579 | 2.0 | 2688.4 | 20 | 0.25 | 0.3933 |
| 30 | 1579 | 2.0 | 2688.4 | 48 | 0.25 | 0.4446 |
| 30 | 1579 | 2.0 | 2688.4 | 120 | 0.25 | 0.4367 |
| 30 | 1579 | 2.0 | 2688.4 | 240 | 0.25 | 0.3996 |

Table F.10 Effect of Cycling Frequency on Performance under Gas Limited Conditions

| Pressure (psig) | Feed Concentration (mol/m3) | Flow Rate, Qmean (ml/min) | Space time (s) | Cycle Time (s) | Cycle Split | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| 30 | 1494 | 3.0 | 1792.2 | Steady | Steady | 0.3077 |
| 30 | 1494 | 3.0 | 1792.2 | 6.0 (0.166 Hz) | 0.5 | 0.3293 |
| 30 | 1494 | 3.0 | 1792.2 | 12.0 (0.0833 Hz) | 0.5 | 0.3815 |
| 30 | 1494 | 3.0 | 1792.2 | 24.0 (0.0416 Hz) | 0.5 | 0.3954 |
| 30 | 1494 | 3.0 | 1792.2 | 48.0 (0.0208 Hz) | 0.5 | 0.4123 |
| 30 | 1494 | 3.0 | 1792.2 | 96.0 (0.0104 Hz) | 0.5 | 0.4342 |
| 30 | 1494 | 3.0 | 1792.2 | 192.0 (0.0052 Hz) | 0.5 | 0.3815 |

Table F. 11 Effect of Pressure (Transition from Gas Liquid to Liquid Limited Conditions)

| Pressure (psig) | Feed Concentration (mol/m3) | Flow Rate, Qmean (ml/min) | Space time (s) | Cycle Time (s) | Cycle Split | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| 30 | 1545 | 2.5 | 2150.7 | Steady | Steady | 0.4065 |
| 30 | 1545 | 2.5 | 2150.7 | 60 | 0.33 | 0.4626 |
| 60 | 1545 | 2.5 | 2150.7 | Steady | Steady | 0.51412 |
| 60 | 1545 | 2.5 | 2150.7 | 60 | 0.33 | 0.6067 |
| 120 | 1545 | 2.5 | 2150.7 | Steady | Steady | 0.6109 |
| 120 | 1545 | 2.5 | 2150.7 | 60 | 0.33 | 0.7167 |
| 180 | 1545 | 2.5 | 2150.7 | Steady | Steady | 0.6721 |
| 180 | 1545 | 2.5 | 2150.7 | 60 | 0.33 | 0.7469 |

Table F.12 Effect of Cycle Time and Split at Higher Feed Concentration (~ 30 % v/v AMS)

| Pressure (psig) | Feed Concentration (mol/m3) | Flow Rate, Qmean (ml/min) | Space time (s) | Cycle Time (s) | Cycle Split | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| 30 | 1484.0 | 7.0 | 771.0 | Steady | Steady | 0.23 |
| 30 | 1484.0 | 7.0 | 771.0 | 60 | 0.5 | 0.293 |
| 30 | 1484.0 | 7.0 | 771.0 | 60 | 0.33 | 0.3422 |
| 30 | 1484.0 | 7.0 | 771.0 | 60 | 0.25 | 0.3577 |
| 30 | 2082.0 | 3.0 | 1799.2 | Steady | Steady | 0.239 |
| 30 | 2082.0 | 3.0 | 1799.2 | 60 | 0.5 | 0.2634 |
| 30 | 2082.0 | 3.0 | 1799.2 | 60 | 0.25 | 0.2804 |
| 30 | 2082.0 | 3.0 | 1799.2 | 60 | 0.1 | 0.2874 |

Table F.13 Effect of Cycling frequency at Higher Feed Concentration (~ 30 % v/v AMS)

| Pressure (psig) | Feed Concentration (mol/m3) | Flow Rate (ml/min) Qmean | Space time (s) | Cycle Time (s) | Cycle Split | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| 60 | 2084 | 3.0 | 1792.2 | Steady | Steady | 0.23 |
| 60 | 2084 | 3.0 | 1792.2 | 6.0 (0.166 Hz) | 0.5 | 0.2408 |
| 60 | 2084 | 3.0 | 1792.2 | 12.0 (0.0833 Hz) | 0.5 | 0.2635 |
| 60 | 2084 | 3.0 | 1792.2 | 24.0 (0.0416 Hz) | 0.5 | 0.2726 |
| 60 | 2084 | 3.0 | 1792.2 | 48.0 (0.0208 Hz) | 0.5 | 0.2844 |
| 60 | 2084 | 3.0 | 1792.2 | 96.0 (0.0104 Hz) | 0.5 | 0.2647 |
| 60 | 2084 | 3.0 | 1792.2 | Steady | Steady | 0.23 |
| 60 | 2084 | 3.0 | 1792.2 | 10.0 (0.1 Hz) | 0.2 | 0.2483 |
| 60 | 2084 | 3.0 | 1792.2 | 30.0 (0.033 Hz) | 0.2 | 0.2546 |
| 60 | 2084 | 3.0 | 1792.2 | 90.0 (0.0111 Hz) | 0.2 | 0.2698 |
| 60 | 2084 | 3.0 | 1792.2 | 180.0 (0.0055 Hz) | 0.2 | 0.2562 |

Table F. 14 Base Flow Pulsing under Liquid Limited Conditions (High Pressure and Low Feed Concentration) (~ 5 % v/v feed concentration)

| Pressure (psig) | Feed Concentration (mol/m3) | Flow Rate, Qmean (Qbase) (ml/min) | Space time (s) | Cycle Time (s) | Cycle Split | Conversion |
| --- | --- | --- | --- | --- | --- | --- |
| 150 | 784 | 5.0 | 2148.6 | Steady | Steady | 0.7881 |
| 150 | 784 | 5.0 | 2148.6 | 30 | 0.1 | 0.8595 |
| 150 | 784 | 5.0 | 2148.6 | 50 | 0.1 | 0.871 |
| 150 | 784 | 5.0 | 2148.6 | 100 | 0.1 | 0.877 |
| 150 | 784 | 5.0 | 2148.6 | 200 | 0.1 | 0.8812 |

# Appendix G. Simulation of Flow using CFDLIB

- Problem Definition

Predicting the fluid dynamics of trickle bed reactors is important for their proper scale-up to industrial scale. Previous studies have resorted primarily to prediction of overall phase holdup and pressure drop based on an empirical or phenomenological approach. Recent advances in simulation of multiphase flow and development of robust codes that can handle two and three dimensional simulations have made flow simulation feasible in complex flows such as those observed in trickle beds. CREL has access to the CFDLIB multiphase flow codes developed by the Los Alamos National Laboratory which are used in this study.

- Research Objectives

The objective of this study is to simulate the effect of single point and multi-point inlet flow distribution on the flow distribution inside the reactor and to expand the test database beyond the preliminary simulations presented by Kumar (1995,1996). This involves modification of conventional drag and interfacial exchange terms implemented in CFDLIB using drag formulations developed at CREL as well as those available in trickle bed literature. Another objective is to study the influence of surface tension on the spreading of liquid in single and multi-point inlet conditions. This will serve as a benchmark for comparison with experimental velocities and phase holdup data which have not yet been reported in the open literature.

- Research Accomplishments

C1. Modeling Interphase Exchange and Interfacial Tension Terms

The underlying equations for the CFDLIB code have been discussed in detail in earlier reports by Kumar (1996) and can be found in Kashiwa et al. (1994). The special case of one fixed phase (the catalyst bed) has also been incorporated in the code for single and two phase flow simulation. The important terms in simulating trickle bed reactors is the drag equation and the influence of phasic pressure difference due to interfacial tension. Phenomenological models developed at CREL by Holub (1990) are incorporated in simulating the drag between the stationary solid phase and each of the flowing phases. The code models the drag force in terms of phase fractions and relative velocity given for any combination of phases k and l as

FD(k−l)=qkqlXkl(uk−ul)							(1)

where the Xkl is modeled by the modified Ergun equation (Holub, 1990, Saez and Carbonell, 1985) with Ergun constants either determined by single phase experiments or using universal values.

X(l−S)=(1−qS)qL3E1ReLGaL+E2ReL2GaLρLg|uLS|(1−qS)					(2)

X(G−S)=(1−qS)qG3E1ReGGaG+E2ReG2GaGρGg|uGS|(1−qS)				(3)

For gas-liquid drag either no interaction is assumed or interaction based on a drag coefficient is used as

X(G−L)=0.75ρLCD|uGL|qLdp								(4)

For modeling interfacial tension the famous Leverett’s J function (Dankworth et al., 1990) is used to yield the difference between the gas and liquid pressure calculated in the code in terms of the interfacial tension ( ), bed permeability (k) and phase fractions as

pL=pG−1−qSk12s0.48+0.036.ln(1−qS−qLqL)				(5)

The bed permeability (k) is related to Erguns constant E1 and equivalent particle diameter (de) as

((1−qS)k)12=(qS)E1(1−qS)de								(6)

The simulations are conducted both by discounting and incorporating the above equation and results presented in the next section.

C2. Simulation of Test Cases: Results and Discussion

Case I: (Reactor Dimensions 60x11 cm, B=0.4, dp= 1.5 mm,

UsL=0.036 cm/s, UsG=3.63 cm/s)

- Point Source Inlet with Uniform Bed Porosity

- Point Source Inlet: Including Surface Tension Effects

For the test case of point source liquid inlet when the surface tension forces are ignored, Figure 1 shows that the liquid stays mainly in the central core without much spreading and with almost negligible wall flow. For the case where interfacial tension was included and modeled using a capillary pressure given by the Leverett Function, significant liquid spreading was observed at a depth of over 10 cm from the inlet. Figures 2 and 3 show the contours of the liquid holdup and development of the velocity and holdup profile down the reactor to a constant and fairly uniform flow profile at the reactor exit. In this case some wall flow is indeed observed after a depth of 28 cm as shown in Figure 2. This is corroborated by visual and photography experiments done at CREL on a 2D bed (Jiang, 1997) and by the tomography results of Lutran et al.(1991).

Case II: Uniform Liquid and Gas Inlet : No Surface Tension Effects, Uniform Porosity everywhere except at the wall and exit section (Reactor Dimensions 28.8x7.2 cm, B=0.4 in the core and B=0.5 at the wall, dp= 3 mm, UsL=0.1 cm/s, UsG=5.0 cm/s)

- No Gas Flow (Unsaturated Liquid Flow)

- Low Gas Flow (Low Interaction)

- High Gas Flow (Moderate Interaction)

For the uniform distribution tests, it was assumed that the wall zone extends three particle diameters from the wall and the exit of the reactor for which a higher porosity was assigned, and to the rest of the central core a uniform lower porosity was assigned. It is much more difficult to discern the flow distribution profiles in this case as compared to Case I, primarily due to local non-uniformity in the phase holdups and velocity. The only obvious effect is that of higher wall flow of gas at the wall as compared to the central core as depicted in Figure 4 for the moderate gas flow rate (case IIc). For the low and zero gas flow rate such clear gas wall flow was not observed. Larger reactor size tests for this case are underway and results will be reported in future reports.

Case III: Multipoint Inlet of Liquid (Including Surface Tension Effects) (Reactor Dimensions 60x23 cm, B=0.4, dp= 1.5 mm, UsL=0.052 cm/s, UsG=3.47 cm/s)

This case is shown to illustrate conditions similar to industrial distributors with multiple points of a large distributor each of which is similar to that in Case I. The holdup contour plot shown in Figure 5 shows underutilization of the top part of the bed indicating the required pitch between distributor inlet points should be smaller than 5 cm that was used here to achieve better distribution. This mal-distribution may result in zones where hotspots may develop such as in the gas phase adjacent to the boundary of the gas-liquid zone (where both gas and liquid reactants are abundantly supplied to the catalyst for the case of non-volatile liquid reactant) and in the gas rich zones in the case of volatile liquid reactant between the well irrigated zones. Such simulations can yield direct information as to the extent and consequence of maldistribution due to well separated multi-point inlets typical in industrial reactors.

- Future Work

The potential for CFDLIB to predict flow distribution in trickle bed reactors is shown in this report for several test cases. Further work will encompass simulation of more complex reactor geometry and flow situations and extension of flow simulations from cold-flow modeling to reactive cases where accurate prediction of product distribution and hot-spot formation is crucial to optimal and safe operation of pilot and large scale reactors.

- Nomenclature

CD	= Drag Coefficient

dp	= Particle Diameter

de	= Particle Equivalent Diameter

E1,E2	= Erguns Constants

FD(kl)	= Drag Force between Phases k and l.

g	= Gravitational Acceleration

Ga	= Phase Galileo Number

k	= Bed Permeability

p	= Phase Pressure

Re	= Phase Reynolds Number

uk	= Interstitial Velocity of Phase k.

Xkl	= Interphase Exchange Coefficient between phases k and l.

Greek Symbols

k      	= Phase Fraction of Phase k.

	= Phase Density

- = Interfacial Tension

- References

- Dankworth, D. C., Kevrekidis, I.G., and Sundaresan, S., Time Dependent Hydrodynamics in Multiphase Reactors, Chem. Eng. Sci., Vol. 45, No. 8, pp. 2239-2246 (1990).

- Holub, R. A., Hydrodynamics of Trickle Bed Reactors. Ph.D. Thesis, Washington University in St. Louis, MO (1990).

- Jiang. Y., Unpublished Results on 2-D Trickle Bed Reactors using Point and Uniform Liquid Inlet Distributors (1997).

- Kashiwa, B. A., Padial, N. T., Rauenzahn, R. M. and W. B. VanderHeyden, A Cell centered ICE Method for Multiphase Flow Simulations, ASME Symposium on Numerical Methods for Multiphase Flows, Lake Tahoe, Nevada (1994)

- Kumar, S. B., Simulation of Multiphase Flow Systems using CFDLIB code CREL Annual Meeting Workshop (1995).

- Kumar, S. B., Numerical Simulation of Flow in Bubble Columns, CREL Annual Report (1996).

- Lutran, P. G., Ng, K. M. and Delikat, E. P., Liquid distribution in trickle beds: An experimental study using Computer-Assisted Tomography. Ind. Engng. Chem. Res. 30 (1991)

- Saez, A. G. and Carbonell, R. G., Hydrodynamic Parameters for Gas-Liquid Cocurrent Flow in Packed Beds, AIChE J. 31, 52 (1985)

Figure 1: Liquid Holdup Contours for Single Point Source Inlet (Case Ia)

Figure 2: Liquid Holdup Contours for Single Point Source Inlet Including Interfacial Tension Effects (Case Ib)

Figure 3: Liquid Holdup and Velocity Profiles for Single Point Source Liquid Inlet (with Interfacial Tension Effects)(Case Ib)

Figure 4: Gas Holdup and Flow Profiles for Case IIc (Moderate Gas Flow)

Figure 5: Liquid Holdup and Velocity for Multipoint Inlet Distributor (Case III).

# Appendix H. Improved Prediction of Pressure Drop in High Pressure Trickle Bed Reactors

A. Problem Definition

Trickle-bed reactors are fixed beds of catalyst particles contacted by downflow of gas and liquid. They are used widely in petroleum, petrochemical, and chemical industry and are usually operated at high pressure (20-30 MPa). A basic understanding of the hydrodynamics of trickle bed reactors is essential to their design, scale-up, scale-down, and performance. Pressure drop and liquid holdup are important operational parameters. Several correlations have been proposed in recent years to predict pressure drop and holdup at low and high pressure conditions (as listed in Table 1). The Holub et al. (1993) phenomenological model predicts pressure drop and liquid holdup in trickle flow regime better than the available correlations at atmospheric pressure. Al-Dahhan and Dudukovic (1994) noted that although it also predicts pressure drop and liquid holdup better than all the  reported high pressure correlations (Table 1), it underpredicts them at high pressure and high gas flow rates within trickle flow regime. This is because the interactions between phases increase at high pressure and high gas flow rate which are not currently accounted for in the present simplest form of Holub et al. model.

The Holub et al.  (1993) model has been extended to account for the interaction between the gas and liquid phases by incorporating the velocity and the shear slip factors between the phases.  It is necessary to examine the extended model predictions and the nature and numerical values of the correction factors (shear slip factor, fs and velocity slip factor, fv) and their contribution to the prediction of pressure drop and holdup.

- Research Objectives

The objective of this study is to examine the prediction of previous pressure drop and holdup prediction correlations and Holub’s (1992) simple and extended model for high pressure data. Based on the predictions for high pressure, develop correlations for the shear and velocity slip factors (fs and fv, respectively) and estimate pressure drop and holdup predictions at moderate interaction conditions encompassing high pressure and high gas flow rates.

C. Research Accomplishments

Holub et al. (1993), (1992) proposed a phenomenological model based on representation of the complex geometry of the actual void space in a packed bed of particles at the pore level by a single flat walled slit of average half width. The developed model is a modified Ergun equation that ties together the pressure drop and holdup in trickle flow regime (Table 1).

E1 and E2 are the Ergun constants that characterize the bed. They are determined from single phase (gas) flow through the packing of interest (dry bed). Hence, this simplest form does not contain any parameters which need to be fitted to two phase flow data and neglects the interaction between gas and liquid phases. As a result the model predicts pressure drop and liquid holdup better than the available correlations only in the region of low interaction between the phases, at atmospheric pressure and at low flow rates (Holub et al., (1993,1992); Holub, (1990)). Al-Dahhan and Dudukovic (1994) noted that although the model  also predicts pressure drop and liquid holdup better than the recently reported high pressure correlations (Larachi et al., (1991); Wammes et al., (1991); Ellman et al., (1988, 1990)) (Table 1), it systematically underpredicts them at high pressure and high gas flow rates. Under these conditions, the relative error in pressure drop prediction (~-48%) is more noticeable compared to that of holdup prediction (~-9%). This is because the interaction between phases increases at high pressure and high gas flow rate which were not accounted for in the original form of the Holub's model .

The degree of interaction between the gas and liquid phases can be accounted for by incorporating the velocity and shear slip factors between the phases. Holub (1990) and Holub et al. (1993) derived a detailed model based on the two phase flow momentum balance for the slit, which incorporates the velocity slip factor (fv) and the shear slip factor (fs) as shown below.

C1. The Extended Model

The degree of interaction between the gas and liquid phases accounted for by

1. Velocity slip factor (fv)

ViG=fv ViL

2. Shear slip factor (fs)

tiL=fs tiG

Equations based on the two phase flow momentum balance for the slit

YG=eBeB−eL3E1(ReG−fveGRei)GaG+E2(ReG−fveGRei)2GaG

YL=eBeL3E1ReLGaL+E2ReL2GaL+fseGeL(1−ρGρL−YL)

where

hL=15(E1)0.25YLGaLeLeB3(1+fseGeLρGρLYGYL)

and

Physical Interpretation of  fs and fv

| fs | fv |  |
| --- | --- | --- |
| 0 | 0 | No Interaction |
| 1 | 1 | Continuity of profiles between phases |
| <0 | <0 | Circulation |

Case 1: 				Case 2:

fs = 0.0

fv = 0.0

fs and fv characterize the degree of phase interaction at the gas-liquid interface. Hence, when fs=fv=0 (i.e., no interaction occurs), the extended model simplifies to the Holub et al. model shown in Table 1. The rationale behind assuming fs=fv=0 is that for atmospheric pressure data, Holub et al. (1993) have shown that fv and fs can both be zero (no interaction) with only a small increase in error over the observed minimum error. However, this is not the case when high interaction between the phases occurs at high pressure and high gas flow rate in the trickle flow regime (Al-Dahhan and Dudukovic, (1994)). Accordingly, the model represented by the extended model equations is suggested as a two phase flow form of the Ergun equation containing the two phase interaction parameters, fs and fv, which must be determined from two phase flow experimental data. Ergun's constants E1 and E2, characterize the bed, and are still determined from single (gas) phase flow experiments. The last equation in the model is an implicit equation in liquid holdup formed by equating the dimensional pressure gradient in the gas and liquid phase and is solved for liquid holdup from which pressure drop is then evaluated.

C2. Results

Experimental data of Al-Dahhan (1993) and Al-Dahhan and Dudukovic (1994) that cover low to high pressure and gas flow rates are used to evaluate fv and fs using the extended model. Due to the limited number of data points available (see Table 2), it was not possible to observe a strong discernible dependence of fs and fv with either ReL or ReG. Therefore, correlations for fs and fv are developed by fitting the values of fs and fv based on minimizing the pressure drop error. The correlation can  be used to predict fs and fv as the two phase flow parameters in the model equations. As a result, the prediction of pressure drop improves significantly compared to the simplified model as shown in Figure (1)(relative error decreased from 48 % to 20%). Liquid holdup prediction remains within the same range of predictability as that by Holub's simplified model as shown in Figure (2)(relative error is about 9% for both simple and extended model). This reveals that pressure drop is more affected by the interaction between phases compared to liquid holdup.

D. Conclusions and Future work

This study demonstrates that shear or velocity based correction factors are necessary for accurate predictions of pressure drop and holdup particularly in the moderate  interaction range within the trickle flow regime. It is noteworthy to mention that a large bank of high pressure and gas flow rate data is needed to develop sound correlations for the prediction of fs and fv which is not available at present. Moreover, high pressure data in the literature cannot be used directly since E1 and E2 were not reported and these parameters can only be obtained from single phase flow experiments. More work on correlation of fs and fv using data at moderate phase interaction is recommended in order to understand their dependence on flow variables.

E. Nomenclature

Dp	= Equivalent spherical diameter of packing particle

E1,E2	= Ergun equation constants for single phase flow

f	= Phase interaction parameters

g	= Gravitational acceleration

Gaa	= Galileo number (g Dp3 eB3/na2(1-eB)2)

Rea	= Reynolds number of a phase (VaDp/na(1-eB))

Va	= Superficial velocity of a phase

Greek symbols

eB	= Bed Porosity

ea	= Bed holdup of a phase

ha	= Pseudo bed Reynolds number based on a phase

ma	= Viscosity of a phase

na	= Kinematic viscosity of a phase

ra	= Density of the a phase

Ya	= Dimensionless body force on the a phase

Subscripts

a	= General subscript meaning gas (G) or liquid (L) phase

G	= Gas phase

L	= Liquid phase

s	= Shear

v	= Velocity

F. Bibliography

- Al-Dahhan, M. H., "Effects of High Pressure and Fines on the Hydrodynamics of Trickle-Bed Reactors" DSc thesis, Washington University, St. Louis (1993).

- Al-Dahhan, M. H., and M. P. Dudukovic, Chem. Engng. Sci., 49 (24B) (1994).

- Ellman, M. J., Midoux, N., Laurent, A., and J. C. Charpentier, Chem. Engng. Sci., 43, 2201 (1988).

- Ellman, M. J., Midoux, N., Wild, G., Laurent, A., and J. C. Charpentier, Chem. Engng. Sci., 45, 7, 1677(1990).

- Holub, R. A., "Hydrodynamics of Trickle Bed Reactors", DSc Thesis, Washington University, St. Louis (1990).

- Holub, R. A., M. P. Dudukovic, and P. A. Ramachandran, Chem. Engng. Sci., 47, 9/11, 2343 (1992).

- Holub, R. A., Dudukovic, M. P., and P. A. Ramachandran, AIChE J., 39(2) (1993).

- Larachi, F., Laurent, A., Midoux, N., and G. Wild, Chem. Engng. Sci., 46, 5-6, 1233 (1991).

- Wammes, W. J. A., Mechielsen, S. J., and K. R. Westerterp, Chem. Engng. Sci., 46, 409 (1991).

Table 1. Recent Pressure drop correlations

| Correlation | Equations for P | Prediction Error (%) |
| --- | --- | --- |
| Ellman et al. (1988) |  | 65 |
| Larachi et al. (1991) | (DPZ)dhρG2G2=1(ReLWeL)0.25XG1.531.3+17.3(ReLWeL)0.25XG0.5XG=GLρLρGWeL=L2dpρLsL | 89 |
| Wammes et al. (1991) | DP0.5ρGUG2dpZ=155ρGUGdpeBmG(1−eB)−0.371−eBeB(1−βt) | 88 |
| Holub et al. (1992) | YL=DPZρLg+1=eBeL3E1ReLGaL+E2ReL2GaL YG=DPZρGg+1=eBeB−eL3E1ReGGaG+E2ReG2GaG | 40 |

Table 2. Range of Operating Conditions for the Data Used in developing fv and fs Correlations

| System (# of data) | Hexane-Nitrogen -glass beads/ cylinders (63) | Hexane-Helium -glass beads (15) | Water-Nitrogen -glass beads/ cylinders (43) | Water-Helium -cylinders (4) |
| --- | --- | --- | --- | --- |

| Pressure (0.3-3.5 MPa) | Gas Velocity (0.01-0.09 m/s) | Liquid Velocity (0.001-0.005 m/s) |
| --- | --- | --- |

Figure 1. Comparison of Dimensionless Pressure Drop (DP/(rLgZ)) Prediction by Simple and Extended Model and Experimental Data.

Figure 2. Comparison of Liquid Holdup Prediction by Simple and Extended Model and Experimental Data.

# References

Al-Dahhan, M. H., “Effects of High Pressure and Fines on the Hydrodynamics of Trickle Bed Reactors” D.Sc. Dissertation, Washington University, St. Louis, Missouri, (1993).

Al-Dahhan, M. H., Y. Wu, and M. P. Dudukovic, “A Reproducible Technique for Packing Beds with Mixture of Catalyst and Fines”, Ind. & Eng. Chem. Res., 34, 741 (1995).

Al-Dahhan, M. H. and M. P. Dudukovic, ‘Liquid Solid External Contacting in Trickle-Bed Reactors with Fines’, Chem. Eng. Sci. 50, 2377 (1995).

Al-Dahhan, M. H., F. Larachi, M. P. Dudukovic, and A. Laurent, ”High-pressure Trickle-bed Reactors: A Review” Ind. Eng. Chem. Res., 36, 3292 (1997).

Beaudry, E. G., M. P. Dudukovic, and Mills P. L., ‘Trickle Bed Reactors: Liquid Diffusional Effects in a Gas Limited Reaction’,  AIChE J., 33, 1435 (1987).

Berruti, F., R. R. Hudgins, E. Rhodes, and S. Sicardi, ”Oxidation of Sulfur-dioxide in a Trickle Bed Reactor: A Study of Reactor Modeling” Can. J. Chem. Eng. 62, 644 (1984).

Blok, J. R. and A. A. H. Drinkenburg “Hydrodynamics and Mass Transfer in Pulsing Trickle-bed Columns” Chem. Eng. J., 25, 89 (1982).

Brad Sims, W., S. W. Gaskey, and D. Luss, “Effect of Flow Regime and Liquid Velocity on Conversion in a Trickle Bed Reactor” Ind. Eng. Chem. Res. 22, 2530 (1994).

Carruthers, D. J. and D. J. DiCamillo, ‘Pilot plant testing of Hydrotreating Catalyst: Influence of Catalyst Condition, Bed Loading and Bed Dilution’, Applied Catalysis ,43 (1988).

Castellari, A. and P. M. Haure, “Experimental Study of the Periodic Operation of a Trickle-bed Reactor” AIChE J., 41, 1593 (1995).

Chu, C. F. and K. M. Ng, Liquid Dispersion in Trickle Bed Reactors with Gas-Liquid Cocurrent Downflow, Paper 10p, AIChE Annual Meeting, Miami Beach, FL (1986).

Collins, G. M., R. K. Hess, B. D. Hook, and A. Ackgerman, Paper 31p AIChE Annual Meeting, San Fransisco, CA (1984).

Dudukovic, M. P. and P. L. Mills, ‘Contacting and Hydrodynamics of Trickle Bed Reactors’, Encyclopedia of Fluid Mechanics, Gulf Publishing Company, Houston, 969 (1986).

Doraiswamy, L. K. and M. M. Sharma,  Heterogeneous Reactions, Vol-I,  John Wiley and Sons, New York (1984).

Dwivedi, P. N. and Upadhyah, S. N. “Particle-Fluid Mass Transfer in Fixed and Fluidized Beds”, Ind. Eng. Chem. Process Des. Dev., 16, 157 (1977).

Fukushima, S. and K. Kusaka ‘Liquid-Phase Volumetric Mass Transfer Coefficient and Boundary of Hydrodynamic Flow Region in Packed Column with Cocurrent Downward Flow’, J. Chem. Eng. of Japan, 10, 468 (1977).

Funk, G. A., M. P. Harold, and K. M. Ng, “Novel Model for Reaction in Trickle Beds with Flow Maldistribution”, 29, 738 (1990).

Gabarain, L.,  A. T. Castellari, J. Cechini, A. Tobolski, and P. Haure, “Analysis of Rate Enhancement in a Periodically Operated Trickle-bed Reactor”, 43, 166 (1997).

Germain, H., ‘Design of Laboratory Hydrotreating Reactors: Scaling down of Trickle Flow Reactors’, Applied Catalysis, 43, 285, (1988).

Germain, A., M., P. Crine, P.  Marchot, and G. A. L’Homme, “Modeling of a Trickle-Bed Reactor: The Hydrogenation of 2-Butanone on a Ruthenium Catalyst” ACS. Symp. Ser. Int. Symp. On Chem. React. Eng. (1978).

Goto, S., and K. Mabuchi, “Oxidation of Ethanol in Gas-Liquid Cocurrent Up-flow and Down-flow Reactors”, Can. J. Chem. Eng., 62, 865, (1984).

Goto, S., T. Chatani, and M. H. Matouq ‘Hydration of 2-Methyl-2-Butene in Gas-Liquid Cocurrent Up-flow and Down-flow Reactors’, Can. J. Chem. Eng., 71, 821, (1993).

Gregory, A. R., and, N. H. Sweed,”Parametric Pumping: Behavior of Open Systems  Part 1: Analytical Solutions”  Chem. Eng. J., 4, 139 (1972).

Gupta, R., “Pulsed Flow Vapor-Liquid Reactors”, U.S. Patent # 4526757, (1985).

Harold, M. P. and P. C. Watson, P. C., “Bimolecular Exothermic Reaction with Vaporization in the Half-wetted Slab Catalyst” Chem. Eng. Sci., 48, 981 (1993).

Hasokawati, W., R. R. Hudgins, and P. L. Silveston, ‘Loading, Draining, and Hold-up in Periodically operated Trickle bed Reactors’, The Canadian Journal of Chemical Engineering, 72, 405 (1994).

Haure, P.M., S. M. Bogdashev, M. Bunimovich, A. N. Stegasov, R. R. Hudgins, and  P. L. Silveston, ‘Thermal Waves in Periodic Operation of Trickle Bed Reactor’, Chem. Eng. Sci., 45, 2255 (1990).

Hekmat, D. and Vortmeyer, D. “Modeling of Biodegradation Processes in Trickle Bed Bioreactors” Chem. Eng. Sci. 49, 25 (1994).

Herskowitz, M. “Modeling of a Trickle Bed Reactor: The Hydrogenation of Xylose to Xylitol”, Chem. Eng. Sci. 40, 1309 (1985)

Herskowitz, M., R. G. Carbonnel, and J. M. Smith, “Effectiveness Factor and Mass Transfer in Trickle-bed Reactors” AIChE J.  25, 272 (1978).

Holub, R. A., ‘Hydrodynamics of Trickle Bed Reactors’, D.Sc. Dissertation, Washington University, St. Louis, Missouri (1990).

Jackson, R., Transport in Porous Catalysts, Elsevier Scientific, New York (1977).

Kheshgi, H. S., S. C. Reyes, R. Hu, and T. C. Ho, “Phase Transition and Steady State Multiplicity in a Trickle-bed Reactor” Chem. Eng. Sci., 47, 1771 (1992).

Korsten, H. and U. Hoffmann, “Three Phase Reactor Model for Hydrotreating in Pilot Trickle-bed Reactors” AIChE J., 42, 1350 (1996).

Krishna, R., ‘Problems and Pitfalls in the use of Fick Formulation for Intraparticle Diffusion’, Chem. Eng. Sci., 48, 845 (1993).

LaVopa, V., and C. N. Satterfield, “Some Effects of Vapor-liquid Equilibriaa on Performance of a Trickle-bed Reactor” Chem. Eng. Sci. 43, 2175 (1988).

Lakota, A. and J. Levec ‘Solid-Liquid Mass Transfer and Wetting Efficiency in Packed Beds with Cocurrent Downwards Two-phase Flow’, AIChE J., (1990).

Lapidus, L. and R. Luss, Optimal Control of Engineering Processes, Blaisdell Publishing Company, Waltham, Mass.,(1967).

Lange, R., J. Hanika, D. Stradiotto, R. R. Hudgins, and P. L. Silveston, ‘Investigations of Periodically operated Trickle-bed Reactors’, Chem. Eng. Sci.,49, 5615 (1996).

Lara Marquez, A., F. Larachi, G. Wild, and A. Laurent ‘Mass Transfer Characteristics of Fixed Beds with Cocurrent Up-flow and Down-flow. A Special Reference to Effect of Pressure’ Chem. Eng. Sci., 47, 3485 (1992).

Larachi, F., A. Laurent, G. Wild, and N. Midoux, ‘Some Experimental Liquid Saturation Results in Fixed bed Reactors Operated Under Elevated Pressure in Cocurrent Up-flow and Down-flow of  Gas and Liquid’, Ind. Eng. Chem. Res., 30, 2404, (1991).

Laximinarsimhan, C. S., R. P. Verma, and P.A. Ramachandran, “Continuous Lumping Model for Simulation of Hydrocracking” AIChE J., 42, 2645 (1996).

Lekshmi-Narayanan, H., C. Pan, P. Haure, and R. R. Hudgins, “Model for Trickle Bed Reactors in View of Changing Gas Velocity”, 68, 327 (1990).

Lee, C. K., and J. E. Bailey, “Diffusion Waves and Selectivity Modifications in Cyclic Operation of a Porous Catalyst” 1974, Chem. Engng. Sci., 29, 1157-1163.

Lee, J. K., R. R. Hudgins, and P. L. Silveston, “Cycled Trickle Bed Reactor for SO2 Oxidation”, Chem. Eng. Sci., 50, 2523 (1995).

Lemcoff, N. O., A. L. Cukierman, and O. M. Martinez, “Effectiveness Factor of Partially Wetted Catalyst Particles: Evaluation and Application to the Modeling of Trickle Bed Reactors” Catal. Rev. Sci. Eng. 30, 393 (1988).

Mazzarino, I., M. Occhetti, G. Baldi, and S. Sicardi, ‘Performance of a Packed Bed reactor with a Two Phase Upward Flow’, Chem. Eng. Comm., 75, 225 (1989).

Mills, P. L. and M. P. Dudukovic, ‘Analysis of Catalyst Effectiveness in Trickle-Bed Reactors Processing Volatile or Nonvolatile Reactants’, Chem. Eng. Sci., 35, 2267 (1980).

Mills, P. L., E. G. Beaudry, and M. P. Dudukovic, ‘Comparison and Prediction of reactor Performance for Packed Beds with Two Phase Flow : Down-flow, Up-flow and Countercurrent Flow’, Inst. Chem. Eng. Symp. Ser., 87, 527 (1984).

Oh, M., and C. C. Pantelides ‘A Modeling and Simulation Language for Combined Lumped and Distributed Parameter Systems’, Computers and Chemical Engineering, (1995).

Park I. H., M. Petkovska, and D. D. Do, “ Frequency Response of an Adsorber with Modulation of the Inlet Molar Flow Rate-I. A Semi-batch Adsorber” Chem. Eng. Sci., 53, 819-832 (1998).

Rajashekharam, M. V., R. Jaganathan, and R. V. Chaudhari “A Trickle-bed Reactor Model for Hydrogenation of 2,4 Dinitrotolune: Experimental Verification”  Chem. Eng. Sci. 53, 787 (1998).

Ramachandran, P.A. and R. V. Chaudhari, R. V., Three Phase Catalytic Reactors, Gordon Breach Publishers, NY, (1983).

Reiss, L. P. “Cocurrent Gas-Liquid Contacting in Packed Columns”, Ind. Eng. Chem. Proc. Des. Dev. 6, 486 (1967).

Ring Z. E. and R. W. Missen “Trickle-bed Reactors: Effect of Wetting Geometry on Overall Effectiveness Factor”, 34th Can. Chem. Eng. Conf. , Quebec. (1984).

Ritter, A. B. and J. M. Douglas, “Frequency Response of Nonlinear Systems”, Ind. Eng. Chem. Fundam., 9, 21 (1970).

Ruzicka, J. and J. Hanika “Partial Wetting and Forced Reaction Mixture Transition in a Model Trickle Bed Reactor” Cat. Today. Proc. (International Symposium on the Dynamics of Catalytic Systems) 20, 467 (1994).

Saroha, A. K. and K. D. P. Nigam, “Trickle Bed Reactors” Reviews in Chemical Engineering, 12, 207 (1996).

Satterfield, C. N., and Roberts, G.W., 1968, AIChE J.  14, 159.

Sei, S. T., ‘Scale Effects in Laboratory and Pilot plant reactors For Trickle-Flow Processes’, Revue De L'Institut Francais Du Petrole, 46, 4 (1991).

Silveston, P., 1990, in Unsteady State Processes in Catalysis, 217-232.

Shah, Y. T., R. D. Mhaskar, and J. A. Paraskos, Ind. Eng. Chem. (Proc. Des. Dev), 15, 400, (1976).

Sicardi, S., “Modeling and Design of a Trickle Bed Reactor”, ISCRE-8 Edinburgh Scotland, (1984).

Stegasov, A.N., V. A. Kirillov, and  P. L. Silveston, ‘Modeling of Catalytic SO2 Oxidation for Continuous and Periodic Liquid Flow Through a Trickle Bed’, Chem. Eng. Sci., 49, 3699 (1994).

Sundmacher, K. and U. Hoffmann, “Multicomponent Mass and Energy Transport on Different Length Scales in a Packed Reactive Distillation Column for Heterogeneously Catalyzed Fuel Ether Production”, Chem. Eng. Sci., 49, 4443 (1994).

Sweed, N. H., and R. H. Wilhelm, “Parametric Pumping”, Ind. Eng. Chem. Fundam., 8, 221 (1969).

Tan, C. S. and J. M. Smith ‘Catalyst particle Effectiveness with Unsymmetrical Boundary Conditions’, Chem. Eng. Sci., 35, 1601 (1980).

Taylor, R., and R. Krishna, Multicomponent Mass Transfer, John Wiley and Sons Inc., New York, (1993).

Toppinen, S., J. Aittamaa, and Salmi, T., “Interfacial Mass Transfer in Trickle-Bed Reactor Modeling” Chem. Eng. Sci.,  51, 4335 (1996).

Tremblay, M. D., and R. Luss, ‘Optimization of Non-Steady-State Operation of Reactors’, Can. J. Chem. Eng., 67, 494, (1989).

Tsamatsoulis, D. and N. Papayannakos, “Simulation of Non-ideal Flow in a Trickle Bed Reactor by a Cross-flow Model”, Chem. Eng. Sci., 50, 3685 (1995).

Tukac, V., I. Mazzarino, G. Baldi, A. Gianetto, S. Sicardi, and V. Specchia ‘Conversion rates in a Laboratory Trickle-bed Reactor during the Oxidation of Ethanol’ Chem. Eng. Sci., 41, 17 (1986).

Valerius, G., X. Zhu, H. Hofmann, D. Arntz, and T. Haas, “Modelling of a Trickle Bed Reactor: II. The Hydrogenation of 3-hydroxy propanal to 1,3-propanediol”, Chem. Eng. Processing., 35, 11 (1996).

Van Klinken, J. and R. H. Van Dongen, ‘Catalyst Dilution for Improved performance of Laboratory Trickle-flow Reactors’, Chem. Eng. Sci., 35, 59 (1980).

Villadsen, J.V. and M. L. Michelsen Solution of Differential Equation Models by Polynomial Approximation, Prentice Hall, Englewood Cliffs, New Jersey (1978).

Warna, J. and T. Salmi, “Dynamic Modeling of Three Phase Reactors”, Compu. And Chem. Eng. 20, 39 (1996).

Weekman, V.W., and J. E. Myers, “Fluid Flow Characteristics of Cocurrent Gas-Liquid Flow in Packed Beds”  AIChE J., 10, 951 (1964).

Wilhelm, R. H., A. W. Rice, and A. R. Bendelius,  Ind. Eng. Chem. Fundam., 5, 141 (1966).

Wu R., M. J. McReady, and A. Varma, A., “Influence of Mass Transfer Coefficient Fluctuation Frequency on Performance of Three-Phase Packed Bed Reactors” Chem. Eng. Sci., 50, 3333 (1995).

Wu, X., and P. C. Wankat, “Continuous multicomponent Parametric Pumping”, Ind. Eng. Chem. Fundam., 22, 172 (1983).

Yan, T. Y., “Dynamics of a Trickle Bed Hydrocracker with a Quenching System” Can. J. Chem. Eng., 58, 259 (1980).

Yang, J. and J. Li, “Simulation of Hydrodewaxing Reactor by Stochastic Method” J. Chem. Eng. Chin. Univ., 6, 264 (1992).

Yamada, H. and S. Goto, “Periodic Operation of Trickle Bed Reactor for Hydrogenolysis in Gas-Liquid-Liquid-Solid Four Phases”, J. Chem. Eng. Jpn., 30, 478 (1997).

# VITA

Publications

“Investigation of a Complex Reaction Network for Production of Amino Alcohol: Part I. Experimental Observations in a Trickle Bed Reactor”, Khadilkar, M. R., Jiang, Y., Al-Dahhan, M., Dudukovic, M. P., Chou, S. K., Ahmed, G., and Kahney, R., Accepted for Publication, AIChE Journal (1997).

“Investigation of a Complex Reaction Network for Production of Amino Alcohol: Part II. Kinetics, Mechanism and Model Based Parameter Estimation”, Jiang, Y., Khadilkar, M. R., Al-Dahhan, M., Dudukovic, M. P., Chou, S. K., Ahmed, G., and Kahney, R., Accepted for Publication, AIChE Journal (1997).

“Prediction of Pressure Drop and Liquid Holdup in High Pressure Trickle Bed Reactors”, Al-Dahhan, M., Khadilkar, M. R., Wu. Y., and Dudukovic, M. P., Accepted for Publication, I&EC Research (1997).

“Comparison of Trickle-Bed and Upflow Reactor Performance at High Pressure: Model Predictions and Experimental Observations”, Khadilkar, M. R., Wu. Y., Al-Dahhan, M., Dudukovic’, M. H. and Colakyan, M., Chem. Eng. Sci., 51, 10, 2139 (1996).

“Simulation of Flow Distribution in Trickle Bed Reactors Using CFDLIB”, Khadilkar, M. R. and Dudukovic, M. P, CREL Annual Report, Washington University (1996-1997)

“Evaluation of Trickle-bed Reactor Models for a Liquid Limited Reaction” Wu, Y., Khadilkar, M. R., Al-Dahhan, M., and Dudukovic, M. P., Chem. Eng.  Sci., 51, 11, 2721 (1996).

“Comparison of Upflow and Downflow Two Phase Flow Reactors With and Without Fines”, Wu. Y., Khadilkar, M. R., Al-Dahhan, M., and Dudukovic, M. P., I&EC Research, 35, 397 (1996).

Presentations

“Simulation of Unsteady State Operation of Trickle Bed Reactors” Khadilkar, M. R., Al-Dahhan,

M., and Dudukovic’, M. P., Presentation 254e, AIChE Annual Meeting, Los Angeles, CA  (1997).

“Investigation of a Complex Reaction Network in a High Pressure Trickle Bed Reactor”, Khadilkar, M. R., Jiang, Y., Al-Dahhan, M., Dudukovic, M. P., Chou, S. K., Ahmed, G., and Kahney, R., Presentation 252a, AIChE Annual Meeting, Los Angeles, CA  (1997).

“Prediction of Two Phase Flow Distribution in Two Dimensional Trickle bed Reactors” Jiang, Y.,

Khadilkar, M. R., Al-Dahhan, M., Dudukovic, M. P., Poster Presentation 276a, AIChE Annual Meeting, Los Angeles, CA  (1997).

“Simulation of Unsteady State Operation of Trickle Bed Reactors” Khadilkar, M. R., Al-Dahhan, M., and Dudukovic’, M. P., Poster Presentation TRP19, Engineering Foundation Conference, Banff, Canada (1997).

“Effect of Catalyst Wetting on the Performance of Trickle Bed Reactors” Wu. Y., Khadilkar, M. R., Al-Dahhan, M., and Dudukovic, M. P., Second Joint AIChE/CSCE Chemical Engineering Conference, Beijing China (1997).

“Comparison of Trickle-Bed and Upflow Reactor Performance at High Pressure: Model Predictions and Experimental Observations”, Khadilkar, M. R., Wu. Y., Al-Dahhan, M., Dudukovic’, M. H. and Colakyan, M., Presentation at ISCRE-14, Brugge, Belgium  (1996).

“Evaluation of Trickle-bed Reactor Models for a Liquid Limited Reaction” Wu. Y., Khadilkar, M. R., Al-Dahhan, M., and Dudukovic, M. P., Poster Presentation at ISCRE-14, Brugge, Belgium (1996).

“Effect of Fines on the Performance of Downflow (trickle-bed) and Upflow (packed bubble column) Reactors”, Khadilkar, M. R., Wu, Y., Al-Dahhan, M., and Dudukovic’, M. P., Presentation 66e, AIChE Annual Meeting, Miami Beach, Florida (1995).